| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On AMS Scanning Femto ABS |  |
| Date Submitted | 2010-10-24 |  |
| Source(s) | Ying Li, Anshuman Nigam, Jung Je Son, Young Bo Cho, Sudhir Ramakrishna, Kaushik Josiam, Zhouyue Pi Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D9 change requests. |  |
| Abstract | This contribution is to propose changes in support femtocells. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On AMS Scanning Femto ABS

Ying Li, Anshuman Nigam, Jung Je Son, Young Bo Cho

Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi

Samsung Electronics

# Introduction

- Some clarification on AMS scanning femtocells is needed.

# Proposed Text

Please revise D9 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

[on page 896, line 12-26]

After scanning and identifying the existence of any Femto ABSs, the AMS may report FA, A-Preamble index or BS IDs and measurement results according to the Trigger conditions included in the AAI-SCD message by sending AAI-SCN-REP. The AAI-SCN-REP may contain the neighbor request indication and it may additionally contain Neighbor_Request_BS_Type, to which the ABS may unicast an AAI-NBR-ADV that includes a list of femto ABSs which is formed based on the reported FA, A-preamble index or BSIDs, or the reported measurement. When AAI-SCN-REP indicates it is for the neighbor request, it can include AMS desired CSGID(s) of the subscribed CSG Femto BSs, and the ABS can response with a list of ABSs belonging to the requested CSGID(s) in AAI-SCN-REP message.

If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the S-ABS using an AAI-SCN-REP message. For that purpose, AAI-SCN-REP includes the detected 12 bit LSB of the BSID of neighbor Femto ABS(s).

AMS may include the CSGID(s) of the subscribed CSG femto BSs in the AAI-SCN-REP message.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------