| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On Femtocell Interference Mitigation |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Ying Li, Anshuman Nigam, Jung Je Son, Young Bo Cho, Sudhir Ramakrishna, Kaushik Josiam, Zhouyue Pi Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D9 change requests. |  |
| Abstract | This contribution is to propose changes in support femtocells. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On Femtocell Interference Mitigation

Ying Li, Anshuman Nigam, Jung Je Son, Young Bo Cho

Sudhir Ramakrishna, Kaushik Josiam, Zhouyue Pi

Samsung Electronics

# Introduction

- Some editorial clarification on femtocell interference management is needed.

- For example, the ranging purpose indication description for femto interference mitigation needs more clarification.

# Proposed Text

Please revise D9 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Page 87, line 57

Table 678—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Request Femto interference mitigation. Experiencing "femto interference" 0b1101 = NS/EP call setup 0b1110 - 0b1111 = reserved | - |

16.4.11 Interference avoidance and interference mitigation

[on page 898, line 51-60]

If an AMS is placed into outage by an inaccessible Femto ABS (e.g. the CSG-closed Femto ABS of which it is not a member) and only if the AMS has no connection with neighbor ABS, it may indicate this problem to that Femto ABS by sending an AAI-RNG-REQ with the Ranging Purpose Indication code 0b1100, to request interference mitigation.

Upon inaccessible Femto ABS receiving an AAI-RNG-REQ with the Ranging Purpose Indication code 0b1100 from AMS, the Femto ABS may perform interference mitigation based on the measurement and legitimate of the AMS. The Femto ABS may either reduce its transmit power locally or communicate with the network entity to cooperate on the interference mitigation process.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------