| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | R_IdleTime constraint in ARS frame (section 16.6.3) |  |
| Date Submitted | 2010-11-25 |  |
| Source(s) | Youngbin Chang, Jaeweon Cho, Hyunjeong Kang, Rakesh Taori Samsung Electronics | yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | IEEE 802.16m Sponsor Ballot Recirc # 3 |  |
| Abstract | This contribution proposes text on control message on Relay (section 16.6.3) |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

R_IdleTime constraint in ARS frame

Youngbin Chang, Jaeweon Cho, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

## Set Tadv value

In relay frame structure, Tadv was defined in D9 spec. Depending on the value of Tadv, R-TTI and R-TTI in UL can be ‘0’ or ‘1’ OFDM symbol.

Currently, in FDD UL relay frame structure, transition gap was defined in D9 spec as following

Tadv= IdleTime−R_IdleTime

In UL relay frame, R-TTI can be zero if R_IdleTime – RTD/2 is more than ARSTTG, R-RTI can be zero if Tadv +RTD/2 is more than ARSRTG.

R−TTI=0 if R_IdleTime−RTD2≥ARSTTGTs if R_IdleTime−RTD2<ARSTTG

R−RTI=0 if Tadv+RTD2≥ARSTTGTs if Tadv+RTD2<ARSTTG

The one of the concern is the inter-symbol interference issue between MS to BS transmission and MS to RS transmission if R_IdleTime is changed. If Tadv is equal or less than CP value, then it does not much cause inter-symbol interference issue. CP values are derived from table 794.

Table 1. CP values

| CP ratio (Ts – Tb) | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- |
| 1/4 | 22.885us | 32us | 25.6us | 22.885us | 22.885us |
| 1/8 | 11.457us | 16us | 12.8us | 11.457us | 11.457us |
| 1/16 | 5.743us | 8us | 6.4us | 5.743us | 5.743us |

In case of 5/10/20MHz, 1/8 CP case, Table 2 describes maximum ARSTTG and ARSRTG can be calculated using maximum CP values when R-TTI and R-RTI is zero.

Table 2. Calculation of maximum value of ARSTTG and ARSRTG

| Cell coverage | RTD/2 | IdleTime | R_IdleTime | Tadv | Max(ARSTTG) | Max(ARSRTG) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 km | 3.3us | 62.857us | 51.4us | 11.457us | 48.1us | 14.757us |
| 2 km | 6.6us | 62.857 us | 51.4us | 11.457us | 44.8us | 18.057us |
| 5 km | 16us | 62.857 us | 51.4us | 11.457us | 35.4us | 27.457us |

RS ARSTTG and ARSRTG are the value depending on RS implementation. However, these values have meaningful to make a zero R-TTI and R-RTI when ARS is implemented.

In TDD UL relay frame structure, transition gap was defined in D9 spec [1] as following

Tadv=TTG−R_IdleTime

R-RTI = 0 if Tadv−RTD2≥ARSRTGTs if Tadv−RTD2<ARSRTG

In case of 5/10/20MHz, 1/8 CP case, Table 3 describes maximum ARSTTG and ARSRTG can be calculated using maximum CP values when R-RTI is zero as the same way of table 2.

Table 3. Calculation of maximum value of ARSTTG and ARSRTG

| Cell coverage | RTD/2 | TTG | R_IdleTime | Tadv | Max(ARSTTG) |
| --- | --- | --- | --- | --- | --- |
| 1 km | 3.3us | 105.714us | 94.257us | 11.457us | 8.157us |
| 2 km | 6.6us | 105.714 us | 94.257us | 11.457us | 4.857us |
| 5 km | 16us | 105.714 us | 94.257us | 11.457us | X |

. If Tadv is more than OFDM useful time and less OFDM symbol time, then it does not cause inter-symbol interference issue. It is described table 4.

Table 3. Calculation of maximum value of ARSTTG and ARSRTG

| Cell coverage | RTD/2 | TTG | R_IdleTime | Tadv (Ts) | Max(ARSTTG) |
| --- | --- | --- | --- | --- | --- |
| 1 km | 3.3us | 105.714us | 2.857us | 102.857us | 99.557us |
| 2 km | 6.6us | 105.714 us | 2.857us | 102.857us | 96.257us |
| 5 km | 16us | 105.714 us | 2.857us | 102.857us | 86.257us |

Therefore, Tadv can be a following constraint. (Ts: one OFDM symbol time, Tb: useful OFDM symbol time)

Condition 1:  0 ≤ Tadv ≤ Ts-Tb (CP length) or

Condition 2:  Tb ≤ Tadv ≤ Ts

These two conditions are translated using R_IdleTime.

Condition 1: IdleTime (or TTG) – (Ts – Tb) ≤ R_IdleTime ≤ IdleTime (or TTG)

Condition 2: TTG-Ts ≤ R_IdleTime ≤ TTG-Tb

## Signaling to ARS

In D9 spec, there is no way to ABS to signal R_IdleTime to ARS. Reasonable scenario is that ABS can set R_IdleTime after knowing ARSTTG and ARSRTG. Therefore, ABS sends R_IdleTime in the ARS-CONFIG_CMD which contains operation parameters during the ARS network entry.

# Proposed text

[Change: Add following parameters in table 751 on page 275:]

Table 751—AAI-ARS-CONFIG-CMD Message Field Description

| Field | Size (bits) | Value/Descrption | Condition |
| --- | --- | --- | --- |
| R_IdleTime | 11 | Unit is 0.1us. In FDD case, it is inserted between ARS frame and next ARS frame. R_IdleTime shall be set within following range: IdleTime– (Ts – Tb) ≤ R_IdleTime ≤ IdleTime (See table 794) In TDD, it is inserted between ARS downlink subframe and ARS uplink subframe. R_IdleTime shall be set within following range: TTG – (Ts – Tb) ≤ R_IdleTime ≤ TTG or TTG-Ts ≤ R_IdleTime ≤ TTG-Tb (See table 794) |  |

[Modify the following text of line 64 on page 929:]

The duration of the R_IdleTime for the ARS UL frame is signaled by ABS to ARS through AAI-ARS-CONFIG-CMD during ARS network entry. ARS informs the duration of the R_IdleTime by the ARS to its AMSs through the AAI_SCD message during ARS operational mode.

[Modify the following text of line 43 on page 931:]

The duration of the R_IdleTime for the ARS UL subframe is signaled by ABS to ARS through AAI-ARS-CONFIG-CMD during ARS network entry. ARS informs the duration of the R_IdleTime by the ARS to its AMSs through the AAI_SCD message during ARS operational mode.