| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ACK Timer for AAI-MSG-ACK (section 16.11) |  |
| Date Submitted | 2010-11-09 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | IEEE 802.16m Sponsor Ballot Recirc # 3 |  |
| Abstract | This contribution proposes text on ACK timer for AAI-MSG-ACK (section 16.11) |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ACK Timer for AAI-MSG-ACK

Youngbin Chang, Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

ACK timer operation is defined for AAI-MSG-ACK in D9 spec, but detail timer value is not defined in Global parameter section. This contribution defines ACK timer in the Global parameter section.

ACK timer value should be more than 2*(T_ReTx_Interval*DL/UL_N_MAX_ReTx (1*4 = 4 frames, minimum value for T_ReTx_Interval is 1 and DL/UL_N_MAX_ReTx is 4)) which is HARQ process completion time in Tx + HARQ process completion time in Rx. (Maximum round trip of HARQ process). In the implementation, ACK timer can be longer than 2* T_ReTx_Interval*DL/UL_N_MAX_ReTx considering transmitter scheduling time and receiver processing time of control message.

# Proposed text

[Change: Add following parameters in table 977:]

Table 977—Parameters and constants

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS AMS | ACK timer | This timer is for AAI-MSG-ACK. The ACK timer operation is described in 16.2.22. | 8 frames |  |  |