| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to clean up E-MBS parameters in ranging message (16.2.3.) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com , kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to E-MBS parameters in range message |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to clean up E-MBS parameters in ranging message (16.2.3)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak

- LG Electronics

# Introduction

In AAI_RNG-RSP the E-MBS related parameters should be clarified. For example, the E-MBS IDs and FIDs flow mapping does not need to be included in AAI_DSA-RSP message because the AMS can always get the E-MBS IDs and FIDs for neighbor E-MBS Zone through the AAI_E-MBS-CFG message. That is, there is no case that the AMS updates the E-MBS IDs and FIDs mapping information through the AAI_DSA-RSP message. Therefore, update the related parameters in the AAI_RNG-RSP message.

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy: Modify Table 679 as follows, on page 96, line 44 and page 100, line 51:]

Table 679 – AAI_RNG-RSP Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| …… | … | …… | … |
| New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| for(i=1;i<= N-E-MBS-Zone-IDs ;i++) { |  |  |  |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
| If (Service_flow_update_indicator == 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |  |
| For each 1-bit in the Service Flow Update Bitmap{ |  |  |  |
| New_E-MBS ID | 12 | New E-MBS identifier. |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| }else if (Service_flow_update_indicator == 0b1) { |  |  |  |
| for(j=0;j<N_E-MBS_IDs;j++) { |  | N_EMBS_IDs is the number of EMBS IDs listed here Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current E-MBS ID | 12 |  |  |
| Current FID | 4 |  |  |
| New_E-MBS ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| } // end of if |  |  |  |
| }//end of for (N-E-MBS-Zone- IDs) |  |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| … | … | … | … |
| If(it is under network re-entry for HO){ |  |  |  |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| for(i=1;i<= N-E-MBS-Zone-IDs ;i++) { |  |  |  |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update |  |
| Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | … |
| If (Service_flow_update_indicator == 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |  |
| For each 1-bit in the Service Flow Update Bitmap{ |  |  |  |
| New_E-MBS ID | 12 | New E-MBS identifier. |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| }else if (Service_flow_update_indicator == 0b1) { |  |  |  |
| for(j=0;j<N_E-MBS_IDs;j++) { |  | N_EMBS_IDs is the number of EMBS IDs listed here Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current E-MBS ID | 12 |  |  |
| Current FID | 4 |  |  |
| New_E-MBS ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| } // end of if |  |  |  |
| }//end of for (N-E-MBS-Zone- IDs) |  |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| … | … | … | … |

--------------Proposed Texts  ---------------------------------------------------------------------------