| Project | Sponsor ballot comments on p802.16m/D9 (an attachment to Dan Gal’s comment submitted via IEEE 802 MyBallot tool on 10-25-2010) <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested changes to sections 16.2.12.8, 16.2.12.9 and 16.2.12.10 |  |
| Date Submitted | 2010-10-27 |  |
| Source(s) | Dan Gal, Suresh Nair, Peretz Feder et al | ALU |
| Re: | Sponsor ballot, Recirculation 3 of p802.16m (draft D9) |  |
| Abstract | Ballot comments on p802.16m/D9 |  |
| Purpose | Modify draft D9 in accordance with the suggestions of this document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

1.   The following are suggested changes to sections 16.2.12.8 and 16.2.12.9 of draft p802.16m/D9

2.   Please make the following global-change to all DSX-REQ/RSP tables and replace “Emergency Indication Parameter” with “Emergency or NS/EP Indication Parameter”.

16.2.12.8 	Emergency and NS/EP Service Flows

Service flow parameters for Emergency service (e.g., E-911) and National Security/Emergency Preparedness (NS/EP)  services may be pre-defined in the system. When such  emergency and/or NS/EP service flow is predefined, a FID for the Emergency or NS/EP service flow is either predefined or allocated by the ABS.

During network entry, the ABS shall allocate the Emergency  service FID for the emergency service flow through AAI-RNG-RSP upon receiving AAI-RNG-REQ with Ranging Purpose Indication set to code 0b0101. For the NS/EP service flow, a NS/EP FID  shall be assigned through AAI-RNG-RSP upon receiving AAI-RNG-REQ with Ranging Purpose Indication set to code 0b1101. During the Connected State, if a FID for the Emergency or NS/EP service flow is not pre-defined, the ABS shall allocate the FID for the service flow through AAI-DSA-RSP upon receiving the appropriate indication in the AAI-DSA-REQ. The Emergency service FIDs, NS/EP service FIDs, and FIDs for regular (non-emergency and non- NS/EP) service flows shall be distinct from one another.

16.2.12.9 	Emergency and NS/EP Service Notification during initial ranging

The AMS may request Emergency or NS/EP Service flow setup during the initial ranging process by setting the Ranging Purpose Indication to code 0b0101 for Emergency  services and code 0b1101 for NS/EP services in the AAI-RNG-REQ message.

If the service flow parameters are pre-defined, the AMS transmits the Emergency or NS/EP message using the FID for the appropriate Emergency or NS/EP service flow without going through the complete service flow setup through DSA transaction. The ABS grants resources according to the service flow parameters pre-defined for Emergency or NS/EP service. If no service flow parameters are pre-defined for the Emergency or NS/EP service, the AMS and the ABS shall establish the emergency service flow via DSA transaction.

If a service provider wants to support NS/EP services, the ABS uses its own algorithm as defined by its local country regulation body. For example, in the US the algorithm to support NS/EP is defined by the FCC in the so-called Hard Public Use Reservation by Departure Allocation (PURDA).

16.2.12.10 	Emergency and NS/EP Service Notification during Connected state

When an AMS requests Emergency or NS/EP Service in the connected state and service flow parameters are pre-defined for the services, the AMS shall send a bandwidth request using the appropriate FID for the Emergency or NS/EP service flow without going through the complete service flow setup through DSA transaction. When an AMS requests for Emergency or NS/EP Service in the connected state and no service flow parameters are pre-defined for the services, the AMS shall establish the appropriate Emergency or NS/EP service flow using service flow setup procedure through DSA transaction and include the emergency service notification (a 1-bit Emergency or NS/EP Indication parameter in Table 734) in the AAI-DSA-REQ.

For Emergency  service, the value of the Emergency Indication parameter shall be set to 0. For NS/EP service, the value of the Emergency Indication parameter shall be set to 1. Absence of the Emergency Indication parameter in the AAI-DSA-REQ message denotes normal  services.