| Project | Sponsor ballot comments on p802.16m/D9 (an attachment to Dan Gal’s comment submitted via IEEE 802 MyBallot tool on 10-25-2010) <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested changes to sections 16.2.12.8, 16.2.12.9 and 16.2.12.10 and Annex R – ASN.1 Coding |  |
| Date Submitted | 2010-10-27 |  |
| Source(s) | Dan Gal, Suresh Nair, Peretz Feder Ming Lai Jungshin Park | ALU Telcordia Samsung |
| Re: | Sponsor ballot, Recirculation 3 of p802.16m (draft D9) |  |
| Abstract | Ballot comments on p802.16m/D9 |  |
| Purpose | Modify draft D9 in accordance with the suggestions of this document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

1.  Add a new flag named NS/EP Service Indication Parameter.

Insert the flag definition as new table entries in the following three places:

(a) On page 229, in Table 734 - AAI-DSA-REQ Message Field Description, after line 26

(b) On page 246, in Table 737 - AAI-DSC-REQ Message Field Description, after line 7, and

(c) On page 424, in Table 781 - Service flow/convergence sublayer parameters, after line 34

Field name: NS/EP Service Indication Parameter

Size (bits): 1

Value/Description: Indicates the associated flow is used for NS/EP services

Conditions: 1=NS/EP flow, 0=Not an NS/EP flow.

2.  Make the following text changes to sections 16.2.12.8, 16.2.12.9 and 16.2.12.10

16.2.12.8 	Emergency and NS/EP Service Flows

The AAI system shall support two types of emergency services, E-911 type Emergency services for all users and National Security/Emergency Preparedness (NS/EP) services for designated emergency service personnel.

Service flow parameters for Emergency service (e.g., E-911) may be pre-defined in the system. When such emergency service flow is predefined, a FID for the Emergency service flow is either predefined or allocated by the ABS.

During the network entry, if the FID is not predefined, the ABS shall allocate the Emergency service FID for the Emergency service flow through AAI-RNG-RSP upon receiving AAI-RNG-REQ with Ranging Purpose Indication set to code 0b0101. For the NS/EP service flow, a NS/EP FID shall be assigned through AAI-RNG-RSP upon receiving AAI-RNG-REQ with Ranging Purpose Indication set to code 0b1101.

During the Connected State, if the FID for the Emergency  service flow is not pre-defined, the ABS shall allocate the FID for the service flow through AAI-DSA-RSP upon receiving the appropriate indication in the AAI-DSA-REQ. For the NS/EP service flow, the ABS shall allocate the FID through AAI-DSA-RSP upon receiving the NS/EP service notification in the AAI-DSA-REQ.

16.2.12.9 	Emergency and NS/EP Service Notification during initial ranging

The AMS may request Emergency or NS/EP Service flow setup during initial ranging process by setting the Ranging Purpose Indication to code 0b0101 for Emergency type services or code 0b1101 for NS/EP services in the AAI-RNG-REQ message.

Upon receiving the request for Emergency Service from the AMS, the ABS shall allocate a FID for the Emergency service flow through the AAI-RNG-RSP, if the FID is not pre-defined. Upon receiving the request for NS/EP Service from the AMS, the ABS shall allocate a FID for the NS/EP service flow though the AAI-RNG-RSP.

If the service flow parameters are pre-defined, the AMS transmits the Emergency or NS/EP message using the FID for the appropriate Emergency or NS/EP service flow without going through the complete service flow setup through DSA transaction. The ABS grants resources according to the service flow parameters pre-defined for Emergency or NS/EP service. If no service flow parameters are pre-defined for the Emergency or NS/EP service, the AMS and the ABS shall establish the Emergency or NS/EP service flow via DSA transaction.

If a service provider wants to support National Security/Emergency Preparedness (NS/EP) services, the ABS uses its own algorithm as defined by its local country regulation body. For example, in the US the algorithm to support NS/EP is defined by the FCC in the so-called Hard Public Use Reservation by Departure Allocation (PURDA).

16.2.12.10 	Emergency and NS/EP Service Notification during connected state

When an AMS requests Emergency  Service in the connected state and service flow parameters are pre-defined for the emergency services, the AMS shall send a bandwidth request using the appropriate FID for the Emergency service flow without going through the complete service flow setup through DSA transaction. 

When no service flow parameters are pre-defined for either the Emergency or the NS/EP services, the AMS shall establish the appropriate Emergency or NS/EP service flow using the service flow setup procedure through DSA transaction and raise (set to 1) either  the Emergency Indication Parameter  or the NS/EP Indication Parameter (see Table 734), whichever is appropriate, in the AAI-DSA-REQ.

3.  Make the following changes to Annex R - ASN.1 Coding Recommendations

| Change # | Page # | after line # | Change description |
| --- | --- | --- | --- |
| 1 | 1014 | 20 | Insert: NSEPIndication BOOLEAN OPTIONAL |
| 2 | 1017 | 12 | Insert: NSEPIndication BOOLEAN OPTIONAL |