| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Emergency Alert in AAI-TRF_IND-RSP (16.2.3) |  |
| Date Submitted | 2010-11-05 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee Institute for Information Industry (III) | cwchen@iii.org.tw |
| Re: | Sponsor Ballot P802.16m |  |
| Abstract | This contribution proposes text for emergency alert service |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of Emergency Alert in AAI-TRF_IND-RSP (16.2.3)

Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee
Institute for Information Industry (III)

Proposed text for the P80216m/D9:

-----------------------------------------------Start of the Text Proposal --------------------------------------------

[Make the following changes to Section 16.2.3.29 in Page 181 beginning Line 45 in P802.16m/D9.]

#### 16.2.3.29 AAI-TRF_IND-RSP

Table 706—AAI-TRF_IND-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI-TRF_IND-RSP message_format() { | — | — |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information for supporting the emergency alert service 0b0: there is no emergency information 0b1: there is emergency information |  |
| Frame_Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start 0~1023 |  |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the negative traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time. Value: 0~1023 Sleep Cycle Length = Value + 1 |  |
| } | — | — |  |

-----------------------------------------------End of the Text Proposal --------------------------------------------

[1] IEEE P80216m/D9, “P802.16m/D9: Advanced Air Interface”