| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Other RAT Discovery |  |
| Date Submitted | 2010-11-09 |  |
| Source(s) | Jin Lee, kiseon Ryu, Youngsoo Yuk and JinSam Kwak LG Electronics | E-mail: jin1.lee@lge.com |
| Re: |  |  |
| Abstract | This contribution is for consistency between other RAT discovery procedure (16.2.6.5.2.1.2.1) and MAC message part. |  |
| Purpose | Discuss and adopt the proposed texts in D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Other RAT Discovery

Jin Lee, Kiseon Ryu, Youngsoo Yuk and JinSam Kwak

LG Electronics

# Overview

Section 16.2.6.5.2.1.2.1 needs to be consistent with MAC message part

# Proposed changes

[Add texts in Red, Delete texts in Blue]

## [Remedy #1, Modify Table 720(AAI-PKM-REQ) in page 205 as following:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 Reauth-Request; PKM v3 message code =1 - PKMv3 EAP-Transfer; PKM v3 message code =2 -PKMv3 Key_Agreement-MSG#2; PKM v3 message code =4 - PKMv3 TEK-Request; PKM v3 message code =6 - PKMv3 TEK-Invalid; PKM v3 message code =8 - PKMv3 MIH Initial Request ; PKM v3 message code =9 912-16 : reserved |  |

## [Remedy #2, Add rows at the end of Table 720(AAI-PKM-REQ) in page 206 as following:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (PKM v3 message code ==9) { |  |  |  |
| MIHF frame type | 2 | Bit 0: Service Management Bit 1: Event Service Bit 2: Command Service Bit 3: Information Service |  |
| Delivery Method and Status Code | 2 | Bit 0: Unicast Bit 1: Broadcast Bit 2 : Requested Information is not available |  |
| MIHF Frame | Variable | The encapsulated MIHF query request |  |
| } |  |  |  |

## [Remedy #3, Modify Table 721(AAI-PKM-RSP) in page 207 as following:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 -PKMv3 MIH Acknowledge; PKM v3 message code =10 -PKMv3 MIH Comeback Response; PKM v3 message code 11 912-16 : reserved |  |

## [Remedy #4, add rows at the end of table 721(AAI-PKM-RSP) in page 208 as following:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (PKM v3 message type code == 10) { |  |  |  |
| Cycle | 16 | The 8 LSB of the absolute frame number of the first frame where the MIH response is expected to be ready for transmittal to the AMS, and the interval between subsequent frames where the MIH response may be transmitted |  |
| Query ID | 8 | This value uniquely identifies a pending MIH query at the ABS. |  |
| Delivery Method and Status Code | 2 | Bit 0: Unicast Bit 1: Broadcast Bit 2 : Requested Information is not available |  |
| MIHF Frame | Variable | The encapsulated MIHF query response |  |
| } |  |  |  |
| If (PKM v3 message type code == 11) { |  |  |  |
| MIHF frame type | 2 | Bit 0: Service Management Bit 1: Event Service Bit 2: Command Service Bit 3: Information Service |  |
| Query ID | 8 | This value uniquely identifies a pending MIH query at the ABS. |  |
| Delivery Method and Status Code | 2 | Bit 0: Unicast Bit 1: Broadcast Bit 2 : Requested Information is not available |  |
| MIHF Frame | Variable | The encapsulated MIHF query response |  |
| } |  |  |  |

## [Remedy #5, Modify Table 722(PKM v3 message types) in page 207 as following:]

| Code | PKM message type | MAC control message name |
| --- | --- | --- |
| 9 | PKMv3 MIH Initial Request | AAI-PKM-REQ |
| 10 | PKMv3 MIH Acknowledge | AAI-PKM-RSP |
| 11 | PKMv3 MIH Comeback Response | AAI-PKM-RSP |
| 912-16 | reserved |  |

## [Remedy #6, Include texts in page 215 line 40 as following:]

16.2.3.43.9 MIH Initial Request message

The AMS sends this message to the ABS to deliver an MIH query encapsulated in an MIHF frame.

Code: 9

Attributes are shown in table xxx.

Table xxx - MIH Initial Request message attributes

| Attribute | Contents |
| --- | --- |
| MIHF frame type | Indicates the type of MIH frame Bit 0: Service Management Bit 1: Event Service Bit 2: Command Service Bit 3: Information Service |
| Delivery Method and Status Code | Bit 0: Unicast Bit 1: Broadcast Bit 2: Requested information is not available. |
| MIHF frame | The encapsulated MIHF query |

16.2.3.43.10 MIH Acknowledge message

This message is sent by the ABS to the AMS to acknowledge a received MIH query encapsulated in an MIHF frame. The response to the query is sent in a later MIH Comeback Response message, and the AMS uses a Query ID, received in this MIH Acknowledge message and associated with the AMS Initial Request by virtue of the stateful nature of the MIH Acknowledge, to correlate the MIH Initial Request message query with the later response.

Code : 10

Attributes are shown in Table xxx.

Table xxx – MIH Acknowledge message attributes

| Attribute | Contents |
| --- | --- |
| Cycle | Bit 0–7: Specify the MIH Cycle Offset. The MIH Cycle Offset is used to indicate subsequent frames when the ABS may allocate bandwidth for the AMS in the A-MAP or send an AAI-SII-ADV message including the MIH response. The subsequent frames are calculated by adding multiples of the MIH Cycle Offset to the original absolute frame number transmission opportunity. Bit 8–15: Specify the 8 LSB of the absolute frame number when the ABS may indicate that the MIH response is ready to be delivered to the AMS by allocating bandwidth for the MS in the A-MAP, if it is unicasting the MIH response, or when the ABS may send an AAI-SII-ADV message including the MIH response, if the ABS is broadcasting the MIH response. |
| Query ID | This value uniquely identifies a pending MIH query at the ABS. Since MIH responses may be broadcast, the value of Query ID shall be unique per ABS. |
| Delivery Method and Status Code | Bit 0: Unicast Bit 1: Broadcast Bit 2 : Requested Information is not available |

16.2.3.43.11 MIH Comeback Response

The ABS sends this message to the AMS to deliver a query response encapsulated in an MIHF frame.

Code: 11

Attributes are shown in Table xxx.

Table xxx – MIH Comeback Response message attributes

| Attribute | Contents |
| --- | --- |
| MIHF frame type | Indicates the type of the included MIHF frame. Only included when an MIHF frame is present. Bit 0: System Management Bit 1: Event Service Bit 2: Command Service Bit 3: Information Service |
| Query ID | This value uniquely identifies a pending MIH query at the ABS. Since MIH responses may be broadcast, the value of Query ID shall be unique per ABS. |
| Delivery Method and Status Code | Bit 0: Unicast Bit 1: Broadcast Bit 2 : Requested Information is not available |
| MIHF frame | The encapsulated MIH response |

## [Remedy #7, Include texts in page 963 line 62 (at the end of table 977) as following:]

| System | Name | Time Reference | Minimum value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS | MIH max cycles | The maximum number of cycles that an AMS waits for an MIH response during initial entry. | 3 | 3 | - |

## [Remedy #8, Include texts in page 264 line 43 (at the end of table 744) as following:]

Table 744 - AAI-SII-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MIHF Frame | Variable | MIHF frame |  |

## [Remedy #9, Modify texts in page 363 line 26 as following:]

16.2.6.5.2.1 Network topology acquisition

WirelessMAN-OFDMA Advanced systems advertise information about other RATs (such as RAT Type,pre-registration supported, RAN information etc.) to assist the AMS with network discovery and selection. WirelessMAN-OFDMA Advanced systems provide a mechanism for AMS to obtain information about other access networks in the vicinity of the AMS from an ABS either by making a query or listening to a system information broadcast.