| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Idle Mode operation in IEEE 802.16m (P802.16m/D9-16.2.18) |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Lei Zhang, Yang Liu, Yuqin Chen ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: Sponsor Ballot / Area: Chapter 16.2.18 (Idle Mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.18 of IEEE P802.16m/D9 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Idle Mode operation in IEEE 802.16m (P802.16m/D9-16.2.18)

Lei Zhang, Yang Liu, Yuqin Chen

ZTE Corporation

- Introduction

In the current P802.16m/D9, some descriptions on Idle Mode operation are still very unclear. This proposal is used to clarify these operation descriptions.

- Proposed text

======================== Start of Proposed Text ========================

[Modify section 16.2.18 (from line 53 on page 475) of P802.16m/D9 as follows]

When an AMS operating in legacy Idle Mode operation capable of WirelessMAN-OFDMA Advanced co-existing System operations selects a Mixed Mode ABS as a preferred ABS, the AMS may stay in the Lzone of the Mixed Mode ABS and perform the legacy Idle Mode operation as specified in 6.3.236.2.23.

The AMS staying in the Lzone of the Mixed Mode ABS may find and move to a neighbor ABS or Mzone of Mixed Mode ABS. If an AMS in Idle Mode decides to change its Idle Mode operation mode between legacy Idle Mode operation and advanced Idle Mode operation, the AMS shall send an AAI-RNG-REQ message with Ranging Purpose Indication set to 0b0010 to initiate perform full network reentry in the new Idle Mode operation mode.

The Idle Mode operation mode change is caused also by zone switching between LZone and MZone of a mixed mode ABS and handover between an ABS and a WirelessMAN-OFDMA Advanced co-existing System R1 Reference BS.

- References

[1] IEEE P802.16m/D9. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Oct 2010.