| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Updates to Annexes T and U, IEEE P802.16m/D9 |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Roshni Srinivasan, Reza Arefi Hassan Yaghoobi Intel Corporation Roger Marks WiMAX Forum | roshni.m.srinivasan@intel.com r.b.marks@ieee.org *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot of P802.16m/D9 |  |
| Abstract | This contribution proposes updates to with Annex U containing radio specifications. |  |
| Purpose | This is proposed as the basis of a 802.16m Sponsor Ballot Recirc comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Updates to Annexes T and U, IEEE P802.16m/D9

Roshni Srinivasan, Reza Arefi, Hassan Yaghoobi

Intel Corporation

Roger Marks

WiMAX Forum

Note to Editor:

This contribution proposes changes to the Normative References and to Annexes T and U. New text is shown in blue, text to be deleted is struck through in red, existing text is shown in black.

___________________________Begin Proposed Text Change #1______________________

2. Normative references

[Insert the following text:]

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

IETF RFC 2132, “DHCP Options and BOOTP Vendor Extensions”, S. Alexander, R. Droms, March 1997.

IETF RFC 3315, “Dynamic Host Configuration Protocol for IPv6 (DHCPv6)”, R. Droms, et al., July 2003.

Internet Assigned Numbers Authority (IANA) , “Dynamic Host Configuration Protocol (DHCP) and Bootstrap Protocol (BOOTP) Parameters”

Internet Assigned Numbers Authority (IANA), “Dynamic Host Configuration Protocol for IPv6 (DHCPv6)”

NIST Special Publication 800-38C-Recommendation for Block Cipher Modes of Operation: The CCM Mode for Authentication and Confidentiality, May 2004.

WiMAX Forum WMF-T23-005-R015v04, “WiMAX Forum® Mobile Radio Specification,” September 2010.

___________________________Begin Proposed Text Change #1______________________

___________________________Begin Proposed Text Change #2______________________

Annex T

(Normative)

T.1 Supported Frequency Bands

The standard supports deployment in all bands identified for IMT in ITU-R Radio Regulations. In addition, the standard supports non-IMT bands below 6 GHz allocated to the Fixed Service and/or Mobile Service.

See below for Subclause 1.1 of WiMAX Forum WMF-T23-005-R015v04 identifies more information on some of the bands in which systems based on the standard can be deployed, along with the channel bandwidth and duplex mode.

___________________________End Proposed Text Change #2______________________

___________________________Begin Proposed Text Change #3______________________

Delete all text starting from line 23, page 1087 until the end of Annex T.

___________________________End Proposed Text Change #3______________________

___________________________Begin Proposed Text Change #4______________________

Apply the following changes to Annex U starting on page 1089:

Annex U

(Normative)

U.1 Radio specifications for WirelessMAN-Advanced Air Interface

“Subclause 2.1.1 of WiMAX Forum WMF-T23-005-R015v04 specifies the center frequency step size for the WirelessMAN-Advanced Air Interface.”

“Subclause 2.1.2 of WiMAX Forum WMF-T23-005-R015v04 specifies the RF profile for the WirelessMAN-Advanced Air Interface.”

“Subclause 2.1.3 of WiMAX Forum WMF-T23-005-R015v04 specifies the power class profile for the WirelessMAN-Advanced Air Interface.”

“Subclauses 2.1.4-2.2.6 of WiMAX Forum WMF-T23-005-R015v04 specify the unwanted emissions requirements for the WirelessMAN-Advanced Air Interface.”

___________________________End Proposed Text Change #4______________________

___________________________Begin Proposed Text Change #5______________________

Delete all text starting from line 14, page 1088 until the end of Annex U.

___________________________End Proposed Text Change #5______________________