| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Removal of LDM parameters from AAI-NBR-ADV |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Kiseon Ryu LG Electronics | E-mail: kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to AAI-NBR-ADV message clean-up |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Removal of LDM parameters from AAI-NBR-ADV

Kiseon Ryu

- LG Electronics

# Introduction

Text clarification of AAI-NBR-ADV message is needed. This contribution is to resolve the comment #10041.

# Text proposal for the IEEE 802.16m amendment

--------------Start of Proposed Texts -------------------------------------------------------------------

[Remedy 1: Modify the text as follows, on page 137, line 26:]

16.2.3.13 AAI-NBR-ADV

Table 690—AAI-NBR-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| LDM Parameter - AI | 4 | The new AI for LDM | Shall be included when the LDM parameters are changed. These parameters will be broadcasted for a certain duration of time as decided by the network |
| LDM Parameter - UAI | 8 | The new UAI for LDM | Shall be included when the LDM parameters are changed. These parameters will be broadcasted for a certain duration of time as decided by the network |
| LDM Parameter - SFO | 9 | The new SFO for LDM | Shall be included when the LDM parameters are changed. These parameters will be broadcasted for a certain duration of time as decided by the network |

[Remedy 2: Modify the text as follows, on page 1035, line 65 and on page 1037, line 18:]

AAI-NBR-ADV ::= SEQUENCE {

changeCountINTEGER (0..7),

…

ldmParametersLDMParametersOPTIONAL

…

LDMParameters ::= SEQUENCE {

newAIINTEGER(0..15),

newUAIINTEGER(0..255),

newSFOINTEGER(0..511)

}

…

--------------End of Proposed Text----------------------------------------------------------------------