| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Paging Cycle Inclusion in AAI-PAG-ADV |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Kiseon Ryu LG Electronics | E-mail: kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to AAI-PAG-ADV message clean-up |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Paging Cycle Inclusion in AAI-PAG-ADV

Kiseon Ryu

- LG Electronics

# Introduction

Text clarification of AAI-PAG-ADV message is needed. In legacy network mode, MAC Address Hash is used to identify the paged MS. There is no need to include Paging Cycle as well as DID in the AAI-PAG-ADV message in legacy network mode. This contribution is to resolve the comment #10048.

# Text proposal for the IEEE 802.16m amendment

--------------Start of Proposed Texts -------------------------------------------------------------------

[Remedy 1: Modify the text as follows, on page 168, line 44:]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Table 700—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved | Present if the Network Configuration indicates ABS is attached to the advanced network |

[Remedy 2: Modify the text as follows, on page 1048, line 65:]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Idle Mode Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

MACAddressHash ::= BIT STRING (SIZE(24))

PagedMSInfo ::= SEQUENCE {

-- if Network Configuration indicates ABS is attached to the advanced network

deRegId DREGID OPTIONAL,

-- if Network Configuration indicates ABS is attached to the legacy network

macAddressHash MACAddressHash OPTIONAL,

-- if Network Configuration indicates ABS is attached to the legacy network

pagingCycle PgCycle OPTIONAL,

--------------End of Proposed Text----------------------------------------------------------------------