| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of paging parameter update |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Giwon Park, Kiseon Ryu LG Electronics | E-mail: giwon.park@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to paging parameters update |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of paging parameters update

Giwon Park, Kiseon Ryu

- LG Electronics

# Introduction

Text clarification of paging parameters update is needed. This contribution is to resolve the comment #036.

# Text proposal for the IEEE 802.16m amendment

--------------Start of Proposed Texts -------------------------------------------------------------------

[Modify the text as follows, on page 96, line 21:]

16.2.3.2 AAI-RNG-RSP

Table 679—AAI-RNG-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(Location Update Response == 0x0) { | - |  |  |
| Paging Group ID update | 32 | Old PG ID, New PG ID The Paging Group ID update specifies a new PGID that replaces an old PGID. | Shall be included when the Paging parameters are changed. |
| Paging Offset update | 24 | Old Paging Offset, New Paging Offset The Paging Offset update specifies a new Paging Offset that replaces an old Paging Offset. | Shall be included when the Paging parameters are changed. |
| New Paging Cycle | 4 | New Paging Cycle to be assigned to the AMS 0b0000-0b1111 | Shall be included when the new Paging parameters are changed assigned. |
| New Paging Group ID | 16 | New PGID of the AMS | Shall be included when the new Paging parameters are changed assigned. |
| New Paging Offset | 12 | New paging offset of AMS which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. | Shall be included when the new Paging parameters are changed assigned. |
| … |  |  |  |

[Editor’s Note: Modify the text as follow, on page 984, line 23]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Network entry / re-entry messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

SuccessOfLocationUpdate ::= SEQUENCE {

paginggroupidupdate 			BIT STRING(SIZE(32)) OPTIONAL,

pagingoffsetupdate			BIT STRING(SIZE(24)) OPTIONAL,

newPagingCycle			PgCycle OPTIONAL,

newPagingGroupID 			PGID OPTIONAL,

newPagingOffset 			PgOffset OPTIONAL,

…