| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on padding in Coverage Loss detection procedure (Section 16.2.2.1) |  |
| Date Submitted | 2010-11-11 |  |
| Source(s) | Yeongmoon Son, Hyunjeong Kang Samsung Electronics Co., Ltd. | ym1004.son@samsung.com |
| Re: | Harmonized text for the comment #C10122 in the commentary DB 80216-10/0052r1. |  |
| Abstract | The contribution clarifies the text on BR header transmission. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on padding in Coverage Loss detection procedure (Section 16.2.26.1)

Yeongmoon Son, Hyunjeong Kang

Samsung Electronics

- Introduction

This contribution includes the harmonized text for padding in Coverage Loss detection procedure.

- Proposed Text Changes

[Modify text on page 509, line 54 in IEEE802.16m/D9 as follows:]

------------------------------------------------Start proposed text----------------------------------------------------------------

Upon each expiration of the active_ABS_timer, to check whether an AMS is still alive in active mode, the ABS shall grant UL burst to the AMS and the AMS shall transmit a UL MAC PDU with data or if the AMS has no pending UL MAC PDU to transmit no data pending to be transmitted, then just with a BR header with BR size set to 0 padding bytes (see 16.2.4.7) on the UL grant.

-------------------------------------------------End proposed text----------------------------------------------------------------