| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Clean-up for UL Multi-BS MIMO with Single BS Precoding (16.5.2) |  |
| Date Submitted | 2010-12-16 |  |
| Source(s) | Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI Tsung-Yu Tsai, Chun-Che Chien, Chun-Yen Hsu, Youn-Tai Lee III | E-mail: pinghengkuo@itri.org.tw |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D10) |  |
| Abstract | This contribution aims to resolve some font inconsistency issues in Section 16.5.2.1.2 and tidy up Equation (334). |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Clean-Up for UL Multi-BS MIMO

with Single BS Precoding (16.5.2)

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

Tsung-Yu Tsai, Chun-Che Chien, Chun-Yen Hsu, Youn-Tai Lee

III

# Introduction

- In the current draft [1], there are some font inconsistencies in Section 16.5.2. In particular, the italic fonts for abbreviations “PMI” should be removed for document consistency, and the notation for precoding matrices W should be bold instead of italic. Also, equation (334) looks messy and should be re-arranged. In order to improve readability, this contribution proposes text clean-up to address these issues. Moreover, we suggest adding a few sentences in operational procedures to clarify the determinations of interference sensitivity level (ISL).

- Text proposal for inclusion in the 802.16m Draft 10

[Modify the following text in Section 16.5.2.1.2]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining multiple PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; the others respectively minimize interference generated to each of the Nnbr neighboring cells.

The operation procedure of PMI combination follows the steps below.

- PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the S-ABS and the jth (j=1… Nnbr) neighboring ABS may determine PMI maximizing transmission power and PMImin,j PMImin,j (j=1..Nnbr) minimizing interference, respectively. Each of the Nnbr neighboring ABSs should also determine its own interference sensitivity level (ISL), λ. The jth neighboring ABS may inform the S-ABS of PMImin,j provide the information of its PMImin,j and ISL (λj) to the S-ABS through the backhaul network.

Those PMIs The correspondent precoders of those PMIs ,  and   (j=1..Nnbr), should be selected based on the following procedure:

(333)

where Hs and HI,j denote information of channels from the AMS to its serving and the jth neighboring ABS, respectively, and vi is the ith codeword of the base codebook.

2) The S-ABS shall inform the PMI in UL Basic Assignment A-MAP IE or UL Subband Assignement A-MAP-IE and PMImin,j PMImin,j, λj (interference sensitivity level of the jth neighboring ABS) and PCR in the AAI-MBS-MIMO-SBP message to the AMS.

The transmitted precoder W combining multiple precoders should be generated as follows:

(334)

where  and .

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

References

[1] IEEE P802.16m/D10, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, Nov 2010.