| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-PGID-INFO message table and ASN.1 code Clarification |  |
| Date Submitted | 2011-01-12 |  |
| Source(s) | Joey Chou Shantidev Mohant Intel Hyunjeong Kang Samsung | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D10) |  |
| Abstract | AAI-PGID-INFO message table and ASN.1 code Clarification |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI-PGID-INFO message table and ASN.1 code Clarification

Joeyn Chou,

Shantidev Mohant

Intel

Hyunjeong Kang

Samsung

Introduction

This contribution proposes changes to message table and ASN.1 code to clarify AAI-PGID-INFO conditions.

Proposed Text

Remedy 1:

In the P802.16m/D10, page 170, line372, modify Table 701 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 701—PGID-Info Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| Num_PGIDs | 2 | Indicate the number of PGID included in PGID-Info message 1..4 |  |
| PG_uniformity flag | 1 | 0: Otherwise 1: All PGs have same paging indication bitmap | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups |
| For(i=0; i<Num_PGIDs; i++) { |  | Indicate the number of PGID included in PGID-Info message 1..4 |  |
| PGID | 16 | Indicate Paging group identifier where the ABS belongs 0..216 -1 | Shall be present |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation Npaging frame = AMS's deregistration identifier mod m to determine a predefined paging frame for the AMS 1..4 | Shall be present |
| Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap (L) equals to the number of carriers in the AAI-Global-CFG message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI-Global-CFG message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. The paging carriers indicated in the PGID-Info are chosen among the carriers on the same frequency band where the PGID-Info message is received. | Present only for when an ABS supports multiple carrier operation and when PG_uniformity flag=0 |
| } |  |  |  |
| If (an ABS supports multiple carrier operation) { |  |  |  |
| If (All PGs have same paging indication Bitmap) { |  |  |  |
| Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap( L) equals to the number of carriers in the AAI-Global-CFG message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI-Global-CFG message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. The paging carriers indicated in this PGID-Info are chosen among the carriers on the same frequency band where the PGID-Info message is received. | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups with PG_uniformity flag=1 |
| } |  |  |  |
| Else { |  |  |  |
| For(i=0; i<Num_PGIDs; i++) { |  | Indicate the number of PGID included in PGID-Info message 1..4 |  |
| Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap( L) equals to the number of carriers in the AAI-Global-CFG message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI-Global-CFG message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. The paging carriers indicated in this PGID-Info are chosen among the carriers on the same frequency band where the PGID-Info message is received. |  |
| } |  |  |  |
| } // Else |  |  |  |
| } // End If (an ABS supports multiple carrier operation) |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D10, page 1062, line 20, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Paging Group Information

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-PGID-INFO ::= SEQUENCE {

--Present only when an ABS supports multiple carrier operation

-- and belongs to multiplepaging groups

pgUniformityFlag               ENUMERATED {

multipleIndicationMap,

uniformIndiationMap

} OPTIONAL,

pagingGroupInfoArray           SEQUENCE (SIZE (1..4)) OF SEQUENCE {

pagingGroupID                  PGID,

m                              INTEGER (1..4),

-- Present only when pgUniformityFlag indicates multipleIndicationMap

-- The size of paging carrier indication bitmap equals to the number

-- of carriers in the AAI-Global_Config message

pagingCarrierBitmap            BIT STRING (SIZE (1..16)) OPTIONAL

},

<del>    -- Present only when pgUniformityFlag indicates uniformIndicationMap

-- The size of paging carrier indication bitmap equals to the number of carriers

-- in the AAI-Global_Config message</del>

<ins>    multiCarrierPgidInfo           CHOICE {

samePagingCarrierIndicationBitmap       BIT STRING (SIZE (1.. <del>16</del><ins>64</ins>)),

pagingCarrierIndicationBitmapArray     SEQUENCE (SIZE (1..4)) OF SEQUENCE {</ins>

pagingCarrier<ins>Indication</ins>Bitmap        BIT STRING (SIZE (1..<del>16</del><ins>64</ins>))} OPTIONAL,

} OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------