| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Corrections to the Subsection about MAC Control Messages with CMAC Protection (16.2.4.8) |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Anil Agiwal, Youngkyo Baek, Hyunjeong Kang, Youngbin Chang, Rakesh Taori Samsung Electronics | anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | IEEE 802.16m Sponsor Ballot Recirc # 4 |  |
| Abstract | This contribution proposes corrections to the text on MAC Control Messages transmission with CMAC Protection |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D11 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Corrections to the Subsection about MAC Control Messages with CMAC Protection (16.2.4.8)

Anil Agiwal, Youngkyo Baek, Hyunjeong Kang, Youngbin Chang, Rakesh Taori

Samsung Electronics

- Introduction

The current text in P802.16m D10 draft [1] suggests that only MAC control message content is fragmentable. This is not correct as MAC control message together with CMAC tuple is a single unit for the fragmentation function. The CMAC protected MAC control message including the CMAC tuple can be fragmented at any byte boundary. CMAC tuple itself can be transmitted in one (as shown in figure 1) or multiple MAC PDUs (as shown in figure 2).

Figure 1

Figure 2

- Suggested changes in the 802.16m/D10

Change 1: Modify lines 44 to 47 on page 307 as follows

CMAC protected MAC control message together with CMAC tuple can be fragmented at any byte boundary. Figure 398 illustrates the construction of a MAC PDU with fragmented MAC control message that includes CMAC tuple. It shows that CMAC tuple is shall be sent in the MAC PDU last MAC control message fragment.

- References

[1] IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”