| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Capacity Negotiation messages AAI-SBC-REQ/RSP |  |
| Date Submitted | 2011-1-3 |  |
| Source(s) | Jeongho Park, Ji-Yun Seol, Youngkyo Baek Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D10 |  |
| Abstract | This contribution proposes text for Clarification on Capacity Negotiation messages AAI-SBC-REQ/RSP. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D11 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Capacity Negotiation messages AAI-SBC-REQ/RSP (16.2.3.5)

Jeongho Park, Ji-Yun Seol, Youngkyo Baek

Samsung

- References

[1] IEEE 802.16m/D10

- Backgrounds and Scopes

This contribution clarifies the capability negotiation fields in AAI-SBC-REQ/RSP.

Capability negotiation is very necessary because inter-operability could fail without knowledge of capability between ABS and AMS. In this sense, every optional feature in SPEC should be included in negotiation procedure so that ABS can make AMS operate well and prevent situations of AMS’s mal-function.

For an example of insufficient description, current D10 has ‘UL sounding’ field for negotiation but does not differentiate the methods for multiplexing (there are two kinds of methods – CDM based and FDM based). If ABS commands to report CDM based sounding to AMS who is not capable of sending CDM sounding sequence, the operation regarding sounding would be corrupted.

And for another example, there is not any negotiation field about open loop region even though it is not ‘mandatory’ feature. There is no way for ABS to figure out the ability of AMS to enjoy this feature. Therefore, ABS might not use this feature at all. We need to newly add features which are absent in current negotiation procedure.

For some optional features, however, ABS can intrinsically recognize the ability of AMS, for example during initial network entry, or other procedure defined like AAI-MC-REQ/RSP.

During initial network netry, ABS can figure out whether or not AMS can support the following features : Tone dropping, legacy support on UL TDM/FDM, Long TTG frame configuration, Async. Ranging format. Also, ABS can figure out the following features during AAI-MC-REQ/RSP procedure: Use of guard subcarrier.

Consequently, in this contribution, we are trying to clarify the following features

- Sounding sequence multiplexing : CDM / FDM

- OL Region type 0, 1, 2

- Long-term/Short-term reporting in MFM

- DL/UL pilot pattern for MU MIMO operation

As for the last item of ‘DL/UL pilot pattern for MU MIMO operation’, it is necessary to put this because ABS can assign 3rd or/and 4th stream index (SI) to AMS who has only 2Tx antennas. That is to say, the maximum stream number of UL SU-MIMO cannot cover the pilot pattern and this item needs to be included in capacity negotiation procedure.

- Proposed Text

Remedy #1: Modify line 15~63 of page 109 (143) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 682——AAI-SBC-REQ message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| UL sounding | 1 2 | If Bit#0 =1,it supports If Bit#0=1, decimation separation based sounding (FDM) supports If Bit#1=1, cyclic shift separation based sounding (CDM) supports | Present as needed |
| OL Region | 3 | If Bit#0=1, OL Region type 0 supports If Bit#1=1, OL Region type 1, CDR and CoFIP supports If Bit#2=1, OL Region type 2 supports | Present as needed |
| … | … | … | … |
| Max. Number of streams for CL MU-MIMO(MIMO mode 4) in AMS point of view in DL MIMO | 1 | The number in the range 1 through 2 that is higher by 1 than this field | Present as needed |
| DL MIMO mode | 6 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports | Present as needed |
| feedback support for DL | 9 11 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports If Bit#9 =1, Long-term reporting disabling support for MFM 0,4,7 If Bit#10 =1, Short-term reporting disabling support for MFM 2,3,5,6 | Present as needed |
| Subband assignment A-MAP IE support | 1 | If Bit#0=1, DL/UL Subband assignment A-MAP IE supports | Present as needed |
| DL pilot pattern for MU MIMO | 2 | If Bit#0 =1, DL 4 stream pilot pattern for DL MU MIMO support If Bit#1 =1, DL 8 stream pilot pattern for DL MU MIMO support | Present as needed |
| Number of Tx antenna of AMS | 2 | The number in the range {1, 2, 4} that is higher by 1 than this field | Present as needed |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 that is higher by 1 than this field | Present as needed |
| Max. Number of streams for MU-MIMO in AMS point of view in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 34 that is higher by 1 than this field | Present as needed |
| UL pilot pattern for MU MIMO | 3 | If Bit#0 =1, UL 2 stream pilot pattern support If Bit#1 =1, UL 4 stream pilot pattern support If Bit#2 =1, UL 8 stream pilot pattern support | Present as needed |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #2: Modify line 57~59 of page 112 (146) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 683——AAI-SBC-RSP message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| UL sounding | 1 2 | If Bit#0 =1,it supports If Bit#0=1, decimation separation based sounding (FDM) supports If Bit#1=1, cyclic shift separation based sounding (CDM) supports | Present as needed |
| OL Region | 3 | If Bit#0=1, OL Region type 0 supports If Bit#1=1, OL Region type 1, CDR and CoFIP supports If Bit#2=1, OL Region type 2 supports | Present as needed |
| … | … | … | … |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #3: Modify line 16~34 of page 113 (147) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 683——AAI-SBC-RSP message Field Description

| … | … | … | … |
| --- | --- | --- | --- |
| Max. Number of streams for CL MU-MIMO(MIMO mode 4) in AMS point of view in DL MIMO | 1 | The number in the range 1 through 2 that is higher by 1 than this field | Present as needed |
| DL MIMO mode | 6 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports | Present as needed |
| feedback support for DL | 9 11 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports If Bit#9 =1, Long-term reporting disabling support for MFM 0,4,7 If Bit#10 =1, Short-term reporting disabling support for MFM 2,3,5,6 | Present as needed |
| Subband assignment A-MAP IE support | 1 | If Bit#0=1, DL/UL Subband assignment A-MAP IE supports | Present as needed |
| DL pilot pattern for MU MIMO | 2 | If Bit#0 =1, DL 4 stream pilot pattern for DL MU MIMO support If Bit#1 =1, DL 8 stream pilot pattern for DL MU MIMO support | Present as needed |
| Number of Tx antenna of AMS | 2 | The number in the range {1, 2, 4} that is higher by 1 than this field | Present as needed |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 that is higher by 1 than this field | Present as needed |
| Max. Number of streams for MU-MIMO in AMS point of view in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 34 that is higher by 1 than this field | Present as needed |
| UL pilot pattern for MU MIMO | 3 | If Bit#0 =1, UL 2 stream pilot pattern support If Bit#1 =1, UL 4 stream pilot pattern support If Bit#2 =1, UL 8 stream pilot pattern support | Present as needed |
| … | … | … | … |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #4: Modify ASN.1 code in line 62 of page 1009 (1043) to line 65 of page 1012(1046) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Basic Capability Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SBC-REQ ::= SEQUENCE {

amsNspRequest CHOICE {

serviceInfoQuery BIT STRING {

reqNspIdList (0),

reqVerboseNameList (1)

} (SIZE (2)),

sbcRequest SEQUENCE {

capabilityIndex CapabilityIndex,

deviceClass DeviceClass,

clcRequest CLCRequest OPTIONAL,

dlLongTTI FeatureSupport OPTIONAL,

ulSounding BIT STRING { FeatureSupport OPTIONAL,

fDM (0),

cDM (1)

} (SIZE (2)) OPTIONAL,

oLRegion BIT STRING {

type0 (0),

type1 (1),

type2 (2)

} (SIZE (3)) OPTIONAL,

dlFfrResourceMetric FeatureSupport OPTIONAL,

dlMaxNumOfSuMimoStreams INTEGER (1..8) OPTIONAL,

dlMaxNumOfMuMimoStreams INTEGER (1..2) OPTIONAL,

ulMaxNumOfSuMimoStreams INTEGER (1..4) OPTIONAL,

ulMaxNumOfMuMimoStreams INTEGER (1..34) OPTIONAL,

dlMIMOModes BIT STRING {

-- 1: supported, 0: not supported

mode0 (0),

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4),

mode5 (5)

} (SIZE (6)) OPTIONAL,

ulMIMOModes BIT STRING {

-- 1: supported, 0: not supported

mode0 (0),

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4)

} (SIZE (5)) OPTIONAL,

dlFeedback BIT STRING { -- 1: supported

differentialMode (0),

mimoFeedbackMode0 (1),

mimoFeedbackMode1 (2),

mimoFeedbackMode2 (3),

mimoFeedbackMode3 (4),

mimoFeedbackMode4 (5),

mimoFeedbackMode5 (6),

mimoFeedbackMode6 (7),

mimoFeedbackMode7 (8),

longTermReporting (9),

shortTermReporting (10)

} (SIZE (911)) OPTIONAL,

subBandAAMAP FeatureSupport OPTIONAL,

dlPilotPatternMUmimo BIT STRING {

dl4Stream (0),

dl8Stream (1)

} (SIZE (2)) OPTIONAL,

ulPilotPatternMUmimo BIT STRING {

ul2Stream (0),

ul4Stream (1),

ul8Stream (2)

} (SIZE (3)) OPTIONAL,

…………………………………………………..

…………………………………………

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Basic Capability Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SBC-RSP ::= SEQUENCE {

sbcRspInfo CHOICE {

nspInformation NspInformation,

noSiq SEQUENCE {

capabilityIndex CapabilityIndex,

deviceClass DeviceClass,

clcResponse CLCResponse OPTIONAL,

dlLongTTI FeatureSupport OPTIONAL,

ulSounding BIT STRING { FeatureSupport OPTIONAL,

fDM (0),

cDM (1)

} (SIZE (2)) OPTIONAL,

oLRegion BIT STRING {

type0 (0),

type1 (1),

type2 (2)

} (SIZE (3)) OPTIONAL,

dlFfrResourceMetric FeatureSupport OPTIONAL,

dlMaxNumOfSuMimoStreams INTEGER (1..8) OPTIONAL,

dlMaxNumOfMuMimoStreams INTEGER (1..2) OPTIONAL,

ulMaxNumOfSuMimoStreams INTEGER (1..4) OPTIONAL,

ulMaxNumOfMuMimoStreams INTEGER (1.. 34) OPTIONAL,

dlMIMOModes BIT STRING {

-- 1: supported, 0: not supported

mode0 (0),

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4),

mode5 (5)

} (SIZE (6)) OPTIONAL,

ulMIMOModes BIT STRING {

-- 1: supported, 0: not supported

mode0 (0),

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4)

} (SIZE (5)) OPTIONAL,

dlFeedback BIT STRING { -- 1: supported

differentialMode (0),

mimoFeedbackMode0 (1),

mimoFeedbackMode1 (2),

mimoFeedbackMode2 (3),

mimoFeedbackMode3 (4),

mimoFeedbackMode4 (5),

mimoFeedbackMode5 (6),

mimoFeedbackMode6 (7),

mimoFeedbackMode7 (8)

longTermReporting (9),

shortTermReporting (10)

} (SIZE (911)) OPTIONAL,

subBandAAMAP FeatureSupport OPTIONAL,

dlPilotPatternMUmimo BIT STRING {

dl4Stream (0),

dl8Stream (1)

} (SIZE (2)) OPTIONAL,

ulPilotPatternMUmimo BIT STRING {

ul2Stream (0),

ul4Stream (1),

ul8Stream (2)

} (SIZE (3)) OPTIONAL,

numberOfTxAntennas ENUMERATED {

oneAntenna,

twoAntenna,

fourAntenna

} OPTIONAL,

-------------------------------------------------Text Change END ---------------------------------------------------------------