| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for Global Carrier Configuration Update (16.2.15.6) |  |
| Date Submitted | 2010-01-06 |  |
| Source(s) | Kiseon Ryu LG Electronics | kiseon.ryu@lge.com +82-31-450-2067 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D10 |  |
| Abstract | The contribution proposes to clarify the update procedure for global carrier configuration information. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for Global Carrier Configuration Update

Kiseon Ryu

LG Electronics

- Introduction

When the information of Global carrier configuration needs to be updated, a unicast AAI-Global-CFG message may be transmitted to the AMS during network (re-)entry. It’s not clearly specified regarding when the unicast AAI-Global-CFG message is sent to the AMS. Moreover, there are redundant texts regarding this, which causes confusion.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Change the texts in line 61 of page 479 to line 9 of page 466 as follows:]

Within the AAI-REG-REQ message, the AMS shall indicate its latest received Global carrier configuration change count. Upon receiving this message, the ABS shall check if AMS's Global carrier configuration information is valid. As a response, the ABS shall include the current Global carrier configuration change count in the AAI-REG-RSP message. If AMS's Global carrier configuration change count is out of date, the ABS shall unicast the AAI-Global-CFG message to the AMS after AAI-REG-REQ/RSP and AAI-MSG-ACK transactions are completed. The ABS shall set Polling bit to 1 in MCEH of the MAC PDU containing the AAI-Global-CFG message to receive an acknowledgement for the AAI-Global-CFG message. If the ABS does not receive the AAI-MSG-ACK message until ACK timer expires, the ABS shall retransmit the AAI-Global-CFG message.

Also, if AMS's Global carrier configuration information is expired out of date, the AMS shall not perform any procedure that refers to the AAI-Global-CFG message until the valid AAI-Global-CFG message is sent by received from the ABS.

If the AAI-Global-CFG message parameters are to be updated, the ABS shall unicast the updated AAI-Global-CFG message to each AMS's served by the ABS after the AMS and ABS complete switching between TSTID and STID. The ABS shall set Polling bit to 1 in MCEH of the AAI-Global-CFG message.

-------------------------------  Text End  ---------------------------------------------------