| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DCD/UCD information in the AAI-HO-CMD message (Section 16.2.3.12) |  |
| Date Submitted | 2011-01-06 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang, Yeongmoon Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE P802.16m Sponsor Ballot Recirculation #4 Section 16.2.3.12 |  |
| Abstract | This contribution proposes to include DCD/UCD information in the AAI-HO-CMD message to support handover to the legacy BS. |  |
| Purpose | To be discussed and adopted by WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# DCD/UCD information in the AAI-HO-CMD message (Section 16.2.3.12)

Jaehyuk Jang, Hyunjeong Kang, Rakesh Taori
Samsung Electronics

# Introduction

This contribution proposes to include DCD/UCD information in the AAI-HO-CMD message to support handover to the legacy BS.

In D10, there are two ways to obtain information of neighbor legacy BSs

- To obtain the DCD/UCD messages from LZone of the serving ABS

- To obtain essential information (i.e., preamble index, FA index) from the AAI-NBR-ADV message.

The 1st option requires for an AMS to perform EBB operation to receive the information from LZone. If an AMS has no capability to perform EBB operation, the AMS needs to go with the 2nd option. In the 2nd option, because the AMS has no information of detailed DCD/UCD messages, the AMS needs to wait until receiving DCD/UCD message in the target legacy BS before performing network reentry to the target legacy BS, and the handover latency cannot meet the SRD (5.1 Legacy Support).

To solve the problem, DCD/UCD information should be included in the AAI-HO-CMD message. Following DCD/UCD parameters are currently used in an R1 BS.

- DCD (Bytes)

  - Configuration Change Count	1,

  - Downlink_Burst_Profile	Nx2,

  - BS EIRP	2,

  - TTG	2,

  - RTG	1,

  - EIRxPIR,max	2,

  - BS Restart Count	1

- UCD (Bytes)

  - Configuration Change Count	1,

  - Uplink_Burst_Profile	Nx2,

  - Contention-based reservation timeout	1,

  - Ranging Backoff Start	1,

  - Ranging Backoff End	1,

  - Request Backoff Start	1,

  - Request Backoff End	1,

  - HO_ranging_start	1,

  - HO_ranging_end	1,

  - Periodic ranging backoff start	1,

  - Periodic ranging backoff end	1,

  - Initial ranging codes	1,

  - Periodic ranging codes	1,

  - Bandwidth request codes	1,

  - Handover Ranging Codes	1,

  - Start of ranging codes group	1,

  - Permutation base	1,

  - HARQ ACK delay for DL burst	1,

  - Size of CQICH_ID field	1,

  - Normalized C/N override 2	8,

  - Initial ranging interval	1,

  - Tx Power Report	3,

  - Relative_Power_Offset_For_UL_HARQ_burst	1,

  - UL_initial_transmit_timing	1,

  - Fast Feedback Region	5,

  - HARQ Ack Region	4,

  - Ranging Region	10

In typical case, the overhead is 82 bytes (23 bytes for DCD (7 DIUCs used), 59 bytesfor UCD (4 UIUCs used)) per BS, and normally one or a few BS will be included in the HO-CMD message.

# Proposed Text

Adopt the following Remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Update Table 689 in pp. 133, line 10 in D10 as follows:]

16.2.3.12 AAI-HO-CMD

…

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| S-SFH application indicator | 1 | Indicate if the SFH delta information at the T-ABS is applied at the Action Time 0b0: not applied 0b1: applied | Shall be included when SFH delta information is included |
| DCD | Variable (size is provided in the ASN.1 code) | Following parameters from the DCD message may be included. - Configuration Change Count - Downlink_Burst_Profile - BS EIRP - TTG - RTG - EIRxPIR,max - BS Restart Count | Optional. May be included if the target BS is a legacy BS. |
| UCD | Variable (size is provided in the ASN.1 code) | Following parameters from the UCD message may be included. - Configuration Change Count - Uplink_Burst_Profile - Contention-based reservation timeout - Ranging Backoff Start - Ranging Backoff End - Request Backoff Start - Request Backoff End - HO_ranging_start - HO_ranging_end - Periodic ranging backoff start - Periodic ranging backoff end - Initial ranging codes - Periodic ranging codes - Bandwidth request codes - Handover Ranging Codes - Start of ranging codes group - Permutation base - HARQ ACK delay for DL burst - Size of CQICH_ID field - Normalized C/N override 2 - Initial ranging interval - Tx Power Report - Relative_Power_Offset_For_UL_HARQ_burst - UL_initial_transmit_timing - Fast Feedback Region - HARQ Ack Region - Ranging Region | Optional. May be included if the target BS is a legacy BS. |
| }//end of for(i=0; i < N_Target_BS; i++) |  |  |  |
| … |  |  |  |

[Remedy #2: Update ASN.1 code based on the updated message table in pp. 986, line 60 as follows:]

PhyCarrierIndex ::= INTEGER (0..62)

PreambleIndex ::= INTEGER (0..1023)

PreambleIndex2 ::= INTEGER (0..63)

PreambleIndex3 ::= INTEGER (0..127)

LegacyPreambleIndex ::= INTEGER (0..127)

[Remedy #7: Update ASN.1 code based on the updated message table in pp. 1044, line 13 as follows:]

TargetBSInfo ::= SEQUENCE {

…

sfhDeltaInfo SEQUENCE {

sfhSubpacket1 OptSFHSubpacket1 OPTIONAL,

sfhSubpacket2 OptSFHSubpacket2 OPTIONAL,

sfhSubpacket3 OptSFHSubpacket3 OPTIONAL

} OPTIONAL

},

legacyDCD LegacyDCD OPTIONAL,

legacyUCD LegacyUCD OPTIONAL

}

LegacyDCD ::= SEQUENCE {

ccc	INTEGER (0..255),

dlBurstProfile	SEQUENCE (SIZE (1..15)) OF LegacyDLBurstProfile OPTIONAL,

bsEIRP	INTEGER (0..65535) OPTIONAL,

ttg	INTEGER (0..65535) OPTIONAL,

rtg	INTEGER (0..255) OPTIONAL,

eirxPIR,max	INTEGER (0..65535) OPTIONAL,

bsRestartCount	INTEGER (0..255) OPTIONAL

}

LegacyUCD ::= SEQUENCE {

ccc	INTEGER (0..255),

ulBurstProfile	SEQUENCE (SIZE (1..15)) OF LegacyULBurstProfile OPTIONAL,

cbReservationTimeout	INTEGER (0..255) OPTIONAL,

rngBackoffStart	INTEGER (0..255) OPTIONAL,

rngBackoffEnd	INTEGER (0..255) OPTIONAL,

reqBackoffStart	INTEGER (0..255) OPTIONAL,

reqBackoffEnd	INTEGER (0..255) OPTIONAL,

hoRngStart	INTEGER (0..255) OPTIONAL,

hoRngEnd	INTEGER (0..255) OPTIONAL,

periodicRngStart	INTEGER (0..255) OPTIONAL,

periodicRngEnd	INTEGER (0..255) OPTIONAL,

initialRngCodes	INTEGER (0..255) OPTIONAL,

periodicRngCodes	INTEGER (0..255) OPTIONAL,

bwReqCodes	INTEGER (0..255) OPTIONAL,

hoRngCodes	INTEGER (0..255) OPTIONAL,

startRngCodesGroup	INTEGER (0..255) OPTIONAL,

permBase	INTEGER (0..255) OPTIONAL,

harqACKdelayforDL	INTEGER (0..255) OPTIONAL,

Size of CQICH_ID field	INTEGER (0..255) OPTIONAL,

normC2Noverride2	BIT STRING (SIZE (64)) OPTIONAL,

initialRngInterval	INTEGER (0..255) OPTIONAL,

txPowerReport	BIT STRING (SIZE (24)) OPTIONAL,

relPowOffsetULHARQ	INTEGER (0..255) OPTIONAL,

ulInitialTXtiming	INTEGER (0..255) OPTIONAL,

fastFeedbackRegion	BIT STRING (SIZE (40)) OPTIONAL,

harqAckRegion	BIT STRING (SIZE (32)) OPTIONAL,

rngRegion	BIT STRING (SIZE (80)) OPTIONAL

}

LegacyDLBurstProfile ::= SEQUENCE {

diuc	INTEGER (0..15),

mcs	INTEGER (0..255)

}

LegacyULBurstProfile ::= SEQUENCE {

uiuc	INTEGER (0..15),

mcs	INTEGER (0..255)

}

------------------------------------------------------ End of Proposed Text ------------------------------------------------------