| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Cleanup AAI-LBS-IND message and corresponding ASN.1 code ( 16.2.3) |  |
| Date Submitted | 2011-01-11 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Hai Wang China Samsung Telecom R&D Center Sangheon Kim, Hyunjeong Kang, Ji-yun Seol, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D10 |  |
| Abstract | This contribution proposes to Cleanup ASN.1 code of AAI-LBS-IND message( Annex R) |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Cleanup AAI-LBS-IND message and corresponding ASN.1 code (16.2.3)

Lei Zhou, Xufeng Zheng, Hai Wang

China Samsung Telecom R&D Center

Sangheon Kim, Hyunjeong Kang, Ji-yun Seol, Jaeweon Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes to clarify condition in AAI-LBS-IND message and ASN.1 code of AAI-LBS-IND message, because ASN.1 code of AAI-LBS-IND message can’t be correctly implemented in D10. AAI-LBS-IND message need support both general U-TDOA and special U-TDOA so ASN.1 code related on neighbor ABS parameters of AAI-LBS-IND message should be optional. We

The Proposed Text in AWD

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy 1: Replace the Table 757 from line# 49 Page 296 in P802.16m/D10 with the followings:]

Table 757—AAI-LBS-IND Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 1 | 0x0: Special U-TDOA 0x1: General U-TDOA |  |
| Frame identifier | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe |  |
| Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |  |
| Ranging opportunity index for the serving ABS | 3 | Indicates the index of the allocated ranging opportunity. |  |
| Subframe Index for the serving ABS | 2 | Indicates the subframe index of the allocated ranging opportunity. |  |
| If (Action code == 0x1) { |  |  |  |
| Neighbor_ABS_Info_Array | 8 | List of neighbor ABS Info | Presented when Action code == 0x1 |
| For (i=0; i< Neighbor_ABS_Info_Array; i ++){ |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to AAI-NBR-ADV message | Presented when Action code == 0x1 is used |
| Action time | 8 | Action Time included in this message is the absolute frame number at the serving ABS. | Presented when Action code == 0x1 is used |
| Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code | Presented when Action code == 0x1 is used |
| Ranging opportunity index of neighbor ABS | 3 | Indicates the index of the allocated ranging opportunity | Presented when Action code == 0x1 is used |
| Subframe Index of neighbor ABS | 2 | Indicates the subframe index of the allocated ranging opportunity | Presented when Action code == 0x1 is used |
| } |  |  |  |
| } |  |  |  |

------------------------------- Text Proposal 1 Start ---------------------------------------------------

------------------------------- Text Proposal 2 Start ---------------------------------------------------

[Remedy2: Modify ASN.1 Code between line 32-51 on Page 1088 in P802.16m/D10 with the followings:]

AAI-LBS-IND ::=             SEQUENCE {

frameIdentifier             INTEGER (0..15),

dedicatedRangingCode        INTEGER (0..31),

rangingOppIndex             INTEGER (0..7),

subframeIndex               INTEGER (0..3),

nbrAbsInfoList              SEQUENCE (SIZE (0..255)) OF SEQUENCE {

nbrAbsIndex                 INTEGER (0..255),

actionTime                  INTEGER (0..255),

nbrDedicatedRangingCode     INTEGER (0..31),

nbrRangingOppIndex          INTEGER (0..7),

nbrSubframeIndex            INTEGER (0..3)

} OPTIONAL,

...

}

------------------------------- Text Proposal 2 End ---------------------------------------------------