| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of some texts related to E-MBS (16.9) |  |
| Date Submitted | 2011-1-11 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com , kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D10 |  |
| Abstract | This contribution proposes the some texts related to E-MBS service |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of some texts related to E-MBS (16.9)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak

- LG Electronics

# Introduction

Some texts related E-MBS service should be cleaned up. For example, an ABS does not allocate the carrier switching time when an AMS stops carrier switching mode and the carrier switching time will be also allocated in E-MBS connection report. And, in E-MBS carrier switching mode E-MBS streams report bears no relation to the E-MBS connection releasing. That is, it will be performed regardless of E-MBS connection releasing.

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested remedy #1: Modify the related sentences in Section 16.9.2.4 as follows, On page 968, line 34 :]

1. to request a start time from the ABS at which AMS can switch to the E-MBS carrier after DSx transaction

2. to update the indication regarding the E-MBS stream(s) the AMS is currently receiving <del> without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP </del>

3. to indicate that the AMS stops E-MBS carrier switching without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP

[Suggested remedy #2: Modify the related sentences in Section 16.9.2.1 as follows, On page 965, line 55 :]

The AAI-E-MBS-REP message described in 16.2.3.59 shall be transmitted from an AMS to an ABS in order to request a start time, to update the receiving E-MBS stream(s), or to indicate <del> the time at which </del> <ins>that</ins> the AMS stops E-MBS carrier switching without releasing the E-MBS connection.

[Suggested remedy #3: Modify the Section 16.2.3.59/16.2.3.60 as follows, On page 288, line 35 :]

16.2.3.59 AAI-E-MBS-REP

An AMS shall transmit AAI-E-MBS-REP message to inform ABS when one of the following conditions are met:

1. to request a start time from the ABS at which AMS can switch to the E-MBS carrier after DSx transaction

2. to update the indication regarding the E-MBS stream(s) the AMS is currently receiving <del> without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP</del>  when the carrier switching mode is set to 0b1 in the AAI-DSA-REQ/RSP message. This interval indication is carried in the E-MBS connection bitmap. The ABS uses the connection bitmap to compute the interval during which the AMS is available at the primary carrier for unicast scheduling.

3. To indicate that the AMS stops E-MBS carrier switching without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP

The AAI-E-MBS-REP message also carries E-MBS Zone ID of the E-MBS Zone from which the AMS is currently receiving E-MBS data

Table 753—AAI-E-MBS-REP Message Field Description

| Attributes/Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- |
| E-MBS Zone ID | 7 | Indicates E-MBS Zone from which the AMS is currently receiving E-MBS data |  |
| Report Mode | 2 | Indicates the AMS starts/changes/ends E-MBS 0b00: AMS requests ABS to assign a carrier switching start time 0b01: AMS updates E-MBS connection Bitmap and requests ABS to assign a carrier switching start time 0b10: AMS ends E-MBS carrier switching 0b11: reserved |  |
| E-MBS Connection Bitmap | 16 | Each bit of the bitmap represents an E-MBS connection for which service flows have been established using AAI-DSx transactions in one E-MBS Zone. The E-MBS service(s) are mapped in ascending order of their E-MBS ID+FID value from LSB to MSB of the bitmap. For each bit: Value 0: The AMS does not intend to receive the E-MBS service. Value 1: The AMS currently receives E-MBS or the AMS may switch to this service in the near future. | Shall be included if the value of Report Mode is 0b01. |

16.2.3.59 AAI-E-MBS-RSP

The AAI-E-MBS-RSP message shall be transmitted by the ABS in response to an AAI-EMBS-REP message sent by the AMS. It shall include the following parameters.

- Carrier Switching Start Time: 4 LSBs of superframe number at which carrier switching happens

- Unicast Available Interval Bitmap: Indicates when the AMS should be available in the primary carrier for unicast scheduling. The ABS uses the connection bitmap in the AAI-E-MBS-REP message to compute the unicast available interval bitmap.

Table 754—AAI-E-MBS-RSP Message Field Description

| Attributes/Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- |
| Carrier Switching Start time | 4 | 4 LSBs of superframe number at which the AMS starts carrier switching operation when the report mode is 0b00 or 0b01 | Present when the report mode in AAI-E-MBS-REP message is 0b00 or 0b01 |
| Unicast Available Interval Bitmap | Variable (maximum: 16) | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present when the report mode AAI-E-MBS-REP message is 0b01 |

--------------Proposed Texts  ---------------------------------------------------------------------------