| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for ASN.1 code of Reference Carrier in AAI-MC-ADV message |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Kiseon Ryu LG electronics | Voice : +82 31 450 1876 E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D10 |  |
| Abstract | The contribution proposes ASN.1 code of AAI-MC-ADV message |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for ASN.1 code of Reference Carrier 
in AAI-MC-ADV message

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG electronics

- Introduction

This contribution proposes ASN.1 code for Reference Carrier in AAI-MC-ADV message.

- Suggested changes in the 802.16m/D10

[Modify text in Annex R.2 in page 1064, line 61 with the following text: ]

ServingABSCarrierInfo ::= SEQUENCE {

pagingGroupID	 	PGID,

carrierIndex 			PhyCarrierIndex,

saPreambleIndex 		PreambleIndex,

pagingCarrierIndicator 	ENUMERATED {

noPagingCarrier,

pagingCarrier

},

referenceCarrier		PhyCarrierIndex,		OPTIONAL,

caSpecificTrigger 		CASpecificTriggers 		OPTIONAL,

pccSpecificTrigger 		Triggers 			OPTIONAL,

sSFHChangeCount 		INTEGER (0..15)		OPTIONAL,

…

- References

[1] IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”