| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS Scheduling Interval (MSI) in IEEE P802.16m |  |
| Date Submitted | 2011-01-12 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak LG Electronics Chiu-Wen Chen, Chun-Yen Hsu Institute for Information Industry Alessandro Triglia OSS Nokalva | E-mail: ekkim@etri.re.kr jeongki.kim@lge.com cwchen@nmi.iii.org.tw sandro@oss.com |
| Re: | “P802.16m/D10,” in response to the IEEE 802.16 Working Group Sponsor Ballot Recirc #4 |  |
| Abstract | To be clear E-MBS Scheduling Interval (MSI) length on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Scheduling Interval (MSI) in IEEE P802.16m

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak

LG Electronics

Chiu-Wen Chen, Chun-Yen Hsu

Institute for Information Industry

Alessandro Triglia

OSS Nokalva

# Introductions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS) operation. According to the current IEEE 802.16m Amendment Draft Standard [1] and agreement on session #70, MSI as well as E-MBS ID and FID flow mapping should be updated properly.

# MSI

For each E-MBS Zone there is an E-MBS scheduling interval (MSI), which is in units of number of superframes: 4, 8, 16, and 32.

Therefore, text about the MSI length should be written with proper value in the IEEE P802.16m as follows:

- AAI_SCD

- AAI_DSA-REQ/RSP

- AAI_E-MBS-RSP

- Annex R.2 MAC Control Message Definitions

# E-MBS ID and FID flow mapping

E-MBS ID and FID flow mapping information should be updated properly as follows:

- It should be included in AAI_RNG-RSP in case of location update procedure of idle mode described in 16.9.2.3.

- It should not be included in AAI_RNG-RSP and related ASN.1 in case of HO network reentry.

# References

- IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” November 2010

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adapt the following change in page 191, Table 708-AAI-SCD in 16.2.3.31 of 802.16m/D10]

Table 708—AAI-SCD Message Field Description

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| If (E-MBS is configured) { |  |  |  |
| Zone_Allocation-Bit-MAP | 20 or 9 or 3 | The number of bits in the bit map = the maximum number of sub-bands for a given bandwidth (i.e., 20MHz, 10MHz, 5MHz) - 1. 20 MHz: b0-b19 10 MHz: b0-b8 5 MHz: b0-b2 bi = 1 if resource is changed between sub-band i and sub-band i+1. bi = 0 if resource is not changed between sub-band i and sub-band i+1. See section 16.9.3.1. | … |
| ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See Section 16.9.3.1 |  |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) 0b00: 4 superframes, 80 ms (NMSI = 4) 0b01: 8 superframes, 160 ms (NMSI = 8) 0b10: 16 superframes, 320 ms (NMSI = 16) 0b11: 32 superframes, 640 ms (NMSI = 32) |  |
| E-MBS AAI frame offset | Variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits MSI length == 0b00: 4 bits MSI length == 0b01: 5 bits MSI length == 0b10: 6 bits MSI length == 0b11: 7 bits |  |
| } |  |  |  |
| … |  |  |  |

[Remedy2: Adapt the following change in page 233, Table 734-AAI-DSA-REQ in 16.2.3.47.1 of 802.16m/D10]

Table 734—AAI-DSA-REQ Message Field Description

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAIDSA message 0b1: carrier switching parameters using AAI-E-MBS-REP message | Present when ABS-initiated DSA for carrier switching |
| If(Carrier Switching Mode==0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present only if AMS is operated in carrier switching and if Carrier Switching Mode = 0b0 |
| } | - |  |  |
| … |  |  |  |

[Remedy3: Adapt the following change in page 241, Table 735-AAI-DSA-RSP in 16.2.3.47.2 of 802.16m/D10]

Table 735—AAI-DSA-RSP Message Field Description

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAIDSA message 0b1: carrier switching parameters using AAI-E-MBS-REP message | Present if ABS indicates carrier switching when receiving AMS-initiated DSA |
| If(Carrier Switching Mode==0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present only if AMS is operated in carrier switching and if Carrier Switching Mode = 0b0 |
| } | - |  |  |
| … |  |  |  |

[Remedy4: Adapt the following change in page 290, Table 754-AAI-E-MBS-RSP in 16.2.3.60 of 802.16m/D10]

Table 754—AAI-E-MBS-RSP Message Field Description

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| Carrier Switching Start time | 4 | 4 LSBs of superframe number at which the AMS starts carrier switching operation when the report mode is 0b00 | Present when the report mode in AAI-E-MBS-REP message is 0b00 or 0b01 |
| Unicast Available Interval Bitmap | Variable (maximum:1632) | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present when the report mode in AAI-E-MBSREP message is 0b01 |
| } | - |  |  |
| … |  |  |  |

[Remedy5: Adapt the following change in line #42-65, page 990 of 802.16m/D10]

-- The length of an MSI in units of the number of superframes

-- 0b00: 2 superframes, 40 ms (NMSI = 2)

-- 0b01: 4 superframes, 80 ms (NMSI = 4)

-- 0b10: 8 superframes, 160 ms (NMSI = 8)

-- 0b11: 16 superframes, 320 ms (NMSI = 16)

MsiLength ::= INTEGER (2 | 4 | 8 | 16)

-- 0b00: 4 superframes, 80 ms (NMSI = 4)

-- 0b01: 8 superframes, 160 ms (NMSI = 8)

-- 0b10: 16 superframes, 320 ms (NMSI = 16)

-- 0b11: 32 superframes, 640 ms (NMSI = 32)

MsiLength ::= INTEGER (4 | 8 | 16 | 32)

-- The location of the AAI frame where the E-MBS data burst ends

-- MSI length == 0b00: 3 bits

-- MSI length == 0b01: 4 bits

-- MSI length == 0b10: 5 bits

-- MSI length == 0b11: 6 bits

-- MSI length == 0b00: 4 bits

-- MSI length == 0b01: 5 bits

-- MSI length == 0b10: 6 bits

-- MSI length == 0b11: 7 bits

EmbsFrameOffset ::= CHOICE {

msiLength2FrameOffset INTEGER (0..7),

msiLength4FrameOffset INTEGER (0..15),

msiLength8FrameOffset INTEGER (0..31),

msiLength16FrameOffset INTEGER (0..63),

msiLength32FrameOffset INTEGER (0..127)

}

[Remedy6: Adapt the following change in line #1-45, page 1028 of 802.16m/D10]

AAI-DSA-REQ ::= SEQUENCE {

fidChangeCount FidChangeCount,

absInitDsaInfo AbsInitDsaInfo OPTIONAL,

directionIndicator DirIndicator,

qoSParameters QosParameter OPTIONAL,

additionalSfInfo AdditionalSfInfo OPTIONAL,

emergencyIndication BOOLEAN OPTIONAL,

embsService EMBSService OPTIONAL,

fullEMBSIDArray SEQUENCE (SIZE (1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex,

embsidFIDMappingList SEQUENCE (SIZE (1..15)) OF SEQUENCE {

embsid EMBSID,

fid FID

}

},

unicastAvailIntervalBitmap UnicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

} OPTIONAL,

groupParameterCreateChange GroupParaCreateChange,

coupledGroupCreateChange CoupledGroupCreateChange,

multicastGroup SEQUENCE (SIZE (1..16)) OF SEQUENCE {

multicastGroupId INTEGER (0..4095),

fid INTEGER (0..15)

} OPTIONAL,

sleepCycleSetting SleepCycleSetting OPTIONAL,

harqChannelsList SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) OPTIONAL,

...

}

[Remedy7: Adapt the following change in line #14-21, page 1029 of 802.16m/D10]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSA Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DSA-RSP ::= SEQUENCE {

fidChangeCount FidChangeCount,

confirmationCode ConfirmationCode,

fid FID OPTIONAL,

groupParameterCreateChange SEQUENCE {

fidArray SEQUENCE (SIZE (1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL

}

} OPTIONAL,

embsService EMBSService OPTIONAL,

fullEMBSIDArray SEQUENCE (SIZE (1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex,

embsidFIDMappingArray SEQUENCE (SIZE (1..15)) OF SEQUENCE {

embsid EMBSID,

fid FID

}

},

carrierSwitching CHOICE {

unicastAvailIntervalBitmap UnicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

},

aaiEmbsRepMsg NULL

},

multicastGroup SEQUENCE (SIZE (1..16)) OF SEQUENCE {

multicastGroupId INTEGER (0..4095)

} OPTIONAL,

sleepCycleSetting SleepCycleSetting OPTIONAL,

...

}

[Remedy8: Adapt the following change in line #42-50, page 1030 of 802.16m/D10]

unicastAvailIntervalBitmap UnicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

} OPTIONAL,

[Remedy9: Adapt the following change in line #61-65, page 1086 of 802.16m/D10]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- E-MBS Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-EMBS-RSP ::= SEQUENCE {

carrierSwitchingStartTime INTEGER (0..15) OPTIONAL,

-- Each bit indicates which service is available during each superframe

-- 0: the AMS is available for E-MBS data in secondary carrier

-- 1: the AMS is available for unicast data in primary carrier

unicastAvailIntervalBitmap UnicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

} OPTIONAL,

...

}

[Remedy10: Insert the following text at line #51, page 1026 of 802.16m/D10]

UnicastAvailIntervalBitmap ::= CHOICE {

nmsi4     BIT STRING (SIZE (4)),

nmsi8     BIT STRING (SIZE (8)),

nmsi16    BIT STRING (SIZE (16)),

nmsi32    BIT STRING (SIZE (32))

}

[Remedy #11: Modify Table 679 as follows, on page 98, line 27:]

Table 679 – AAI_RNG-RSP Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| …… | … | …… | … |
| New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| for(i=1;i<= N-E-MBS-Zone-IDs ;i++) { |  | N-E-MBS-Zone-IDs is the number of E-MBS Zone IDs to be updated in the T-ABS [1..8] | Presented if it needs to be updated |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| for(i=1;i<= N-E-MBS-Zone-IDs ;i++) { |  | N-E-MBS-Zone-IDs is the number of E-MBS Zone IDs to be updated in the T-ABS [1..8] | Presented if it needs to be updated |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
| If (Service_flow_update_indicator == 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |  |
| For each 1-bit in the Service Flow Update Bitmap{ |  |  |  |
| New_E-MBS ID | 12 | New E-MBS identifier. |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| }else if (Service_flow_update_indicator == 0b1) { |  |  |  |
| for(j=0;j<N_E-MBS_IDs;j++) { |  | N_EMBS_IDs is the number of EMBS IDs listed here Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current E-MBS ID | 12 |  |  |
| Current FID | 4 |  |  |
| New_E-MBS ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| } // end of if |  |  |  |
| }//end of for (N-E-MBS-Zone- IDs) |  |  |  |
| For (i=0; i< M; i++) { | - | Number of Multicast Group ID and FID (M) to update in the TABS[ 1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |

[Remedy#12: Modify the Section 16.9.2.2 as follows, on page 966, the line #61.]

16.9.2.2 E-MBS Operation in Connected State

When an AMS moves across E-MBS zone boundaries in Active Mode or Sleep Mode, the AMS performs the handover procedure described in 16.2.6.3.

When the AMS transits to a new E-MBS Zone while in Active Mode or Sleep Mode, and the E-MBS service flow management encodings of the AMS have not been updated, the AMS shall send AAI-RNG-REQ message with Ranging Purpose Indication = 0b0001 at the T-ABS and the ABS shall include E-MBS ID and FID Update in AAI-RNG-RSP parameters to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

Once an AMS has received the E-MBS allocation information in the E-MBS-MAPs, it may not listen to the downlink channels till the next transmission of desired E-MBS flow or the next E-MBS-MAP.

The AMS does monitor down link channels and follow the entire idle mode and connected mode procedures as required by other services. While receiving E-MBS data, the AMS may temporary interrupt the reception of E-MBS packets for any reason, e.g. if time critical scanning is needed, without notifying the ABS.

[Remedy#13: Modify the line#12 to the line #35 on page 999 as follows,.]

EMBSZoneInfoInHandover ::= SEQUENCE {

serviceFlowUpdateIndicator BOOLEAN,

embsZoneID BIT STRING (SIZE(7)),

newEMBSZoneID BIT STRING (SIZE(7)) OPTIONAL,

physicalCarrierIndex INTEGER(0..63) OPTIONAL,

bitmapAndServiceFlowInfo BitmapAndSfInfo

}

RngRspForHoReentryInfo ::= SEQUENCE {

embsZoneInfoList SEQUENCE (SIZE(0..7)) OF EMBSZoneInfoInHandover,

multicastInfo SEQUENCE(SIZE(0..15)) OF SEQUENCE {

currentMulticastGroupID BIT STRING(SIZE(12)),

currentFID BIT STRING(SIZE(4)),

newMulticastGroupID BIT STRING(SIZE(12)),

newFID BIT STRING(SIZE(4))

} OPTIONAL

}

[-------------------------------------------------End of Text Proposal----------------------------------------------------]