| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clarifications to the UL Feedback Channel Allocation and Signaling for DL-only Carrier in Multicarrier Operations (16.2.8.2.8) |  |
| Date Submitted | 2010-12-16 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-4 for P802.16m/D10. |  |
| Abstract | The contribution proposes clarifications to the UL feedback channel allocation for DL-only carrier in multicarrier operations. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarifications to the UL Feedback Channel Allocation and Signaling for DL-only Carrier in Multicarrier Operations (16.2.8.2.8)

Lei Wang

InterDigital Communications, LLC

- Introduction

This contribution proposes some follow-up clarifications to the accepted resolution to the comment C130/contribution 1348r1 in 802.16m commentary database 80216-10_0052r3, as we don't fully agree with it.

The contribution 1348r1 provides the discussions and proposal about where/how the MAC level feedback signals, .e.g, extended headers and MAC messages, are sent for the DL-only carriers, which we agree with. However, for the PHY level feedback signals, e.g., HARQ ACK/NAK, fast feedback channel, the contribution 1348r1 deleted the relevant text, which leaves the PHY level feedback for DL-only carrier incomplete.

In addition, the two paragraphs in line 42 on page 385 need some clarifications regarding how the PHY feedback regions for the DL-only carriers is allocated and signaled. Note that the proposed clarifications are based on the following understanding:

- UL feedback, including HARQ ACK/NAC and fast feedback channel, for a DL-only carrier is transmitted in an UL feedback region on the primary UL carrier of the AMS;

- To an AMS, the allocation of the UL feedback region for an active DL-only carrier is signaled by the AAI-SCD message transmitted on the active DL-only carrier;

- To an AMS, the allocations of the feedback channel for an active DL-only carrier stays the same as that for the primary carrier, i.e., HARQ ACK/NAK channel allocated by the basic/subband/GRP/PA allocation A-MAP IEs, and the fast feedback channel allocated by the Feedback Allocation A-MAP IE; and transmitting those A-MAP IEs on the active DL-only carrier; note that in this way these A-MAP IE stay unchanged, particularly does not need to include a “carrier index” field.

- For an AMS, the same signaling mechanisms are used for an active DL-only partially configure carrier and an active DL-only fully configured carrier regarding the allocation of the UL feedback region and the allocation of the UL feedback channels.

The following are the proposed clarification text changes based on the above understanding.

- Suggested changes in the 802.16m/D10

The following is the proposed change in the 802.16m/D10. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 42, page 385

Change the two paragraphs in line 42 on page 385 as follows:

When only DL of fully configured carrier has been activated, UL feedback channel assigned by the ABS is located at the UL region defined in the SFH on the primary carrier. In this case, the ABS may allocate for an AMS one UL feedback channel per a secondary carrier over a primary carrier. If the AMS receives Feedback Allocation A-MAP IE through the DL only activated carrier, the AMS transmits the UL feedback information for the secondary carrier over the assigned UL Fast Feedback channel of the primary carrier.

In multicarrier aggregation with an active DL-only secondary partially configured carrier, either a DL-only activated fully configured carrier or a DL-only partially configured carrier for DL unicast traffic, the ABS directs the AMS to feedback HARQ ACK/NACK and fast feedback channel with channel quality measurements CINRs of the active DL-only secondary carriers through feedback channel(s) on the primary carrier at the feedback region allocated for the DL-only secondary carrier.  The feedback region is indicated using the physical carrier index for the primary carrier, the start DLRU index for feedback channel, the number of DLRUs for feedback channel and the number of HARQ feedback channel per HARQ feedback region in the AAI-SCD message that is transmitted on the active DL-only secondary partially configured carrier. The feedback region of the active DL-only secondary partially configured carrier follows the feedback region of the primary carrier. In multicarrier aggregation with multiple active DL-only secondary carriers for DL unicast traffic, each active DL-only carrier is allocated with a distinct non-overlapping UL feedback region in the UL of the primary carrier for the AMSs to transmit HARQ ACK/NACK and fast feedback information.

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”