| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MAC control messages related to NE or Security |  |
| Date Submitted | 2011-01-11 |  |
| Source(s) | Youngkyo Baek, Yeongmoon Son, Hyunjeong Kang Samsung Electronics | youngkyo.baek@samsung.com |
| Re: | Proposed text changes to P802.16m/D10 |  |
| Abstract | This contribution proposes text for clarification on network entry messages regarding condition column. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D11 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MAC control messages related to NE or Security (16.2.3)

Youngkyo Baek, Yeongmoon Son,Hyunjeong Kang

Samsung Electronics

- Backgrounds

This contribution clarifies the network entry or security related MAC control messages regarding ‘condition’ column for each attribute field. If we see the MAC control message format table, some attributes’ condition columns are not filled so that it is not easy to understand whether those attributes are optional or mandatory. Hence I suggest filling all condition columns for each attributes with their corresponding condition.

AAI-RNG-REQ/RSP/CFM/ACK, AAI-SBC-REQ/RSP, AAI-REG-REQ/RSP and AAI-PKM-REQ/RSP messages are handled here.

Additionally, we suggest adding some missing attributes such as UL/DL indicator to verify some FID allocation in AAI-RNG-REQ message(e.g. remedy#1) and modifying their related ASN.1 code(e.g. remedy#11), which are marked by yellow-colored background.

C802.16m-10_1428r2: add remedy #5 to clarify Condition field and delete unnecessary remedies from the previous version (i.e., 1428r1)

- Proposed Text

Remedy #1: Modify line 38 of page 88 (122) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 678—— AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode …. …. |  |
| … | … | … | … |
| AMSID* | ….. | … |  |
| … | … | … | … |
| AMS MAC address |  |  |  |
| } |  |  |  |
| MAC version | 8 |  |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 |  |  |
| … | … | … | … |
| Serving BSID |  |  |  |
| Previous STID |  |  |  |
| } else{ |  |  |  |
| if (legacy network mode or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only |
| } else{ |  |  |  |
| STID |  |  |  |
| … | … | … | … |
| [line 15,page90] For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of transport FIDs which the AMS has. 1 ≤ N_FIDs ≤ 15 24 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| … | … | … | … |
| Paging Controller ID |  |  |  |
| PGID |  |  |  |
| Paging Cycle |  |  |  |
| Paging Offset |  |  |  |
| … | … | … | … |
| [line 5,page91] For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of transport FIDs which the AMS has. 1 ≤ N_FIDs ≤ 15 24 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| … | … | … | … |
| Paging Controller ID |  |  |  |
| PGID |  |  |  |
| Paging Cycle |  |  |  |
| Paging Offset |  |  |  |
| … | … | … | … |
| CRID |  |  |  |
| … | … | … | … |
| AMS MAC address |  |  |  |
| MAC version | 8 |  |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 |  |  |
| … | … | … | … |
| CRID |  |  |  |
| … | … | … | … |
| [line 58,page92] For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of transport FIDs which the AMS has. 1 ≤ N_FIDs ≤ 15 24 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| … | … | … | … |
| AMSID* | ….. | … |  |
| } |  |  |  |
| Serving BSID |  |  |  |
| Previous Basic CID |  |  |  |
| … | … | … | … |
| TSTID |  |  |  |
| If (AMSID privacy is enabled) { |  |  |  |
| AMSID* |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| Serving BSID |  |  |  |
| Previous Basic CID |  |  |  |
| If (AMSID privacy is enabled) { |  |  |  |
| AMSID* |  |  |  |
| } |  |  |  |
| … | … | … | … |
| AMS MAC address |  |  |  |
| MAC version |  |  |  |
| Initial Offset for uplink power control (OffsetInitial) |  |  |  |
| CSGID |  |  |  |
| … | … | … | … |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #2: Modify line 6 of page 96 (130) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 679—— AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Ranging Abort Timer |  |  |  |
| … | … | … | … |
| Current Multicast Group ID |  |  |  |
| Current FID |  |  |  |
| New Multicast Group ID |  |  |  |
| New FID |  |  |  |
| … | … | … | … |
| SFID |  |  |  |
| FID |  |  |  |
| Update or delete |  |  |  |
| … | … | … | … |
| [line 23,page101] for (i=0; i<N_CSG_IDs; i++) { |  |  | Optional. If included, to help the AMS make the decision if the Femto ABS is accessible or not. |
| CSGID |  |  | Optional. If included, to help the AMS make the decision if the Femto ABS is accessible or not. |
| } |  |  |  |
| For (i=0; i<N_Redirect_Info; i++) { |  |  |  |
| ABSID for neighbor ABS |  |  |  |
| Preamble index for neighbor ABS |  |  |  |
| Center frequency for neighbor ABS |  |  |  |
| } |  |  |  |
| Ranging Request bit |  |  | … |
| For (i=0; i<N_Invalid_FIDs; i++) { |  | N_Invalid_FIDs is the number of invalid FIDs listed here. The maximum number of N_Invalid_FIDs is 24. | Shall be included if there is any invalid FID. Invalid FID is a FID whose FID change count in AAI-RNGREQ is different from FID change count stored at the ABS. ABS and AMS may exchange the AAI-DSx-messages for invalid FIDs |
| FID | 4 |  | Shall be included if there is any invalid FID. Invalid FID is a FID whose FID change count in AAI-RNGREQ is different from FID change count stored at the ABS. ABS and AMS may exchange the AAI-DSx-messages for invalid FIDs |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| … | … | … | … |
| Current Multicast Group ID |  |  |  |
| Current FID |  |  |  |
| New Multicast Group ID |  |  |  |
| New FID |  |  |  |
| … | … | … | … |
| [line 6,page103] N_SFIDs |  | Number of SFIDs supported in MZone when an AMS performs Zone Switching from LZone to MZone | Present if CID to FID mapping is done through the AAI-RNGRSP message during Zone Switching operation. If this field is not present, all FIDs for the transport connection should be reestablished through the AAI-DSA exchanges after completion of network reentry in MZone. |
| For (i=0; i<N_SFIDs; i++) { |  | N_SFIDs is Number of SFIDs supported in MZone when an AMS performs Zone Switching from LZone to MZone. Its maximal number is 24. | Present if CID to FID mapping is done through the AAI-RNG-RSP message during Zone Switching operation. If this field is not present, all FIDs for the transport connection should be reestablished through the AAI-DSA exchanges after completion of network reentry in MZone. |
| SFID | 32 | FID in MZone should be assigned as defined in section 16.2.6.4.1.3.1 per each DL/UL connections. |  |
| } |  |  |  |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #3: Modify line 62 of page 108 (142) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 682—— AAI-SBC-REQ message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| If (AMS requests transmittal of NSP information) { |  |  |  |
| SIQ (Service information Query) | …… | …. |  |
| } else { |  |  |  |
| CAPABILITY_INDEX |  |  |  |
| DEVICE_CLASS |  |  |  |
| …… | …. | …… | …. |
| [line 25,page111] If (ARS is a sender of AAI-SBC-REQ) { |  |  | //only available during ARS network entry phase |
| Relay mode | 1 | 0b0: TTR relay mode 0b1: STR relay mode | Optional. Present when ARS is a sender of AAI-SBC-REQ |
| Visited NSP ID | 24 | NSP ID of the Network Service Provider the MS intends to be the conduit for authentication to the MS home network. | Present as needed |
| if (Relay mode == 0b0){ |  |  |  |
| ARSTTG |  |  |  |
| ARSRTG |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #4: Modify ASN.1 code as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

[line 22 page 997]

-- complex type definition for ranging messages

FidInfo ::= SEQUENCE {

flowIdentifier FID,

fidChangeCount FidChangeCount,

dlULIndicator ENUMERATED {

dl,

ul

}

}

[line 32 page 1000]

fidList SEQUENCE (SIZE (1..1524)) OF FidInfo OPTIONAL,

[line 44 page 1000]

fidList SEQUENCE (SIZE (1.. 1524)) OF FidInfo OPTIONAL,

[line 29 page 1001]

fidList SEQUENCE (SIZE (1.. 1524)) OF FidInfo OPTIONAL,

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #5: Add a sentence to clarify ‘Condition field’ in MAC Control messages:

-------------------------------------------------Text Change Start --------------------------------------------------------------

[Add the following texts on Page 82, Line 41:]

Unless condition field for a parameter is specified in AAI message Field Description tables (Table 678 to Table 757), the condition field shall be interpreted as follows:

- If a parameter is within an if-statement or a for-statement, the parameter without specific note in the condition field shall be present whenever condition for the closest if-statement or for-statement that the parameter belongs to is met.

- Otherwise, the parameter shall be always present in the MAC Control message.

-------------------------------------------------Text Change END ---------------------------------------------------------------