| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Frame Configuration for ARS operation (16.6.3) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot Recirc #4 |  |
| Abstract | This contribution provides the clarification of the Relay FDD Frame Configuration. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Frame Configuration for ARS operation (16.6.3)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

In the previous meetings, the proposed text in the contribution IEEE802.16m-10/1337 was accepted but it’s not inserted in IEEE802.16m/D10.

- Text proposal in the P802.16m/D9

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 61 on the page 947 as below:]

16.6.3.2.3 Set of frame configurations for ARS operation

ABS or ARS informs sets of the frame configurations and indexing for AMS in S-SFH SP1 IE in Table 800, Table 801 and Table 802. Some sets of the frame configuration and indexing are not used when ABS supports ARS. If basic frame structure in TDD or FDD is decided, the ratio of access zone and relay zone is also decided by Table 974.

Table 974 Frame Configuration for ARS operation

| Bandwidth (MHz) | CP | Frame configuration index | Duplex type | AAI DL Access zone: AAI DL relay zone | AAI UL Access zone: AAI UL relay zone | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 5/10/20 | 1/16 | 0 | TDD | 3:3 | 1:1 |  |
| 5/10/20 | 1/16 | 1 | TDD | 3:2 | 2:1 |  |
| 5/10/20 | 1/16 | 2 | TDD | 2:2 | 2:2 |  |
| 5/10/20 | 1/16 | 3 | TDD | 2:1 | 3:2 |  |
| 5/10/20 | 1/16 | 4 | FDD | 4:4 | 4:4 |  |
| 5/10/20 | 1/8 | 0 | TDD | 3:3 | 1:1 |  |
| 5/10/20 | 1/8 | 1 | TDD | 3:2 | 2:1 |  |
| 5/10/20 | 1/8 | 2 | TDD | 2:2 | 2:2 | R-TTI shall be equal to 0 |
| 5/10/20 | 1/8 | 4 | FDD | 4:4 | 4:4 |  |
| 5/10/20 | 1/8 | 17 | TDD | 3:3 | 1:1 |  |
| 5/10/20 | 1/8 | 18 | TDD | 2:3 | 1:2 | R-TTI shall be equal to 0 |
| 5/10/20 | 1/4 | 3 | FDD | 4:33:4 | 4:33:4 |  |
| 8.75 | 1/16 | 0 | TDD | 2:3 | 1:1 |  |
| 8.75 | 1/16 | 1 | TDD | 2:2 | 2:1 |  |
| 8.75 | 1/16 | 2 | TDD | 2:1 | 3:1 |  |
| 8.75 | 1/16 | 3 | FDD | 4:33:4 | 4:33:4 |  |
| 8.75 | 1/8 | 3 | FDD | 3:4 | 3:4 |  |
| 8.75 | 1/4 | 3 | FDD | 3:3 | 3:3 |  |
| 7 | 1/16 | 2 | FDD | 4:2 3:3 | 4:2 3:3 |  |
| 7 | 1/8 | 2 | FDD | 3:2 2:3 | 3:2 2:3 |  |
| 7 | 1/4 | 2 | FDD | 3:2 2:3 | 3:2 2:3 | R-TTI shall be equal to 0 |

-----------------------------------------------   Text End  -----------------------------------------------------------