| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on AAI-SCD Message (16.2.3) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee Institute for Information Industry (III) Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng Tamkang University (TKU) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) | cwchen@iii.org.tw |
| Re: | Sponsor Ballot P802.16m |  |
| Abstract | This contribution propose the clarification on AAI-SCD message |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on AAI-SCD Message (16.2.3)

Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee
Institute for Information Industry (III)

Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng
Tamkang University (TKU)

Shiann-Tsong Sheu
National Central University (NCU)

Whai-En Chen
National Ilan University (NIU)

Introduction

In [1], The E-MBS scheduling interval (MSI) can be 4, 8, 16 and 32 superframes long. The indication for the length of the MSI is transmitted in the AAI-SCD message. However, the context of MSI in AAI-SCD message is not consistent with the descriptions in Section 16.9.3.3. In this contribution, the AAI-SCD message fields are modified to ensure consistency.

Proposed text for the P80216m/D10:

-----------------------------------------------Start of the Text Proposal ---------------------------------------------

[Remedy 1: Make the following changes to Section 16.2.3.31 in Page 191 beginning Line 46 in P802.16m/D10.]

#### 16.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 708—AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 24 superframes, 4080 ms (NMSI = 24) 0b01: 48 superframes, 80160 ms (NMSI = 48) 0b10: 816 superframes, 160320 ms (NMSI = 816) 0b11: 1632 superframes, 320640 ms (NMSI = 1632) |  |
| E-MBS AAI frame offset | Variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 34 bits MSI length == 0b01: 45 bits MSI length == 0b10: 56 bits MSI length == 0b11: 67 bits |  |
| } |  |  |  |

[Remedy 2: Make the following changes in Page 990 beginning Line 42 in P802.16m/D10.]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- System Configuration Descriptor Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

…

-- The length of an MSI in units of the number of superframes

-- 0b00: 24 superframes, 4080 ms (NMSI = 24)

-- 0b01: 48 superframes, 80160 ms (NMSI = 48)

-- 0b10: 816 superframes, 160320 ms (NMSI = 816)

-- 0b11: 1632 superframes, 320640 ms (NMSI = 1632)

MsiLength ::= INTEGER (2 | 4 | 8 | 16 | 32)

-- The location of the AAI frame where the E-MBS data burst ends

-- MSI length == 0b00: 34 bits

-- MSI length == 0b01: 45 bits

-- MSI length == 0b10: 56 bits

-- MSI length == 0b11: 67 bits

EmbsFrameOffset ::= CHOICE {

msiLength2FrameOffset INTEGER (0..7),

msiLength4FrameOffset INTEGER (0..15),

msiLength8FrameOffset INTEGER (0..31),

msiLength16FrameOffset INTEGER (0..63),

msiLength16FrameOffset INTEGER (0..127)

-----------------------------------------------End of the Text Proposal ---------------------------------------------

[1] IEEE P80216m/D10, “P802.16m/D10: Advanced Air Interface”