| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Unicast Available Interval Bitmap (16.2.3) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee Institute for Information Industry (III) Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng Tamkang University (TKU) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) | cwchen@iii.org.tw |
| Re: | Sponsor Ballot P802.16m |  |
| Abstract | This contribution propose the clarification on unicast available interval bitmap |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Unicast Available Interval Bitmap (16.2.3)

Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee
Institute for Information Industry (III)

Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng
Tamkang University (TKU)

Shiann-Tsong Sheu
National Central University (NCU)

Whai-En Chen
National Ilan University (NIU)

Introduction

The unicast available interval bitmap is a string of N bits b0,b1,b2,...,bN-1, and the value of N depends on the E-MBS scheduling interval (MSI). However, the context of MSI in AAI-DSA-REQ/RSP, AAI-DSC-REQ, AAI-E-MBS-RSP messages are not consistent with the descriptions in Section 16.9.3.3 [1]. In this contribution, the AAI-DSA-REQ/RSP, AAI-DSC-REQ, AAI-E-MBS-RSP message fields are modified to ensure consistency.

Proposed text for the P80216m/D10:

-----------------------------------------------Start of the Text Proposal 1--------------------------------------------

[Remedy 1: Make the following changes to Section 16.2.3.47.1 in Page 233 beginning Line 31 in P802.16m/D10.]

#### 16.2.3.47.1 AAI-DSA-REQ

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |

[Remedy 2: Make the following changes in Page 1028 beginning Line 23 in P802.16m/D10.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSA Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16)),

nmsi32 BIT STRING (SIZE (32))

} OPTIONAL,

-----------------------------------------------End of the Text Proposal 1---------------------------------------------

-----------------------------------------------Start of the Text Proposal 2--------------------------------------------

[Remedy 1: Make the following changes to Section 16.2.3.47.2 in Page 241 beginning Line 5 in P802.16m/D10.]

#### 16.2.3.47.2 AAI-DSA-RSP

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |

[Remedy 2: Make the following changes in Page 1028 beginning Line 23 in P802.16m/D10.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSA Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...

carrierSwitching CHOICE {

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16)),

nmsi32 BIT STRING (SIZE (32))

},

aaiEmbsRepMsg NULL

},

-----------------------------------------------End of the Text Proposal 2---------------------------------------------

-----------------------------------------------Start of the Text Proposal 3--------------------------------------------

[Remedy 1: Make the following changes to Section 16.2.3.47.4 in Page 252 beginning Line 38 in P802.16m/D10.]

#### 16.2.3.47.4 AAI-DSC-REQ

Table 734—AAI-DSC-REQ Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes,4 frames per bit |  |

[Remedy 2: Make the following changes in Page 1030 beginning Line 42 in P802.16m/D10.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSC Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16)),

nmsi32 BIT STRING (SIZE (32))

} OPTIONAL,

-----------------------------------------------End of the Text Proposal 3---------------------------------------------

-----------------------------------------------Start of the Text Proposal 4--------------------------------------------

[Remedy 1: Make the following changes to Section 16.2.3.60 in Page 290 beginning Line 5 in P802.16m/D10.]

#### 16.2.3.60 AAI-E-MBS-RSP

Table 754—AAI-E-MBS-RSP Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| Unicast Available Interval Bitmap | Variable (maximum: 1632) | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present when the report mode in AAI-E-MBS-REP message is 0b01 |

[Remedy 2: Make the following changes in Page 1086 beginning Line 59 in P802.16m/D10.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- E-MBS Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

…

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16)),

nmsi32 BIT STRING (SIZE (32))

-----------------------------------------------End of the Text Proposal 4---------------------------------------------

[1] IEEE P80216m/D10, “P802.16m/D10: Advanced Air Interface”