| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for condition field in DSx MAC control messages (16.2.3.47.1) |  |
| Date Submitted | 2011-01-06 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Rakesh Taori Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | The contribution proposes to clean up table formats of AAI-DSx MAC control message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for condition field in DSx MAC control messages

Hyunjeong Kang, Yeongmoon Son, Rakesh Taori

Samsung Electronics

- Introduction

A condition field in each MAC control message indicates whether a corresponding parameter is mandatory or optional or when the parameter, if it is optional, presents in the message. However the condition fields for some parameters are missing in MAC control messages.

Therefore, this contribution proposes to clean-up the condition field for AAI-DSx MAC control messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Change AAI-DSA-REQ including the condition fields in Table 734 (page 222, line 34):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Uplink/Downlink Indicator | 1 | 0: uplink; 1: downlink | N/A Shall be present |
| … |  |  |  |
| If (Global Service Class Name is included){ | 84 | Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| I: Uplink/Downlink indicator | 1 | 0: UL 1: DL | Shall be present |
| S: Maximum sustained traffic rule | 6 | Extensible look-up table 187 of 6.3.14.4.1 | Shall be present |
| B: Maximum traffic burst | 6 | Extensible look-up table 187 of 6.3.14.4.1 | Shall be present |
| R: Maximum received traffic rate | 6 | Extensible look-up table 187 of 6.3.14.4.1 | Shall be present |
| L: Maximum latency | 6 | Extensible look-up table 187 of 6.3.14.4.1 | Shall be present |
| P: Paging preference | 1 | 0 = No paging generation 1 = Paging generation | Shall be present |
| S1: Request/Transmission policy | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) … | Shall be present |
| S2: Uplink grant scheduling type | 3 | 0: Reserved … | Shall be present |
| If (Uplink grant scheduling type == BE¦¦nrtPS) { |  |  |  |
| S3: Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 | Shall be present |
| }else if (Uplink grant scheduling type == rtPS) { |  |  |  |
| S3: Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 | Shall be present |
| S5: Unsolicited polling interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. | Shall be present |
| }else if (Uplink grant scheduling type == ertPS){ |  |  |  |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1 | Shall be present |
| S3: Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 | Shall be present |
| S4: Unsolicited grant interval | 6 |  | Shall be present |
| }else if (Uplink grant scheduling type == UGS){ |  |  |  |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. | Shall be present |
| S4: Unsolicited grant interval | 6 | Extensible look-up table 188 of 6.3.14.4.1. | Shall be present |
| } |  |  |  |
| }//End if (Global Service Class Name) |  |  |  |
| QoS parameter set type | 3 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set | Shall be present |
| … |  |  |  |
| If(Uplink/Downlink Indicator==uplink) { |  |  |  |
| UL Grant Scheduling Type | 3 | 0: Reserved … | Shall be present |
| If(UL Grant Scheduling Type==ertPS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds | Shall be present |
| } else if (UL Grant Scheduling Type == UGS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds | Shall be present |
| } else if (UL Grant Scheduling Type == rtPS) { |  |  |  |
| Unsolicited Polling Interval parameter | 16 | Milliseconds | Shall be present |
| } else if (UL Grant Scheduling Type == aGPS) { |  |  |  |
| Primary Grant Polling Interval parameter | 16 | Milliseconds | Present if primary parameter set is needed |
| Primary Grant Size parameter | 16 | Number of bytes. | Present if primary parameter set is needed |
| Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present if secondary parameter set is needed |
| Secondary Grant Size parameter | 16 | Number of bytes. | Present if secondary parameter set is needed |
| Adaptation Method | 1 | 0: ABS-initiated adaptation 1: AMS-initiated adaptation | Shall be present when Uplink grant scheduling type is aGPS |
| } |  |  |  |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. | Present if needed |
| Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) | Present if needed, but shall be present when an ABS initiates AAI-DSA-REQ |
| For(i=10; i<=N-Predefined-BR-indices; i++) { |  |  |  |
| Predefined BR index | 4 | Predefined BR index | Present if N-Predefined-BR-indices is not zero |
| BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved | Present if N-Predefined-BR-indices is not zero |
| If(BR action == 0b10) { |  |  |  |
| BR size | 11 | Number of byte requested by the predefined BR index | Shall be present |
| } |  |  |  |
| } |  |  |  |
| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 | Present if needed |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 | Present if needed |
| Backoff Scaling Factor | 4 | “0010” for Binary Exponential Backoff | Present if needed |
| } // End if (Uplink/Downlink Indicator == uplink) |  |  |  |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in the connection is preserved by the MAC. 0 : not preserved 1 : preserved For ARQ connections, it shall be always set to '1'. | Present if needed |
| If (MAC in-order delivery indicator == 1) { |  |  |  |
| NON_ARQ_REORDERING_TIMEOUT | 6 | 0> and <=32, Unit is PHY frame (5ms) | Shall be present |
| } |  |  |  |
| … |  |  |  |
| If(Vendor-specific classification parameters are needed) { |  | See 16.2.12.12.8.1 |  |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. | Shall be present |
| Vendor-specific classification | Variable | Vendor-specific classification rule parameters | Shall be present |
| } |  |  |  |
| IPv6 Flow Label field | 24 | Flow Lable | Present if needed |
| Classification Action Rule | 1 | 0 = none. 1 = Discard packet | Shall be present |
| IP Type of Service | 7 | DSCP value (0 to 127) | Present if needed |
| PHS DSC Action field | 2 | 0: Add PHS Rule | Present if needed |
| If(PHS Rule field is added) { |  |  |  |
| PHSI field | 8 | Index value | Present only ABS-initiated DSA |
| PHSF field | n8 | String of bytes suppressed | Present if needed |
| PHSM field | n8 | Bit 0: … | Present if needed |
| PHSS field | 8 | Number of bytes in the suppression string | Present if needed |
| PHSV field | 1 | 0: Verify 1: Don’t verify | Present if needed |
| If(Vendor-specific PHS parameters are needed) { |  | see 16.2.12.12.8.2 |  |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. | Shall be present |
| Vendor-specific PHS | Variable | Vendor-specific PHS parameters | Shall be present |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |
| inner source IP | 32 | Source IP address of inner IP header | Present if needed |
| Inner destination IP | 32 | Destination IP address of inner IP header | Present if needed |
| Inner protocol field | 8 | Protocol field of inner IP header | Present if needed |
| Inner TOS (Type of Service) | 8 | TOS field of inner IP header | Present if needed |
| Inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header | Present if needed |
| } //End If (Packet Classification Rule) |  |  |  |
| If (ROHC Parameter is needed) { |  |  |  |
| ROHC Max Context ID | 16 | Non-negative integer | Shall be present |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved | Shall be present |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit | Shall be present |
| } |  |  |  |
| … |  |  |  |
| If(E-MBS is supported) { |  |  |  |
| For(i=0; i<N-E-MBS Zone ID; i++) { | 3 | N-E-MBS Zone ID is the number of E-MBS zone IDs [1..8] |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Shall be present |
| Physical carrier index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| For(i=0; i<N-E-MBS-ID;i++) { |  | N-E-MBS-ID is the number of E-MBS identifiers [1..15] |  |
| E-MBS ID | 12 | E-MBS identifier | Shall be present |
| FID | 4 | Flow ID | Shall be present |
| } |  |  |  |
| } |  |  |  |
| } // End If (E-MBS is supported) |  |  |  |
| Carrier Switching Mode | 1 | 0b0: … | Present when ABS-initiated DSA for carrier switching |
| If(Carrier Switching Mode==0b0){ |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available … | Shall be present |
| } |  |  |  |
| If (Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.12.6 |  |
| Common for Group Create/Change | Variable | Common service flow encodings that … | Shall be present |
| Qty SFID request | 5 | The number of service flows to create | Shall only be sent by an AMS as the last attribute of Group Parameter Create/Change |
| For(i=0; i<N-FIDs;i++) { |  | FIDs of service flows created with this Group Parameter Create/Change N-FIDs is the number of FIDs The maximum value of N-FIDs is 32. |  |
| FID | 4 |  | Shall be present |
| } |  |  |  |
| For(i=0;i<N-FIDs-Noncommon;i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs The maximum value of N-FIDs-Noncommon is 32 |  |
| FID | 4 |  | Shall be present if N-FIDs-Noncommon is not zero |
| Non-common for Group Create/Change | Variable | Non-common service flow encodings that … | Shall be present if N-FIDs-Noncommon is not zero |
| } |  |  |  |
| } |  |  |  |
| If(Coupled Group Create/Change is needed) { |  | Refer to 16.2.12.12 |  |
| Common for Coupled Group | Variable | Common service flow encodings that … | Shall be present |
| Qty Coupled SFID request | 5 | The number of coupled DL service flow and UL service flow of the same common … | Shall only be sent by an AMS |
| For(i=0; i<N-FIDs-Coupled; i++){ |  | FIDs of DL and UL service flow (4 MSB for DL FID and 4 LSB for UL FID) N-FIDs-Coupled is the number of FIDs The maximum value of N-FIDs-Coupled is 16. |  |
| FID | 8 |  | Shall be present |
| } |  |  |  |
| For (i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 |  | Shall be present if N-FIDs-Coupled-Noncommon is not zero |
| Non-common for Coupled Group | Variable | Non-common service flow encodings that … | Shall be present if N-FIDs-Coupled-Noncommon is not zero |
| } |  |  |  |
| } |  |  |  |
| For (i=0; i<Num of Multicast Group ID; i++) { |  | Num of Multicast Group ID is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ Present only if Num of Multicast Group ID > 0 |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID > 0 |
| FID | 4 | Multicast specific FID which is associated with Multicast Group ID | Present only if Num of Multicast Group ID > 0 |
| } |  |  |  |
| If(Sleep Cycle Setting is included) { |  |  | May be present when Sleep Cycle setting needs to be changed or switched |
| Operation | 2 | This indicates operation request type 0b00~0b01 : Reserved 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting | Shall be present |
| If(Operation == 0b10 ¦¦ Operation == 0b11) { |  |  |  |
| SCID | 4 | Sleep Cycle ID 0~15 | Shall be present |
| Start Frame Number | 6 | 0-63 | Shall be present |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept … | Shall be present |
| If (Operation == 0b10) { |  |  |  |
| TIMF | 1 | Traffic Indication Message Flag | Shall be present |
| LWEF | 1 | Listening window Extension Flag | Shall be present |
| NSCF | 2 | Next sleep cycle indicator | Shall be present |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle … | Shall be present |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle … | Shall be present |
| Listening Window | 6 | Assigned duration of AMS’s default Listening Window … | Shall be present |
| If(TIMF==1){ |  |  |  |
| SLPID | 10 | This is an identifier assigned by the ABS … | Shall be present |
| } |  |  |  |
| If(NSCF ==0b10) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS | Shall be present |
| } |  |  |  |
| If(LWEF==1){ |  |  |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | Shall be present |
| } |  |  |  |
| }//ENDIF (Operation ==0b10) |  |  |  |
| }//ENDIF (Operation == 0b10 ¦¦ Operation ==0b11) |  |  |  |
| For(i=0; i<Number of HARQ channels; i++) { |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel | Shall be present when Number of HARQ channels > 0 |
| } |  |  |  |
| For(i=0; Number of HARQ channels; i++) { |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel |  |
| } |  |  |  |

[Remedy 2:]

[Change the AAI-DSA-RSP including the condition fields in Table 735 (page 239, line 54):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(Confirmation Code==0 && AMS initiated AAI-DSA-REQ){ |  |  |  |
| FID | 4 | An identifier of a service flow | Shall be present |
| } |  |  |  |
| If(received an AAI-DSA-REQ including Group Parameter Create/Change parameter){ |  |  |  |
| For(i=0; i<N-FIDs; i++){ |  | N-FIDs is the number of successfully created service flow. | Shall be included when N-FIDs > 0 |
| FID | 4 | FIDs of service flows created successfully with this Group Parameter Create/Change. | Shall be included when N-FIDs > 0 |
| } |  |  |  |
| } |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being successfully setup … | Present if needed |
| If(E-MBS is supported) { |  |  |  |
| For(i=0; i<N-E-MBS Zone ID; i++) { | 3 | N-E-MBS Zone ID is the number of E-MBS zone IDs [1..8] |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone … | Shall be present if N-E-MBS Zone ID >0 |
| Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the EMBS Zone ID is served on a different carrier from the current carrier. |
| For(ji=0; ji<N-E-MBS-ID; ji++) { |  | N-E-MBS-ID is the number of EMBS identifiers [1...15] |  |
| E-MBS ID | 12 | E-MBS identifier | Shall be present if N-E-MBS-ID >0 |
| FID | 4 | Flow ID | Shall be present if N-E-MBS-ID >0 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSA message 0b1: carrier switching parameters using AAI-E-MBS-REP message | Present if ABS indicates carrier switching when receiving AMS-initiated DSA |
| If(Carrier Switching Mode==0b0){ |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier … | Shall be present when Carrier Switching Mode is set to 0b0 |
| } |  |  |  |
| For(i=0; i<Num of Multicast Group ID; i++) { |  | Num of Multicast Group ID is the number of Multicast Group ID to add [1..16] | Present when Num of Multicast Group ID > 0 |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present when Num of Multicast Group ID > 0 |
| FID | 4 | Multicast specific FID which is associated with Multicast Group ID | Present when Num of Multicast Group ID > 0 |
| } |  |  |  |
| … |  |  |  |

[Remedy 3:]

[Change the condition fields of AAI-DSA-ACK in Table 736 (page 244, line 27):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSA-RSP message | N/A Shall be present |
| Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure | Shall be present |

[Remedy 4:]

[Change the AAI-DSC-REQ including the condition fields in Table 737 (page 245, line 50):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| QoS parameter set type | 3 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set | Shall be present |
| … |  |  |  |
| Paging Preference parameter | 1 | 0: No paging generation 1: Paging generation | Present when needed |
| Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
| If (UL scheduling type == UGS¦¦ertPS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds | Present if UL scheduling type is set to UGS or ertPS |
| }else if (UL scheduling type == rtPS) { |  |  |  |
| Unsolicited Polling Interval parameter | 16 | Milliseconds | Present if UL scheduling type is set to rtPS |
| }else if (UL scheduling type == aGPS) { |  |  |  |
| Primary Grant Polling Interval parameter | 16 | Milliseconds | Present when primary parameter set is needed |
| Primary Grant Size parameter | 16 | Number of bytes | Present when primary parameter set is needed |
| … |  |  |  |
| } |  |  |  |
| If (Uplink/Downlink Indicator == 0) { |  |  |  |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. … | Present if needed |
| Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted … | Present if needed |
| For (i=0; i<N-Predefined-BR-indices; i++) { |  | The mapping of predefined BR index used in quick access message to BR … |  |
| Predefined BR index | 4 | Predefined BR index | Present when N-Predefined-BR-indices > 0 |
| BR action | 2 | 0b00: ertPS service … | Present when N-Predefined-BR-indices > 0 |
| If (BR action == 0b10) { |  |  |  |
| BR size | 11 | Number of bytes requested by the predefined BR index | Present when BR action is set to 0b10 |
| } |  |  |  |
| } |  |  |  |
| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 | Present if needed |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 | Present if needed |
| Backoff Scaling Factor | 4 | “0010” for Binary Exponential Backoff | Present if needed |
| }//End If (Uplink/Downlink Indicator==0) |  |  |  |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery … | Present if needed |
| If(Vendor-specific classification parameters are needed) { |  | See 16.2.12.12.8.1 |  |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, … | Present when needed Vendor-specific classification parameters are included |
| Vendor-specific classification | Variable | Vendor-specific classification rule parameters | Present when Vendor-specific classification parameters are included |
| } |  |  |  |
| SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present when needed |
| Time Base parameter | 16 | Time base in milliseconds | Present when needed |
| Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier | Present when needed |
| If (Packet Classification Rule parameter is needed) { |  |  |  |
| Classification Rule Priority field | 8 | 0-255 | Present when needed |
| Protocol field | 8 | Protocol | Present when needed |
| … |  |  |  |
| Classification Action Rule | 1 | 0=none 1=Discard packet | Present when needed |
| … |  |  |  |
| If(PHS DSC Action field == Add PHS Rule ¦¦ Set PHS Rule) { |  |  |  |
| PHSI field | 8 | Index value | Shall be present if PHS DSC Action field is set to Add PHS Rule or Set PHS Rule when ABS initiates AAI-DSC-REQ |
| … |  |  |  |
| If (Vendor-specific PHS parameters are needed) { |  | See 16.2.12.12.8.2 |  |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, … | Present when Vendor-specific PHS parameters are included |
| Vendor-specific PHS | Variable | Vendor-specific PHS parameters | Present when Vendor-specific PHS parameters are included |
| } |  |  |  |
| } |  |  |  |
| Else if (PHS DSC Action field = Delete PHS Rule) { |  |  |  |
| PHSI field | 8 | Index value | Shall be present when PHS DSC Action field is set to Delete PHS Rule |
| } |  |  |  |
| … |  |  |  |
| inner source IP | 32 | Source IP address of inner IP header | Present if needed |
| inner destination IP | 32 | Destination IP address of inner IP header | Present if needed |
| inner protocol field | 8 | Protocol field of inner IP header | Present if needed |
| inner TOS (Type of Service) | 8 | TOS field of inner IP header | Present if needed |
| inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header | Present if needed |
| } |  |  |  |
| }//end if QoS |  |  |  |
| If(ROHC Parameter is needed) { |  |  | Present if needed |
| ROHC Max Context ID | 16 | Non-negative integer | Shall be present |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved | Shall be present |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit | Shall be present |
| } |  |  |  |
| … |  |  |  |
| For (i=0; i<Num of E-MBS Zone ID; i++) { | 3 | Num of E-MBS Zone ID [1..8] is the number of E-MBS ID to update | Present when ABS initiates AAI-DSC-REQ Present only if Num of E-MBS Zone ID > 0 |
| Service_flow_update_indicator | 1 | Indicator whether “bitmap + new service flow” … | N/A Present only if Num of E-MBS Zone ID > 0 |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the … | N/A Present only if Num of E-MBS Zone ID > 0 |
| … |  |  |  |
| If(Service_flow_update_indicator == 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID + FID … | Present only if Service_flow_update_indicator is set to 0b0 |
| For(m=0; m<N_ServiceFlowUpdateBitmap; m++) { |  | N_ServiceFlowUpdateBitmap is the number of bits to be set to 1 … | Presented only if the number of bits set to 1 in Service Flow Update Bitmap > 1 |
| New_E-MBS ID | 12 | New E-MBS identifier | Present when N_ServiceFlowUpdateBitmap > 0 |
| FID | 4 | Flow ID | Present when N_ServiceFlowUpdateBitmap > 0 |
| } |  |  |  |
| }else if (Service_flow_update_indicator == 0b1) { |  |  |  |
| For(k=0; k<N_E-MBS_FID_Mapping; k++) { |  | N_E-MBS_FID_Mapping is the number of mappings of current E-MBS ID and FID … |  |
| Current E-MBS ID | 12 | Current E-MBS identifier | Present when N_E-MBS_FID_Mapping > 0 |
| Current FID | 4 | Current flow ID | Present when N_E-MBS_FID_Mapping > 0 |
| New_E-MBS ID | 12 | New E-MBS identifier | Present when N_E-MBS_FID_Mapping > 0 |
| New FID | 4 | New flow ID | Present when N_E-MBS_FID_Mapping > 0 |
| } |  |  |  |
| } |  |  |  |
| }//end of for |  |  |  |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on … | Shall be present when the Unicast Available Interval needs to be updated |
| If (Carrier switching Mode == 0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary … | Present when Carrier Switching Mode is set to 0b0 |
| } |  |  |  |
| If (Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.12.6 |  |
| Common for Group Create/Change | Variable | Common service flow encodings that are common … | Shall be present when Group Parameter Create/Change parameter is included |
| Qty SFID request | 5 | The number of service flows to change … | Shall only be sent by an AMS |
| For (i=0; i<N-FIDs; i++) { |  | FIDs of service flows changed with this … |  |
| FID | 4 |  | Present when N-FIDs>0 |
| } |  |  |  |
| For(i=0; i<N-FIDs-Noncommon; i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs … |  |
| FID | 4 |  | Present when N-FIDs-Noncommon > 0 |
| Non-common for Group Create/Change | Variable | Non-common service flow encodings that are specific to … | Present when N-FIDs-Noncommon > 0 |
| } |  |  |  |
| } |  |  |  |
| If(Coupled Group Create/Change is needed) { |  | Refer to 16.2.12.12 |  |
| Common for Coupled Group | Variable | Common service flow encodings that … | Present when Coupled Group Create/Change parameter is included |
| Qty Coupled SFID request | 5 | The number of coupled DL service flow and UL service flow … | Present when Coupled Group Create/Change parameter is included |
| For (i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of DL and UL service flow … |  |
| FID | 8 |  | Present when Coupled Group Create/Change parameter is included |
| } |  |  |  |
| For (i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs … |  |
| FID | 4 | Flow identifier | Present when N-FIDs-Coupled-Noncommon >0 |
| Non-common for Coupled Group | Variable | Non-common service flow encodings that are specific to individual service flows … | Present when N-FIDs-Coupled-Noncommon >0 |
| } |  |  |  |
| } |  |  |  |
| For(i=0; i<Num of Multicast Group ID; i++) { |  | Num of Multicast Group ID is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSC-REQ Present only if Multicast Group ID to add exists Num of Multicast Group ID > 0 |
| Multicast Group ID to be added | 12 | Multicast Group ID to be added | Present only if Num of Multicast Group ID > 0 |
| FID | 4 | Multicast specific FID which is associated with newly added Multicast Group ID | Present only if Num of Multicast Group ID > 0 |
| } |  |  |  |
| For(i=0; i<Num of Multicast Group ID; i++) { |  | Num of Multicast Group ID is the number of Multicast Group IDs to delete [1..16] | Present when ABS initiates AAI-DSC-REQ Present only if Multicast Group ID to delete exists Num of Multicast Group ID >0 |
| Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted | Present only if Num of Multicast Group ID > 0 |
| } |  |  |  |
| If(Sleep Cycle Setting is included) { |  |  | May be present when Sleep Cycle setting needs to be changed or switched |
| Operation | 2 | This indicates operation request type 0b00~0b01: Reserved 0b10: Change Sleep Cycle setting 0b11: Switch Sleep Cycle setting | Shall be present |
| If(Operation == 0b10 ¦¦ Operation == 0b11) { |  |  |  |
| SCID | 4 | Sleep Cycle ID 0~15 | Shall be present |
| Start Frame Number | 6 | 0-63 | Shall be present |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept … | Shall be present |
| If(Operation == 0b10) { |  |  |  |
| TIMF | 1 | Traffic Indication Message Flag … | Shall be present |
| LWEF | 1 | Listening window Extension Flag … | Shall be present |
| NSCF | 2 | Next sleep cycle indicator | Shall be present |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle … | Shall be present |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle … | Shall be present |
| Listening Window | 6 | Assigned duration of AMS’s default Listening Window … | Shall be present |
| If(TIMF==1) { |  |  |  |
| SLPID | 10 | This is an identifier assigned by the ABS … | Shall be present |
| } |  |  |  |
| If(NSCF == 1) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS … | Shall be present |
| } |  |  |  |
| If(LWEF==1){ |  |  |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS 0~31 | Shall be present |
| } |  |  |  |
| }//ENDIF (Operation == 0b10) |  |  |  |
| … |  |  |  |
| For(i=0; i<Number of HARQ channels; i++) { |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel | Shall be present when Number of HARQ channels > 0 |
| } |  |  |  |
| For(i=0; Number of HARQ channels; i++) { |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel |  |
| } |  |  |  |

[Remedy 5:]

[Change the AAI-DSC-RSP including the condition fields in Table 738 (page 258, line 26):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Confirmation Code | 1 | 0 = request was successful 1 = request was failure | Shall be present |
| If(received an AAI-DSA-REQ including Group Parameter Create/Change parameter) { |  |  |  |
| For(i=1; i<=N-FIDs; i++) { |  | N-FIDs is the number of successfully created service flow. The range of NFIDs is 1..16. | Shall be included when N-FIDs > 0 |
| FID | 4 | FIDs of service flows created successfully with this Group Parameter Create/Change | Shall be included when N-FIDs > 0 |
| } |  |  |  |
| … |  |  |  |

[Remedy 6:]

[Change the AAI-DSC-ACK including the condition fields in Table 736 (page 261, line 26):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-RSP message | Shall be always present |
| Confirmation Code | 1 | 0 = request was successful 1 = request was failure | Shall be present |

[Remedy 7:]

[Change the AAI-DSD-REQ including the condition fields in Table 740 (page 261, line 63):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(ABS initiated AAI-DSD-REQ) { |  |  |  |
| FID | 4 | Flow identifier to be deleted | Shall be always present when an existing service flow is deleted |
| } |  |  |  |
| If(E-MBS service flow to delete exists){ |  |  |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Shall be present when E-MBS service flow to delete exists |
| N_E-MBS_ID_FID_Mapping | 4 | Indicates the number of mapping of E-MBS ID and FID … |  |
| For(i=1; i<=N_E-MBS_ID_FID_Mapping; i++) { |  |  |  |
| E-MBS ID | 12 | E-MBS Identifier | Shall be present when N_E-MBS_ID_FID_Mapping > 0 |
| FID | 4 | Flow identifier | Shall be present when N_E-MBS_ID_FID_Mapping > 0 |
| } |  |  |  |
| } //ENDIF (E-MBS service flow deletion) |  |  |  |
| … |  |  |  |

[Remedy 8:]

[Change the AAI-DSD-RSP including the condition fields in Table 741 (page 265, line 22):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| FID | 4 | Flow identifier to be deleted | Present when an existing service flow is deleted ABS initiates AAI-DSA-REQ |
| Confirmation Code | 1 | 0 = request was successful 1 = request was failure | Shall be present |
| … |  |  |  |

[Remedy 9:]

[Change ASN.1 code from line 12 of page 1028 to line 34 of page 1028:]

fullEMBSIDArray SEQUENCE (SIZE (1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex,

embsidFIDMappingList SEQUENCE (SIZE (1..15)) OF SEQUENCE {

embsid EMBSID,

fid FID

}

} OPTIONAL,

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

} OPTIONAL,

groupParameterCreateChange GroupParaCreateChange OPTIONAL,

coupledGroupCreateChange CoupledGroupCreateChange OPTIONAL,

[Remedy 10:]

[Change ASN.1 code from line 1 of page 1029 to line 24 of page 1029:]

fullEMBSIDArray SEQUENCE (SIZE (1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex OPTIONAL,

embsidFIDMappingArray SEQUENCE (SIZE (1..15)) OF SEQUENCE {

embsid EMBSID,

fid FID

}

} OPTIONAL,

carrierSwitching CHOICE {

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

},

aaiEmbsRepMsg NULL

} OPTIONAL,

[Remedy 11:]

[Change ASN.1 code from line 51 of page 1029 to line 55 of page 1029:]

AbsInitDscInfo ::= SEQUENCE {

sfid SFID,

fid FID

}

[Remedy 12:]

[Change ASN.1 code from line 15 of page 1030 to line 55 of page 1030:]

fullEMBSIDArray SEQUENCE (SIZE (1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

newEmbsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex OPTIONAL,

serviceFlowUpdateType CHOICE {

bitmapAndNew SEQUENCE {

serviceFlowUpdateBitmap BIT STRING (SIZE (16)),

embsidFIDMappingArray SEQUENCE (SIZE (1..16)) OF SEQUENCE {

newEMBSID EMBSID,

newFID FID

},

currentAndNew SEQUENCE (SIZE (1..16)) OF SEQUENCE {

currentEMBSID EMBSID,

currentFID FID,

newEMBSID EMBSID,

newFID FID

}

}

} OPTIONAL,

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE (4)),

nmsi4 BIT STRING (SIZE (4)),

nmsi8 BIT STRING (SIZE (8)),

nmsi16 BIT STRING (SIZE (16))

} OPTIONAL,

groupParameterCreateChange GroupParaCreateChange OPTIONAL,

coupledGroupCreateChange CoupledGroupCreateChange OPTIONAL,

[Remedy 13:]

[Change ASN.1 code from line 48 of page 1031 to line 65 of page 1031:]

AbsInitDsdEmbsInfo ::= SEQUENCE {

fid FID,

embsZoneID EMBSZoneID,

embsidFIDMappingArray SEQUENCE (SIZE (1..15)) OF SEQUENCE {

embsid EMBSID,

fid FID

}

}

AAI-DSD-REQ ::= SEQUENCE {

fidChangeCount FidChangeCount,

fid FID OPTIONAL,

absInitiatedDsddsdEmbsInfo AbsInitDsdEmbsInfo OPTIONAL,

sleepCycleSetting SleepCycleSetting OPTIONAL,

[Remedy 14:]

[Change ASN.1 code from line 15 of page 1027 to line 64 of page 1027:]

GroupParaCreateChange ::= SEQUENCE {

commonQosParameters SEQUENCE {

qosParameter QosParameter OPTIONAL,

additionalSfInfo AdditionalSfInfo OPTIONAL

},

qtySfid INTEGER (1..32) OPTIONAL,

groupFidList SEQUENCE (SIZE (1..32)) OF SEQUENCE {

fid INTEGER (0..15)

},

groupFidParameterArray SEQUENCE (SIZE (1..32)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

qosParameter QosParameter OPTIONAL,

additionalSfInfo AdditionalSfInfo OPTIONAL

}

} OPTIONAL

}

CoupledGroupCreateChange ::= SEQUENCE {

commonQosParameters SEQUENCE {

qosParameter QosParameter OPTIONAL,

additionalSfInfo AdditionalSfInfo OPTIONAL

},

qtyCoupledSfid INTEGER (0..32) OPTIONAL,

coupledFidArray SEQUENCE (SIZE (1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL

},

coupledNonCommonFidArray SEQUENCE (SIZE (1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

qosParameter QosParameter OPTIONAL,

additionalSfInfo AdditionalSfInfo OPTIONAL

}

} OPTIONAL

}

[Remedy 15:]

[Change ASN.1 code from line 61 of page 1017 to line 37 of page 1018:]

UplinkQosInfo ::= SEQUENCE {

ulGrantScheduling UlGrantScheduling OPTIONAL,

ulSchedulingType CHOICE {

ertPs SEQUENCE {

unsolicitedGrantInterval Interval

}, -- ms

ugs SEQUENCE {

unsolicitedGrantInterval Interval

}, -- ms

rtPs SEQUENCE {

unsolicitedPollingInterval Interval

}, -- ms

agPs SEQUENCE {

priGrantPollingInterval Interval, -- ms

secGrantPollingInterval Interval OPTIONAL, -- ms

priGrantSize GrantSize, -- bytes

secGrantSize GrantSize OPTIONAL, -- bytes

adaptationMethod ENUMERATED {

absInitiated,

amsInitiated

}

}

},

accessClass INTEGER (0..3) OPTIONAL,

differentiatedBrTimer INTEGER (1..64) OPTIONAL, -- frames

predefinedBrIndexList SEQUENCE (SIZE (0..15)) OF PredefinedBrIndex OPTIONAL,

initialBackoffWindowSize INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize INTEGER (0..15) OPTIONAL,

backoffScalingFactor INTEGER (0..15) OPTIONAL

}

-------------------------------  Text End  ---------------------------------------------------