| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for condition field in Multicarrier MAC control messages (16.2.3.55) |  |
| Date Submitted | 2011-01-11 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Jaeweon Cho, Chiwoo Lim Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | The contribution proposes to clean up table formats of multicarrier MAC control message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for condition field in Multicarrier MAC control messages

Hyunjeong Kang, Yeongmoon Son, Jaeweon Cho, Chiwoo Lim

Samsung Electronics

- Introduction

A condition field in each MAC control message indicates whether a corresponding parameter is mandatory or optional or when the parameter, if it is optional, presents in the message. However the condition fields for some parameters are missing in MAC control messages.

Therefore, this contribution proposes to clean-up the condition field for multicarrier MAC control messages.

In addition, there is a problem for DL-only activated fully configured carrier operation.

If the frame configuration (e.g., DL/UL ratio) between primary and DL only activated fully configured secondary carrier is different, the number of HARQ feedback region required within a UL subframe of primary carrier is different. It’s required to fix it by adding one restriction like “The DL-only activation of fully configured secondary carrier shall be performed only if the BW, CP, and Frame configuration index in Table 800 of secondary carrier are same as those of primary carrier.” Please see the Remedy #4.

The revised version :

- (IEEE C802.16m-10/1439r1) includes the remedy #4.

- (IEEE C802.16m-10/1439r2) removes all the ‘shall be present’ and ‘mandatory’ in Condition field of Table 749 and 750

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Change the condition fields of AAI-MC-ADV in Table 749 (page 278, line 31):]

| Field | Size (bits) | Description | Condition |
| --- | --- | --- | --- |
| Multi-carrier configuration change count | 4 | The value is incremented by 1 upon each update |  |
| Serving BS Carrier Number | 3 | Indicates the number of carriers supported by the S-ABS |  |
| MAC Protocol version | 8 | Indicates MAC protocol version that the current carrier supports: it is consistent with REV 2 definition with new MAC protocol version 9 defined for AAI |  |
| … |  |  |  |
| For(i=1; i<=Serving BS Carrier Number-1; i++){ |  |  |  |
| PGID | 16 | Indicate Paging group identifier where the carrier belongs |  |
| Physical carrier index | 6 | Indicates the carrier index that the ABS carriers except current carrier: the physical carrier index refers to AAI-Global-CFG message |  |
| SA-Preamble Index | 10 | Indicates the SA-Preamble index of the carrier |  |
| Paging carrier indicator | 1 | Indicates whether this carrier is a paging carrier or not 0: no paging carrier 1: paging carrier | optional |
| … |  |  |  |
| S-SFH change count | 4 | Change count of S-SFH SP IE(x) |  |
| Change count for AAI-SCD | 4 | Change configuration count of AAI-SCD |  |
| If(SFH_encoding_format==0b00){ |  |  |  |
| SFHSubpkt 1 | Variable | SFH subpacket 1 is included, length depends on FFT size. |  |
| SFHSubpkt 2 | Variable | SFH subpacket 2 is included, length depends on FFT size. |  |
| SFHSubpkt 3 | Variable | SFH subpacket 3 is included |  |
| AAI-SCD_Info | Variable | Information from AAI-SCD is included |  |
| } |  |  |  |
| If(SFH_encoding_format==0b01 or 0b10){ |  |  |  |
| Delta information | Variable | Indicates delta encoding between the reference carrier and the current carrier where this message is transmitted if SFH_encoding_format=0b01, or … |  |
| } |  |  |  |
| } |  |  |  |

[Remedy 2:]

[Change the condition field of ‘Multi-Carrier Configuration Index’ in Table 750 (AAI-Global-CFG, page 283, line 23):]

| Field | Size (bits) | Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For(i=0; i<Number of Carrier Groups; i++){ |  |  |  |
| Duplexing mode | 1 | “0” for TDD “1” for FDD | Mandatory |
| Number of Carriers | 4 | Number of carriers in this configuration. Value Range is 1(0x0)~16(0xF) | Mandatory |
| Multi-Carrier Configuration Index | 6 | Index refers to the multi-carrier configuration depicted in Table 798 |  |
| … |  |  |  |

[Remedy 3:]

[Change the ASN.1 code from line 51 of page 1064 to line 17 of page 1065:]

ServingABSCarrierInfo ::= SEQUENCE {

pagingGroupID PGID,

carrierIndex PhyCarrierIndex,

saPreambleIndex PreambleIndex,

pagingCarrierIndicator ENUMERATED {

noPagingCarrier,

pagingCarrier

} OPTIONAL,

caSpecificTrigger CASpecificTriggers OPTIONAL,

pccSpecificTrigger Triggers OPTIONAL,

sSFHChangeCount INTEGER (0..15) OPTIONAL,

scdChangeCount INTEGER (0..15) OPTIONAL,

sfhScdInfo CHOICE {

sfhSubpack SEQUENCE {

sfhSubpacket1 SFHSubpacket1,

sfhSubpacket2 SFHSubpacket2,

sfhSubpacket3 SFHSubpacket3,

aaiSCDInfo AAI-SCD

},

deltaRefToCurCarrier SfhScdDeltaInfo,

deltaRefToPrevCarrier SfhScdDeltaInfo

}

}

[Remedy 4:]

[Insert the following text to line 6 of page 395:]

The DL-only activation of fully configured secondary carrier shall be performed only if the BW, CP, and Frame configuration index in Table 800 of secondary carrier are same as those of primary carrier.

-------------------------------  Text End  ---------------------------------------------------