| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for condition field in Idle mode MAC control messages (16.2.3.21) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Rakesh Taori Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | The contribution proposes to clean up table formats of idle mode MAC control message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for condition field in Idle mode MAC control messages

Hyunjeong Kang, Yeongmoon Son, Rakesh Taori

Samsung Electronics

- Introduction

A condition field in each MAC control message indicates whether a corresponding parameter is mandatory or optional or when the parameter, if it is optional, presents in the message. However the condition fields for some parameters are missing in MAC control messages. Therefore, this contribution proposes to clean-up the condition field for Idle mode MAC control messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Change the condition fields of AAI-DREG-REQ in Table 698 (page 162, line 7):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| De-registration_Request_Code | 3 | Used to indicate the purpose of this message … 0x06-0x07: reserved | mandatory |
| If (De-registration_Request_Code==0x01){ |  |  |  |
| Paging cycle request | 4 | Used to indicate Paging cycle recommended by AMS 0b0000 – 0b1111: reserved | mandatory |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … | mandatory |
| … |  |  |  |
| If(De-registration_Request_Code==0x04){ |  |  |  |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … | mandatory |
| } |  |  |  |

[Remedy 2:]

[Change the condition fields of AAI-DREG-RSP in Table 699 (page 165, line 5):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 4 | Used to indicate the purpose of this message … 0x10-0x15: reserved | mandatory |
| If (Action code == 0x05){ |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS … | mandatory |
| Paging offset | 12 | Used to indicate Paging offset for the AMS. | mandatory |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS’s idle mode information 0..248-1 | mandatory |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..216-1 | mandatory |
| Deregistration ID | 18 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..218-1 | Present when the Network Configuration indicates ABS is attached to the advanced network |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … | mandatory |
| … |  |  |  |
| If (Action code == 0x07){ |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS … | mandatory |
| Paging offset | 12 | Used to indicate Paging offset for the AMS. … | mandatory |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the AMS’s idle mode information 0..248-1 | mandatory |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..216-1 | mandatory |
| Deregistration ID | 18 | Used to indicate Deregistration ID used to identify the AMS in idle mode 0..218-1 | Present when the Network Configuration indicates ABS is attached to the advanced network |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … | mandatory |
| } |  |  |  |

[Remedy 3:]

[Change the condition fields of AAI-PAG-ADV in Table 700 (page 169, line 8):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI-PAG-ADV message … 1: the paging information for the corresponding PGID is included | mandatory L equals the Num_PGIDs in PGID-Info message |
| … |  |  |  |
| For (j=0; j<Num_AMSs; j++) { |  |  |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the MS to be paged (Deregistration Identifier and Paging Cycle are used to … | Present if the Network … |
| … |  |  |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update | shall be present |
| } |  |  |  |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists … | mandatory |

[Remedy 4:]

[Change the condition field of ‘Num_PGIDs’ in Table 701 (PGID-Info):]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Num_PGIDs | 2 | Indicate the number of PGID included in PGID-Info message 1..4 | mandatory |
| … |  |  |  |
| For (i=01; i<=Num_PGIDs; i++) { |  |  |  |
| PGID | 16 | Indicate Paging group identifier where the ABS belongs 0..216-1 | Shall be present when Num_PGIDs is not zero |
| m | 2 | Time domain hash parameter (1~4) used to … | Shall be present when Num_PGIDs is not zero |

-------------------------------  Text End  ---------------------------------------------------