| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up on Feedback Polling A-MAP IE (16.3.5.5) and UL codebook (16.3.9.3) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Taeyoung Kim, Sangheon Kim, Hyunkyu Yu Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | This contribution proposes the texts related to Feedback Polling A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up on Feedback Polling A-MAP IE (16.3.5.5) and UL codebook (16.3.9.3)

Taeyoung Kim, Sangheon Kim, Hyunkyu Yu

Samsung Electronics Co., Ltd.

# Introduction

In Table 857 – Feedback Polling A-MAP IE, the reserved bit is not included. So, this contribution inserts the reserved bit for the Feedback Polling A-MAP IE.

In order to de-allocate the uplink resource for MIMO feedback in Polling_sub_type=0b0 (e.g., uplink resource allocation), Polling_deallocation_bitmap (3bit) is not necessary because de-allocation for Polling_sub_type=0b0 is to de-allocate the resource indicated by Resource Index (11bit).

In addition, the expression for UL codebook shown in Table 951 is wrong. Therefore, we propose the modification of the expression by directly assigning the element of UL codebook to Cbase,UL matrix.

# Text proposal for inclusion in the 802.16m amendment

<Remedy-1: Modify section “Table 857 – Feedback Polling A-MAP IE” on page 703 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

Table 857 – Feedback Polling A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| … | … | … |
| if (d == 0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Polling_deallocation_bitmap | 3 |  |
| HFA | 6 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| Reserved | 15 |  |
| } else if (d != 0b000){ |  | Feedback allocation |
| … | … | … |

-------------------------------  End of Text Proposal  ---------------------------------------------------

<Remedy-2: Modify section “Table 857 – Feedback Polling A-MAP IE” on page 710 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

Table 857 – Feedback Polling A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MaxUser | 2 | … |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | To reach 40-bit assignment A-MAP IE size |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

-------------------------------  End of Text Proposal  ---------------------------------------------------

<Remedy-3: Modify section “Table 951 – Cbase,UL(4,4,6)” on page 870 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

Table 951 – Cbase,UL(4,4,6)

| Binary Index | m | Cbase,UL(4,4,6,m)=[c1;c2;c3;c4]c11c12c13c14c21c22c23c24c31c32c33c34c41c42c43c44 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c1c11 c21 c31 c41 | c2 c12 c22 c32 c42 | c3 c13 c23 c33 c43 | c4 c14 c24 c34 c44 |
|  |  | … | … | … | … |

-------------------------------  End of Text Proposal  ---------------------------------------------------