| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-NBR-ADV and AAI-SCN-XXX message table and ASN.1 code Clarification and Correction |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Joey Chou Xiangying Yang Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D10) |  |
| Abstract | AAI-NBR-ADV and AAI-SCN-XXX message table and ASN.1 code Clarification and correction |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI-NBR-ADV and AAI-SCN-XXX message table and ASN.1 code Clarification and Correction

Joeyn Chou,

Xiangying Yang

Intel

Introduction

This contribution proposes changes to clarify and correct AAI-NBR-ADV and AAI-SCN-XXX messages.

Proposed Text

Remedy 1:

In the P802.16m/D10, page 134, line5, modify as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.13 AAI-NBR-ADV

To allow AAI-NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI-NBR-ADV message, ABS always provides the total number of cell types, total number of segments for each type and the total number of neighbor ABS per segment. Each AAI-NBR-ADV fragment is indicated by AAI-NBR-ADV Segment Index. Once an AAI-NBR-ADV segment is sent for a given cell type, the ABS should not start sending ABS for different cell type until all AAI-NBR-ADV segment for such given cell type are sent. ABSs with identical type are listed in the AAI-NBR-ADV message in descending order of their cell coverage.

Table 690—AAI-NBR-ADV Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI-NBR-ADV segment. If this is the first AAI-NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI-NBR-ADV segment has one Index which corresponds to the first ABS in that AAINBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index that has the range of [1..256]. |  |
| For (i=0; i<N-NBR-ABSs; i++) { |  | N-NBR-ABSs is the number of neighbor ABSs included in this message, and has the range of [1..64] |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D10, page 140, line 6, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.14 AAI-SCN-REQ

Table 691—AAI-SCN-REQ Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | 8 [1..256] | Each bit position corresponds to a ABS Index of the corresponding AAINBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D10, page 144, line 17, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.15 AAI-SCN-RSP

Table 692—AAI-SCN-RSP Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| If (Req_Bitmap_Index is present) { |  |  |  |
| Req_Bitmap_Index | variable [1..256] | Each bit position in this bitmap corresponds to a ABS Index of the corresponding AAI-SCN-REQ message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last recommended ABS, and ABSs with ABS Index greater than the last recommended BS are not recommended and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not recommended. 1: the corresponding ABS is recommended. Maximum size of this bitmap is 64 256 |  |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | 8 [1..256] | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 4:

In the P802.16m/D10, page 149, line 6, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.16 AAI-SCN-REP

Table 693—AAI-SCN-REP Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | 8 [1..256] | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits |  |
| If (Rsp_Bitmap_Index is present) { |  |  |  |
| Rsp_Bitmap_Index | variable [1..256] | Index of the corresponding AAI-SCN-RSP message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last reported ABS, and ABSs with ABS Index greater than the last reported ABS are not reported and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not reported. 1: the corresponding ABS is reported. Maximum size of this bitmap is 64 256 |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------