| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Sounding Power Control |  |
| Date Submitted | 2011-1-10 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D10 |  |
| Abstract | This contribution proposes text for clarification on capacity negotiation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D11 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Sounding Power Control

Jeongho Park

Samsung Electronics

- References

[1] IEEE 802.16m/D10

- Backgrounds and Scopes

Draft D10 defines sounding channel power control, but it is ambiguous when we think of tone power based on FDM sounding sequence.

The sounding signal transmitted by the AMS is decided by equation (277).

Here bk is the occupied k-th subcarrier which is modulated by BPSK symbols based on Golay sequence.

And two multiplexing methods (FDM, CDM) have different tone density for bk.

[FDM]

[CDM]

Since FDM uses decimated subcarriers only, tone power for used subcarrier is boosted by ‘D’ which is decimation value.

On the other hand, the sounding power control determines the tone power of sounding channel according to parameters that are given by ABS or obtained by AMS (16.3.8.4.5). But the description of relationship between bk and tone power decided by power control is not sufficiently clear.

“The power per subcarrier shall be maintained for the UL sounding transmission as shown in Equation (291) of 16.3.8.4.1.”

This text only considered CDM based sequence which utilizes all subcarriers and does not tone power boosting. When it comes to FDM with power boosting with ‘D’, SPEC is ambiguous. Therefore we need to additionally define the relationship between UL sounding power control and sequence symbol.

- Proposed Text

Remedy #1: Modify line 28~36 of page 890 (856) as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

16.3.8.4.5 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS.’s transmit power for UL sounding channel is controlled separately according to its sounding channel target SINR value. In case of CDM based sounding (multiplexing with cyclic time shift), the transmission tone power of sounding sequence, the power of bk of Equation (277), shall be P(dBm) of Equation (291), which is obtained by this section. In case of FDM based sounding (multiplexing with decimation offset), the transmission tone power of sounding sequence, the power of bk of Equation (277), shall be P(dBm) of Equation (291), which is obtained by this section, plus decimationValueD in (dB) level which is transmitted in AAI-SCD message. The power per subcarrier shall be tone maintained for the UL sounding transmission as shown in Equation (291) of 16.3.8.4.1.

-------------------------------------------------Text Change END ---------------------------------------------------------------