| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-SCN message table and ASN.1 cleanup (16.2.3) |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Joey Chou, Xiangying Yang Intel Corporation | xiangying.yang@intel.com |
| Re: | Sponsor ballot comment on P802.16m/D10 Target topic: “IEEE P802.16m/D10, section 16.2.3”. |  |
| Abstract | The contribution proposes a restructuring of AAI-SCN-REQ/RSP/REP messages |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI-SCN message table and ASN.1 cleanup (16.2.3)

Joey Chou, Xiangying Yang

Intel Corporation

- Introduction

There is a major structural issue about SCN-REQ/RSP/REP messages. The current structure is complicated and there is a lot redundancy, particularly the table.

To our understanding, it is all about how to optimize the way of indication list of BSs. We propose the following change

For all SCN-RXX message, it takes a simple form of

ListOfBS

Sequence array of per BS related info

In which the ListOfBS is a CHOICE of a few possible methods:

1. list of full BSIDs

2. list of preambles (this may be merged with 1 or taken out the CHOICE)

3. list of indexes corresponding to NBR-ADV index

4. a bitmap corresponding to NBR-ADV index

5. a smaller bitmap corresponding to the “1”s in the bitmap received in the last message

Method 1,2,3,4 applies to all SCN-Rxx. Method 5 only applies to SCN-RSP/REP

So the only limitation of this cleanup is that in one single message, one can NOT combine method a with method b (which seem to be allowed to some extend by current message table format. For example, in one SCN-RSP message 3 BS are indicated using full BSID and 4 BS are indicated using bitmap. We believe we should not allow this arbitrary combination in one message anyway.

Reference

[1] IEEE P802.16m/D10, Nov. 2010

- Text change

------------------------------------------ Text Start ---------------------------------------------------

Remedy #1

[Replace Table 691 as following:]

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Interleaving interval | 8 | The period of AMS's Active Mode (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| Scan Iteration | 6 | The requested number of iterating scanning interval by an AMS. |  |
| Recommended start super frame number | 6 | Represents the 6 least significant bits of the absolute super frame index for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Present if AMS recommends a start super frame |
| Recommended start frame index | 2 | Recommended start frame index within a super frame. 0b00: the 1st frame in a super frame 0b01: the 2nd frame in a super frame 0b10: the 3rd frame in a super frame 0b11: the 4th frame in a super frame | Present if AMS recommends a start super frame |
| N_Recommended_ABS | 6 | This is the number of neighboring ABS the AMS plans to scan, which are included in AAI-NBR-ADV message [0..63]. When an AMS receives AAI-SCNRSP message from ABS in response to AAI-SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAISCN- RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI-SCN-REQ message to the ABS. | Present if AMS uses index instead of full 48- bits BSID to identify an ABS it plans to scan |
| If(N_Recommended_ABS > 0) { | - |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | [0..255] | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR- ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 Bits Total number of “1”s in the bitmap shall be equal to N_Recommended_ABS |  |
| } else if (ABS indexes in AAI-NBR-ADV is present) { |  |  |  |
| For(i=0; i<N_Recommended_ABS;i++) { | - |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| } |  |  |  |
| } else if (full ABS ID is present) { |  |  |  |
| For(i=0; i<N_Recommended_ABS;i++) { |  |  |  |
| Recommended ABS ID | 48 | ABS ID of the ABSs the AMS plans to scan |  |
| } |  |  |  |
| } else { |  |  |  |
| For(i=0; i< N_Recommended_ABS; i++) { |  |  |  |
| SA Preamble Index | 6 | SA Preamble indices the AMS plans to scan |  |
| } |  |  |  |
| }//end of If(N_Recommended_ABS > 0) |  |  |  |
| For(i=0; i< N_Recommended_ABS; i++) { | - |  |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0…63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0;j<N_Recommended_Carrier_Index;j++) { | - |  |  |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS |  |
| } |  |  |  |
| } |  |  |  |
| N_Recommended_Carrier_Index_at_Serving_ABS | 6 | Number of carriers the AMS plans to scan at each neighbor ABS. | Present if AMS decides to scan other carriers of the S-ABS |
| For(i=0; i< N_Recommended_Carrier_Index_at_Serving_ABS; i++) { |  |  |  |
| Recommended Carrier index at S-ABS |  | Recommended physical carrier index of the S-ABS the AMS plans to scan | Present if AMS decides to scan other carriers of the S-ABS |
| } |  |  |  |
| for (i=0; i<N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks [0…15]. | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

------------------------------------------------ Text End ---------------------------------------------------