| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-SCN message table and ASN.1 cleanup (16.2.3) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Joey Chou, Xiangying Yang Intel Corporation YanXiu Zheng ITRI Ji-Yun Seol Hyunjeong Kang Jaehyuk Jang Samsung Erik Colban Jia Lin Huawei | xiangying.yang@intel.com zhengyanxiu@itri.org.tw jiyun.seol@samsung.com |
| Re: | Sponsor ballot comment on P802.16m/D10 Target topic: “IEEE P802.16m/D10, section 16.2.3”. |  |
| Abstract | The contribution proposes a restructuring of AAI-SCN-REQ/RSP/REP messages |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI-SCN message table and ASN.1 cleanup (16.2.3)

Joey Chou, Xiangying Yang

Intel Corporation

YanXiu Zheng

ITRI

Ji-Yun Seol, Hyunjeong Kang, Jaehyuk Jang

Samsung

Erik Colban, Jia Lin

Huawei

- Introduction

There is a major structural issue about SCN-REQ/RSP/REP messages. The current structure is complicated and there is a lot redundancy, particularly the table.

To our understanding, it is all about how to optimize the way of indication list of BSs. We propose the following change

For all SCN-RXX message, it takes a simple form of

ListOfBS

Sequence array of per BS related info

In which the ListOfBS is a CHOICE of a few possible methods:

1. list of full BSIDs

2. list of preambles (this may be merged with 1 or taken out the CHOICE)

3. list of indexes corresponding to NBR-ADV index

4. a bitmap corresponding to NBR-ADV index

5. a smaller bitmap corresponding to the “1”s in the bitmap received in the last message

Method 1,2,3,4 applies to all SCN-Rxx. Method 5 only applies to SCN-RSP/REP

So the only limitation of this cleanup is that in one single message, one can NOT combine method a with method b (which seem to be allowed to some extend by current message table format. For example, in one SCN-RSP message 3 BS are indicated using full BSID and 4 BS are indicated using bitmap. We believe we should not allow this arbitrary combination in one message anyway.

Reference

[1] IEEE P802.16m/D10, Nov. 2010

- Text change

------------------------------------------ Proposed Text Starts ---------------------------------------------------

Remedy #1

[Change Table 691 as following:]

Table 691.—AAI-SCN-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Interleaving interval | 8 | The period of AMS's Active Mode (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| Scan Iteration | 6 | The requested number of iterating scanning interval by an AMS. |  |
| Recommended start super frame number | 6 | Represents the 6 least significant bits of the absolute super frame index for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Present if AMS recommends a start super frame |
| Recommended start frame index | 2 | Recommended start frame index within a super frame. 0b00: the 1st frame in a super frame 0b01: the 2nd frame in a super frame 0b10: the 3rd frame in a super frame 0b11: the 4th frame in a super frame | Present if AMS recommends a start super frame |
| N_Recommended_ABS_Index | 6 | This is the number of neighboring ABS the AMS plans to scan, which are included in AAI-NBR-ADV message [0..63]. When an AMS receives AAI-SCN-RSP message from ABS in response to AAI-SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI-SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI-SCN-REQ message to the ABS. | Present if AMS uses index instead of full 48-bits BSID to identify an ABS it plans to scan |
| If(N_Recommended_ABS_Index > 0) { |  |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | Variable 8 [1..256] | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits Total number of “1”s in the bitmap shall be equal to N_Recommended_ABS. |  |
| } // End If (Nbr_Bitmap_Index is present) |  |  |  |
| Else if (ABS indexes are present) { |  |  |  |
| For(i=0; i< N_Recommended_ABS; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| } |  |  |  |
| } // End if (ABS indexes are present) |  |  |  |
| else { // Full ABS ID is present |  |  |  |
| For(i=0; i<N_Recommended_ABS;i++) { |  |  |  |
| Recommended ABS ID | 48 | Full ABS ID of the ABSs that are listed in AAI-NBR-ADV |  |
| } |  |  |  |
| } // End else Full ABS ID is present |  |  |  |
| For(i=0; i< Num of bit which is set to 1 in Nbr_Bitmap_Index; i++) { |  |  |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS |  |
| } |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| For(i=0; i< N_Recommended_ABS_Index; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  | Present when a neighbor ABS is a multicarrier ABS |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS | Present when a neighbor ABS is a multicarrier ABS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| }//end of If(N_Recommended_ABS_Index > 0) |  |  |  |
| N_Recommended_ABS_Full | 6 | Number of neighbor ABS the AMS plans to scan which are using full 48-bit ABSID [0..63] that are not listed in AAI-NBR-ADV. | Present if AMS decides to use full 48 bits BSID to identify an ABS that is not listed in AAI-NBR-ADV |
| For(i=0; i< N_Recommended_ABS_Full; i++) { |  |  |  |
| Recommended ABS ID | 48 | ABS ID of the ABSs the AMS plans to scan |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0..63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS | Present when a neighbor ABS is a multicarrier ABS |
| } |  |  |  |
| } |  |  |  |
| N_Recommended_BS_Full |  | Number of neighbor legacy BS the AMS plans to scan which are using full 48-bit BSID [0..63]. |  |
| If(N_Recommended_BS_Full > 0) { |  |  | Present if AMS decides to use full 48 bits BSID to identify a legacy BS |
| For(i=0; i<N_Recommended_BS_Full; i++) { |  |  |  |
| Recommended BS ID | 48 | BS ID of the legacy BSs the AMS plans to scan |  |
| } |  |  |  |
| }//end of if(N_Recommended_BS_Full > 0) |  |  |  |
| N_Recommended_FA_of_SA_Preamble_Index |  | Number of FAs the AMS plans to scan SA preambles [0..15] |  |
| For(j=0; j< N_Recommended_FA_of_SA_Preamble_Index; j++) { |  |  | Present if AMS requests to scan SA preambles |
| Center Frequency | 32 | Indicates center frequency (in unit of Hz) of the carrier |  |
| N_Recommended_SA_Preamble_Index | 4 | Number of SA preambles the AMS plans to scan [0..15] | Present if AMS requests to scan SA preambles |
| For(i=0; i< N_Recommended_SA_Preamble_Index; i++) { |  |  | Present if AMS requests to scan SA preambles |
| SA Preamble Index | 6 10 | SA Preamble indices the AMS plans to scan | Present if AMS requests to scan SA preambles |
| } |  |  |  |
| } // End For(j=0; j< N_Recommended_FA_of_SA_Preamble_Index |  |  |  |
| N_Recommended_Carrier_Index_at_Serving_ABS | 6 | Number of carriers [0..63] the AMS plans to scan at each neighbor ABS. | Present if AMS decides to scan other carriers of the S-ABS |
| For(i=0; i< N_Recommended_Carrier_Index_at_Serving_ABS; i++) { |  |  |  |
| Recommended Carrier index at S-ABS | 6 | Recommended physical carrier index of the S-ABS the AMS plans to scan | Present if AMS decides to scan other carriers of the S-ABS |
| } |  |  |  |
| for (i=0; i<N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks [0..15]. | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

------------------------------------------ Propose Text Ends ---------------------------------------------------

Remedy #2

[Change Table 692 as following:]

------------------------------------------ Propose Text Starts ---------------------------------------------------

Table 692.—AAI-SCN-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Report mode | 2 | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| Report metric | 4 | Bitmap indicator of trigger metrics that the S-ABS requests the AMS to report. The S-ABS shall indicate only the trigger metrics agreed during AAI-SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: ABS CINR mean Bit 1: ABS RSSI mean Bit 2: Relative delay Bit 3: ABS RTD |  |
| Report period | 8 | The period of AMS's report of scanning result when the AMS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the ABS sends an unsolicited AAI-SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the AMS receives the AAI-SCN-RSP message. For AMS scanning request denied by AAI-SCN-RSP with Scan Duration set to zero, Report period is the number of frames that ABS suggests to AMS before transmitting next AAI-SCN-REQ |  |
| Duration for measurements signal location parameters on D-LBS zone | 4 | Duration (in units of 4 superframes) of the requested scanning. | Present if ABS request AMS to scan D-LBS zone for signal location parameters measurements |
| If(Scan Duration > 0) { |  |  |  |
| Start superframe index | 6 | Represents the 6 least significant bits of the absolute superframe index in which the first Scanning interval starts |  |
| Start frame index | 2 | Represents start frame number in the Start super frame number. 0b00: 1st frame in the super frame 0b01: 2nd frame in the super frame 0b10: 3rd frame in the super frame 0b11: 4th frame in the super frame |  |
| Interleaving interval | 8 | The period of AMS's Active Mode (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| Scan Iteration | 6 | The requested number of iterating scanning interval by an AMS. |  |
| N_Recommended_ABS_Full | 6 | Number of neighbor ABS [0..63] to be scanned, which are using full 48-bit BSID. | Present only this message is sent in response to AAI-SCN-REQ with N_Recommended_ABS_ Full_indication > 0 |
| If(N_Recommended_ABS_Full > 0) { |  |  | Present only this message is sent in response to AAI-SCN-REQ |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| If (Req_Bitmap_Index is present) { |  |  |  |
| Req_Bitmap_Index | Variable [1..256] | Each bit position in this bitmap corresponds to a ABS Index of the corresponding AAI-SCN-REQ message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last recommended ABS, and ABSs with ABS Index greater than the last recommended BS are not recommended and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not recommended. 1: the corresponding ABS is recommended. Maximum size of this bitmap is 64256 Total number of “1”s in the bitmap shall be equal to N_Recommended_ABS. |  |
| } // End If (Req_Bitmap_Index is present) |  |  |  |
| Else if (ABS indexes are present) { |  |  |  |
| For(i=0; i< N_Recommended_ABS; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| } |  |  |  |
| } // End if (ABS indexes are present) |  |  |  |
| else { // Full ABS ID is present |  |  |  |
| For(i=0; i<N_Recommended_ABS;i++) { |  |  |  |
| Recommended ABS ID | 48 | Full ABS ID of the ABSs that are listed in AAI-NBR-ADV |  |
| } |  |  |  |
| } // else Full ABS ID is present |  |  |  |
| For(i=0; i<N_Recommended_ABS;i++) { |  |  |  |
| N_Recommended_Carrier_Index |  | Number of carriers the AMS plans to scan at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  | Present when a neighbor ABS is a multicarrier ABS |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS | Present when a neighbor ABS is a multicarrier ABS |
| SA Preamble Index | 10 | SA Preamble Index to be scanned |  |
| }//end of (i=0; i<N_Recommended_ABS;i++) |  |  |  |
| }//end of if(N_Recommended_ABS > 0) |  |  |  |
| For(i=0; i< Num of bit which is set to 1 in Req_Bitmap_Index; i++) { |  |  |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended_Carrier_Index | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-CFG message |  |
| SA Preamble Index | 10 | SA Preamble Index to be scanned |  |
| CP length | 2 | CP length of the recommended T-ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when neighbor ABS is not included in AAI-NBR-ADV message and AAI-SCN-RSP is not sent as response to AAI-SCN-REQ which included this recommended carrier index |
| } |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| N_Recommded_ABS_Full | 6 | Full ABS ID of the ABSs that are not listed in AAI-NBR-ADV [0..63] |  |
| For(i=0; i< N_Recommended_ABS_Full; i++) { |  |  | Shall be included when NBR ABS is not included in AAI-NBRADV message and AAI-SCN- RSP is not sent as response to AAI-SCNREQ which included this recommended carrier index |
| Recommended ABS ID | 48 | ABS ID of the ABSs to be scanned |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended_Carrier_Index | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-CFG message |  |
| SA Preamble Index | 10 | SA Preamble Index to be scanned |  |
| Bandwidth | 3 | BW size of the recommended neighbor ABS 0b000: 10MHz 0b001:20MHz 0b010: 5MHz 0b011: 7MHz 0b100: 8.75MHz 0b101~0b111: reserved |  |
| CP length | 2 | CP length of the recommended T-ABS 00: 1/8 01: 1/16 10: ¼ |  |
| } |  |  |  |
| }//end of if(N_Recommended_ABS_Full > 0) For(j=0; j< N_Recommended_Carrier_Index |  |  |  |
| }//end of For(i=0; i< N_Recommended_ABS_Full; i++) |  |  |  |
| }//end of if(Scan Duration > 0) |  |  |  |
| N_Recommended_ABS_Index | 6 | This is the number of neighbor ABS [0..63] to be scanned, which are included in AAI-NBR-ADV message. When an AMS receives AAI-SCN-RSP message from ABS in response to AAI-SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI-SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI-SCN-REQ message to the ABS. | Present if ABS uses index to identify an ABSs to be scanned |
| If(N_Recommended_ABS_Index > 0) { |  |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| If (Nbr_Bitmap_Index is present) { |  |  |  |
| Nbr_Bitmap_Index | 8 | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits |  |
| For(i=0; i< Num of bit which is set to 1 in Nbr_Bitmap_Index; i++) { |  |  |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-CFG message |  |
| } |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| For(i=0; i< N_Recommended_ABS_Index; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS to be scanned in AAI-NBR-ADV message |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-CFG message |  |
| } |  |  |  |
| } | - |  |  |
| } |  |  |  |
| }//end of if(N_Recommended_ABS_Index > 0) |  |  |  |
| N_Recommended_BS_Full |  | Number of neighbor legacy BS the AMS plans to scan which are using full 48-bit BSID [0..63]. |  |
| If(N_Recommended_BS_Full > 0) { |  |  | Present if AMS decides to use full 48 bits BSID to identify a legacy BS |
| For(i=0; i<N_Recommended_BS_Full; i++) { |  |  |  |
| Recommended BS ID | 48 | BS ID of the legacy BSs the AMS plans to scan |  |
| } |  |  |  |
| }//end of if(N_Recommended_BS_Full > 0) |  |  |  |
| N_Recommended_FA_of_SA_Preamble_Index |  | Number of FAs to be scanned for SA preambles [0..15] |  |
| For(j=0; j< N_Recommended_FA_of_SA_Preamble_Index; j++) { |  |  | Present if this message is sent in response to AAI-SCN-REQ with N_Recommended_FA_of_SA_Preamble_Index > 0 |
| Center Frequency | 32 | Indicates center frequency (in unit of Hz) of the carrier |  |
| N_Recommended_SA_Preamble_Index |  | Number of SA preambles to be scanned [0..15] |  |
| For(i=0; i<N_Recommended_SA_Preamble_Index; i++) { |  |  |  |
| SA Preamble Index | 10 | SA Preamble indices to be scanned |  |
| } |  |  |  |
| } |  |  |  |
| N_Recommended_Carrier_Index_at_Serving_ABS | 6 | Number of carriers to be scanned at the S-ABS | Present if AMS decides to scan other carriers of the S-ABS |
| For(i=0; i< N_Recommended_Carrier_Index_at_Serving_ABS; i++) { |  |  |  |
| Recommended Carrier index at S-ABS | 6 | Recommended physical carrier index of the S-ABS the AMS plans to scan | Present if AMS decides to scan other carriers of the S-ABS |
| } |  |  |  |
| Scanning_Carrier_Index | 6 | Recommended physical carrier index of the AMS to be used for scanning | Present if the AMS supports multi-carrier scanning. |
| } // End If(Scan Duration > 0) |  |  |  |

------------------------------------------ Propose Text Ends ---------------------------------------------------

Remedy #3

[Change Table 693 as following:]

------------------------------------------ Propose Text Starts ---------------------------------------------------

Table 693.—AAI-SCN-REP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Report mode | 2 | Action code for an AMS's scan report of its measurement 00: Event-triggered report 01: Periodic report according to Scan report period of AAI-SCN-RSP 10: One-time scan report |  |
| For(i=0; i<N-Carriers-SABS; i++) { |  | N-Carriers-SABS is the number of carriers at S-ABS to be reported in this message | Present if AMS reports measurement results measured from carriers at S-ABS |
| Carrier Index | 6 | Carrier Index of carrier at S-ABS |  |
| S-ABS CINR mean | 8 | The S-ABS CINR Mean parameter indicates the CINR measured by the AMS from the S-ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| S-ABS RSSI mean | 8 | The S-ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the S-ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the -103.75 dBm to -40 dBm. An AMS with multiple receive antennas will report the linear sum of the RSSI values calculated over all the receive antennas. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| S-ABS RTD | 8 | The S-ABS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the S-ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. | Present if AMS is asked to report RTD |
| N_Reported_ABS_Index | 6 | Number of neighbor ABS reported in this message [0..63] and which are included in AAI-NBR-ADV message [0..63]. | Present only when the scanning report includes ABSs referred by ABS index |
| If(N_Reported_ABS_Index > 0) { |  |  | Present only when the scanning report includes ABSs referred by ABS index |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| If (NbrRsp_Bitmap_Index is present) { |  |  |  |
| NbrRsp _Bitmap_Index | 8 | Each bit position corresponds to a ABS Index of the corresponding AAI-SCN-RSPNBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested Maximum size of the bitmap is 256 bits Total number of “1”s in the bitmap shall be equal to N_Reported_ABS. |  |
| } // End If (Rsp _Bitmap_Index is present) |  |  |  |
| Else if (ABS indexes are present) { |  |  |  |
| For(i=0; i< N_Reported_ABS; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| } |  |  |  |
| } // End if (ABS indexes are present) |  |  |  |
| else { // Full ABS ID is present |  |  |  |
| For(i=0; i<N_Reported_ABS;i++) { |  |  |  |
| Reported ABS ID | 48 | Full ABS ID of the ABSs the AMS plans to report |  |
| } |  |  |  |
| } // else Full ABS ID is present |  |  |  |
| For(i=0; i< Num of bit which is set to 1 in Nbr_Bitmap_Index; i++) { |  |  |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| Relative delay | 8 | This parameter indicates the delay of the DL signals from the neighbor-ABS relative to the primary carrier of the S-ABS, as measured by the AMS. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } |  |  |  |
| } else { |  |  |  |
| For(i=0; i< N_Reported_ABS_Index; i++) { |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { |  |  | Present when a neighbor ABS is a multicarrier ABS |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } // End For(j=0; j< N_Reported_Carrier_Index |  |  |  |
| Relative delay | 8 | This parameter indicates the delay of the DL signals from the neighbor-ABS relative to the primary carrier of the S-ABS, as measured by the AMS. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } // End For(i=0; i< N_Reported_ABS |  |  |  |
| } |  |  |  |
| }//end of if(N_Reported_ABS_Index > 0) |  |  |  |
| N_Reported_ABS_Full | 6 | Number of neighbor ABS [0..63] reported in this message, which are using full 48-bit | Present only when the scanning report includes ABSs referred by ABS ID |
| If (N_Reported_ABS_Full > 0) { |  |  |  |
| If (Rsp_Bitmap_Index is present) { |  |  |  |
| Rsp_Bitmap_Index | Variable | Index of the corresponding AAI-SCN-RSP message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last reported ABS, and ABSs with ABS Index greater than the last reported ABS are not reported and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not reported. 1: the corresponding ABS is reported. Maximum size of this bitmap is 64 |  |
| For(i=0; i< Num of bit which is set to 1 in Rsp_Bitmap_Index; i++) { |  |  |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| Relative delay | 8 | This parameter indicates the delay of the DL signals from the neighbor-ABS relative to the primary carrier of the S-ABS, as measured by the AMS. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } |  |  |  |
| } else { |  |  |  |
| For(i=0; i< N_Reported_ABS_Full; i++) { |  |  |  |
| Reported ABS ID | 48 | ABS-IDs of the ABSs reported in this message |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0..63] | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { |  |  |  |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| Relative delay | 8 | This parameter indicates the delay of the DL signals from the neighbor-ABS relative to the primary carrier of the S-ABS, as measured by the AMS. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } |  |  |  |
| } |  |  |  |
| }//end of If (N_Reported_ABS_Full > 0) |  |  |  |
| N_Reported_BS_Full |  | Number of neighbor legacy BS [0..63] reported in this message, which are using full 48-bit BSID. |  |
| If (N_Reported_BS_Full > 0) { |  |  | Present only when the scanning report includes legacy BSs referred by BS ID |
| For(i=0; i< N_Reported_BS_Full; i++) { |  |  |  |
| Reported BS ID | 48 | BS ID of the legacy BSs reported in this message |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| }//end of If (N_Reported_BS_Full > 0) |  |  |  |
| N_Reported_FA_of_SA_Preamble_Index |  | Number of FAs reported in this message for SA preambles [0..15] |  |
| For(j=0; j< N_Reported_FA_of_SA_Preamble_Index; j++) { |  |  | Present only when the scanning report includes SA preamble reports |
| Center Frequency | 32 | Indicates center frequency (in unit of Hz) of the carrier |  |
| N_Reported_SA_Preamble_Index | 4 | Number of SA preambles reported in this message [0..15] | Present only when the scanning report includes SA preamble reports |
| For(i=0; i< N_Reported_SA_Preamble_Index; i++) { |  |  | Present only when the scanning report includes SA preamble reports |
| SA Preamble Index | 10 | SA preamble indices reported in this message |  |
| CINR mean | 8 | Indicates the CINR measured by the AMS from this SA preamble. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | Indicates the Received Signal Strength measured by the AMS from this SA preamble. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| } |  |  |  |
| Neighbor_Request_Indication | 1 | Request indication for system information and the list of the neighbor Femto ABSs | Optional |
| If(Neighbor_Request_Indication == 1) { |  |  |  |
| Neighbor_Request_BS_Type | 3 | Indicates type of neighbor Femto ABSs for which system information is requested. Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS | Optional |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs | Optional. Sent by AMS to aid in generation of optimized neighbor list by the ABS |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

------------------------------------------ Propose Text Starts ---------------------------------------------------

Remedy #4

[ReplaceASN.1 code from Page 1057, Line 26 to page 1058, line 27, as the following:]

------------------------------------------ Propose Text Starts ---------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Interval Allocation Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

SuperframeNumberLSB ::= INTEGER (0..63)

RecommandCarrierIndexList ::= SEQUENCE (SIZE (0..63)) OF PhyCarrierIndex

NbrBitmapInfo ::= SEQUENCE {

nbrBitmap                      BIT STRING (SIZE (1..256)),

nbrInfoList                    SEQUENCE (SIZE (1..64)) OF

RecommandCarrierIndexList

}

NbrIndexInfo ::= SEQUENCE {

nbrABSIndex                    INTEGER (0..255),

recommandCarrierIndexList      RecommandCarrierIndexList

}

NbrFullInfo ::= SEQUENCE {

nbrABSID                       BSID,

recommandCarrierIndexList      RecommandCarrierIndexList

}

SaFaOfPreamble ::= SEQUENCE {

centerFrequency                CenterFreq,

saPreambleIndexList            SEQUENCE (SIZE (0..15)) OF PreambleIndex

}

NbrLegacyFullInfo ::= SEQUENCE {

nbrBSID                        BSID

}

AAI-SCN-REQ ::= SEQUENCE {

scanDuration                   INTEGER (0..255),

interleaving                   INTEGER (0..255),

scanIteration                  INTEGER (0..63),

recommendStartSuperFrame       SuperframeNumberLSB OPTIONAL,

recommendedStartFrame          INTEGER (0..3) OPTIONAL,

nRecommendedABS                CHOICE {

zeroRecommendABS             NULL,

nonzeroRecommendABS          SEQUENCE {

configChangeCount          INTEGER (0..7),

nbrIndication              CHOICE {

nbrBitmapInfo            NbrBitmapInfo,

nbrIndexInfoList         SEQUENCE (SIZE (1..64)) OF NbrIndexInfo,

fullAbsIdList            SEQUENCE (SIZE (1..64)) OF NbrFullInfo

}

}

},

nbrLegacyFullInfoList          SEQUENCE (SIZE (0..63)) OF NbrLegacyFullInfo,

saFaOfPreambleList             SEQUENCE (SIZE (0..15)) OF SaFaOfPreamble,

sabsCarrierIndexList           SEQUENCE (SIZE (0..63)) OF PhyCarrierIndex,

csgIDInfoList                  SEQUENCE (SIZE (0..15)) OF SEQUENCE {

csgFemtoCellOperatorID       BIT STRING (SIZE (24)),

csgIDList                    SEQUENCE (SIZE (1..64)) OF CSGID

}

}

------------------------------------------ Propose Text Ends ---------------------------------------------------

Remedy #5

[Replace ASN.1 code from Page 1058, Line 28 to page 1059, line 57 as the following:]

------------------------------------------ Propose Text Starts ---------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Interval Allocation Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

RecommandCarrierInfo ::= SEQUENCE {

carrierIndex                   PhyCarrierIndex,

saPreambleIndex                PreambleIndex

}

ReqBitmapInfo ::= SEQUENCE {

bitmapIndex                    BIT STRING (SIZE (1..64)),

absInfoList                    SEQUENCE (SIZE (1..64)) OF

RecommandCarrierInfo

}

ABSFullInfo ::= SEQUENCE {

absID                          BSID,

absInfo                        RecommandCarrierInfo

}

RecommendABSIndexInfo ::= SEQUENCE {

nbrAbsIndex                    INTEGER (0..255),

absInfo                        RecommandCarrierInfo

}

AbsFullNotInNbrAdvList ::= SEQUENCE {

absID                          BSID,

recommendedCarrierList         SEQUENCE (SIZE (1..64)) OF SEQUENCE {

recommandCarrierIndex        RecommandCarrierInfo,

bandwidth                    INTEGER {

tenMHz (0),

twentyMHz (1),

fiveMHz (3),

sevenMHz (4),

eightDotSevenFiveMHz (5)

} (0..7)

}

}

ScanInfo ::= SEQUENCE {

scanDuration                   INTEGER (1..255),

startSuperFrame                SuperframeNumberLSB,

startFrame                     INTEGER (0..3),

interleavingInterval           INTEGER (0..255),

scanIteration                  INTEGER (0..63),

nbrAdvChangeCount              INTEGER (0..7),

nbrIndication                  CHOICE {

nbrBitmapInfo                ReqBitmapInfo,

nbrIndexInfoList             SEQUENCE (SIZE (1..64)) OF RecommendABSIndexInfo,

fullAbsIdList                SEQUENCE (SIZE (1..64)) OF ABSFullInfo

},

absFullNotInNbrAdvList         SEQUENCE (SIZE (0..63)) OF AbsFullNotInNbrAdvList OPTIONAL,

nbrLegacyFullInfoList          SEQUENCE (SIZE (0..63)) OF NbrLegacyFullInfo,

saFaOfPreambleList             SEQUENCE (SIZE (0..15)) OF SaFaOfPreamble,

recommendSABSCarriers          SEQUENCE (SIZE (0..63)) OF PhyCarrierIndex,

scanningCarrierIndex           PhyCarrierIndex OPTIONAL

}

AAI-SCN-RSP ::= SEQUENCE {

scanDuration                   CHOICE {

zeroScanDuration               NULL,

nonzeroScanDuration            ScanInfo

},

reportMode                     ENUMERATED {

noReport,

periodicReport,

eventTriggerReport,

oneTimeReport

},

reportMetric                   BIT STRING {

cinrMean (0),

rssiMean (1),

relativeDelay (2),

absRTD (3)

},

reportPeriod                   INTEGER (0..255),

d-LBS-measDuration             INTEGER (0..15) OPTIONAL

}

------------------------------------------ Propose Text Ends ---------------------------------------------------

Remedy #6

[Replace ASN.1 code from Page 1059, Line 58 to page 1061, line 31, as the following:]

------------------------------------------ Propose Text Starts ---------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Result Report

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

ScanReport ::= SEQUENCE {

carrierIndex                   PhyCarrierIndex,

cinrMean                       INTEGER (0..255) OPTIONAL,

rssiMean                       INTEGER (0..255) OPTIONAL

}

CarrierReportList ::= SEQUENCE {

scanReportList                 SEQUENCE (SIZE (0..63)) OF ScanReport,

rtd                            INTEGER (0..255) OPTIONAL

}

NbrBitmapReport ::= SEQUENCE {

repBitmap                       BIT STRING (SIZE (1..64)),

nbrReportList                  CarrierReportList

}

NbrIndexReport ::= SEQUENCE {

nbrABSIndex                    INTEGER (0..255),

nbrReport                      CarrierReportList

}

ABSReport ::= SEQUENCE {

reportedABSID                  BSID,

carrierReportList              CarrierReportList

}

NbrLegacyFullReport ::= SEQUENCE {

reportBSID                     BSID,

cinrMean                       INTEGER (0..255) OPTIONAL,

rssiMean                       INTEGER (0..255) OPTIONAL

}

SAPreambleReport ::= SEQUENCE {

centerFrequency               CenterFreq,

saPreambleIndex               SEQUENCE (SIZE (0..15)) OF SAPreambleReportIndex

}

SAPreambleReportIndex ::= SEQUENCE {

saPreambleIndex               PreambleIndex,

cinrMean                       INTEGER (0..255) OPTIONAL,

rssiMean                       INTEGER (0..255) OPTIONAL

}

AAI-SCN-REP ::= SEQUENCE {

reportMode                     ENUMERATED {

eventTriggerReport,

periodicReport,

oneTimeReport

},

sabsCarrierReportList          CarrierReportList,

nbrAdvChangeCount              INTEGER (0..7),

absReportIndication             CHOICE {

nbrBitmapReport               NbrBitmapReport,

nbrABSReportList              SEQUENCE (SIZE (1..63)) OF NbrIndexReport,

absFullReportList             SEQUENCE (SIZE (1..63)) OF ABSReport

},

nbrLegacyFullReportList        SEQUENCE (SIZE (0..63)) OF NbrLegacyFullReport,

saPreambleReportList           SEQUENCE (SIZE (0..15)) OF SAPreambleReport,

neighborRequest                SEQUENCE {

requestBSType                BIT STRING {

csgClosedFemto (0),

csgOpenFemto (1),

osgFemto (2)

} OPTIONAL,

csgIDList                    SEQUENCE (SIZE (0..63)) OF CSGID

} OPTIONAL

}

------------------------------------------ Propose Text Ends ---------------------------------------------------