| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Coverage loss recovery info in AAI-RNG-REQ (16.2.3) |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Xiangying Yang Intel Corporation | xiangying.yang@intel.com |
| Re: | Sponsor ballot comment on P802.16m/D10 Target topic: “IEEE P802.16m/D10, section 16.2.3”. |  |
| Abstract | The contribution proposes a clarification of coverage loss operation |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Coverage loss recovery info in AAI-RNG-REQ (16.2.3)

Xiangying Yang

Intel Corporation

- Introduction

The contribution proposes clarifies coverage loss operation. The text change allows context retrieval via R8 as long as previous serving BS still maintains that context. The change also allows most coverage loss operation to be implemented in the legacy network mode (Network configuration =0b1 in S-SFH SP2).

In the last meeting the change of “If the Resource Retain Time maintained at the AMS does not expire, the AMS shall include its previous serving ABS ID and its previous STID in AAI-RNG-REQ.” is agreed but the associated message table and ASN.1 is not updated.

Reference

[1] IEEE P802.16m/D10, Nov. 2010

- Text change

------------------------------------------ Text Start ---------------------------------------------------

Remedy #1

[Modify line 48 of page 92 as following:]

16.2.3.1 AAI-RNG-REQ

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| } else if (Ranging Purpose Indication == 0b1000) { |  | // Reentry from DCR mode, coverage loss or detection of different ABS restart count. |  |
| CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| Serving BSID | 48 | The BSID of the AMS's previous S-ABS before incurring a coverage loss | Presented when MS recovers from coverage loss and Resource Retain Time maintained at MS is not expired. |
| Previous STID | 12 | The STID which the AMS uses in the previous S-ABS. | Presented when MS recovers from coverage loss and Resource Retain Time maintained at MS is not expired. |
| If (CMAC indicator == 0b1){ |  |  |  |
| … | … | … | … |

Remedy #2

[Modify ASN.1 as following:]

-- ReentryFromDCR is used in the following places:

--    AAI-RNG-REQ

ReentryFromDCR ::=          SEQUENCE {

crid                        CRID,

stidInfo                    SEQUENCE {

previousServingBsid         BSID,

previousSTID                STID

} OPTIONAL,

akCount                     AKCount                     OPTIONAL,

fidList                     SEQUENCE (SIZE (1..15)) OF FidInfo OPTIONAL,

...

}

------------------------------------------------ Text End ---------------------------------------------------