| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to fix errors in AAI-RNG-REQ and AAI-REQ-RSP messages |  |
| Date Submitted | 2011-01-12 |  |
| Source(s) | Xiangying Yang Joey Chou Intel Corporation Erik Colban Huawei | xiangying.yang@intel.com ecolban@huawei.com |
| Re: | Sponsor ballot comment on P802.16m/D10 Target topic: “IEEE P802.16m/D10, section 16.2.3”. |  |
| Abstract | The contribution proposes Changes to fix errors in AAI-RNG-REQ and AAI-REQ-RSP messages |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes to fix errors in AAI-RNG-REQ and AAI-REQ-RSP messages

Xiangying Yang, Joey Chou

Intel Corporation

- Introduction

The contribution proposes clarifies coverage loss operation. The text change allows context retrieval via R8 as long as previous serving BS still maintains that context. The change also allows most coverage loss operation to be implemented in the legacy network mode (Network configuration =0b1 in S-SFH SP2).

In the last meeting the change of “If the Resource Retain Time maintained at the AMS does not expire, the AMS shall include its previous serving ABS ID and its previous STID in AAI-RNG-REQ.” is agreed but the associated message table and ASN.1 is not updated.

This contribution also proposes some cleanup related to “Resource Retain Time”, as follows. We need to distinguish the ABS timer from signaled parameters that are used to initialize this timer.

This contribution also contains changes to fix AAI-RNG-RSP message.

Reference

[1] IEEE P802.16m/D10, Nov. 2010

- Text change

2.1  Remedy #1

The change is needed to allow most coverage loss operation to be implemented in the legacy network mode (Network configuration =0b1 in S-SFH SP2)

[Modify line 39 of page 92 as following:]

16.2.3.1 AAI-RNG-REQ

------------------------------------------ Text Start ---------------------------------------------------

Table 678.—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| } else if (Ranging Purpose Indication == 0b1000) { |  | // Reentry from DCR mode, coverage loss or detection of different ABS restart count. |  |
| CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| Serving BSID | 48 | The BSID of the AMS's previous S-ABS before incurring a coverage loss | Present when the AMS is recovering from coverage loss unless the AMS can determine that the ABS-Resource-Retain-Timer maintained by the previous S-ABS has expired. |
| Previous STID | 12 | The STID which the AMS uses in the previous S-ABS. | Present when the AMS is recovering from coverage loss unless the AMS can determine that the ABS-Resource-Retain-Timer maintained by the previous S-ABS has expired. |
| If (CMAC indicator == 0b1){ |  |  |  |
| … | … | … | … |

------------------------------------------ Text End ---------------------------------------------------

2.2  Remedy #2

The remedy is intended to fix errors in the table.

[Modify line 49 of page 95 as following:]

16.2.3.2 AAI-RNG-RSP

------------------------------------------ Text Start ---------------------------------------------------

Table 679.—AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Abort | 1 | Set to 1 when an ABS rejects the AMS | Presented when an ABS rejects an AMS |
| If (Ranging Abort == 1) { | - |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65534, In units of seconds Value: 65535 (When the received CSGID(s) from the AMS does not match any of the CSGID(s) of the Femto ABS. This value indicates the Ranging Abort Timer is not to be used, and the AMS can range any time.) |  |
| }else{ |  |  |  |
| If (Temporary STID) { |  |  |  |
| Temporary STID | 12 | Used for AMS identification until STID is assigned to the AMS during registration procedure. | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected and when the AMS has not been assigned its STID/DID. |
| } |  |  |  |
| Else { |  |  |  |
| STID | 12 | AMS identification to be used in the T-ABS | Shall be included for uncontrolled HO, NW reentry or Zone switching in case this message is encrypted |
| } |  |  |  |
| MAPMask Seed | 15 | This parameter shall be used at the target ABS to initiate the PRBS generator used to scramble the 40-bit A-A-MAP IE when the STID is used to mask the CRC. See 16.3.5.3.2.4. | Shall be present if STID or TSTID is present |
| If (advanced network mode) { |  |  |  |
| AMSID* | 48 |  | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected when the AMS has not been assigned its STID. |
| } else { // legacy network mode |  |  |  |
| AMS MAC address | 48 |  | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected when the AMS has not been assigned its STID. |
| } |  |  |  |
| STID | 12 | AMS identification to be used in the T-ABS | Shall be included for uncontrolled HO, NW reentry or Zone switching in case this message is encrypted |
| MAPMask Seed | 15 | This parameter shall be used at the target ABS to initiate the PRBS generator used to scramble the 40-bit A-A-MAP IE when the STID is used to mask the CRC. See 16.3.5.3.2.4. | Shall be present if STID or TSTID is present |
| CRID | 72 | The AMS identifier newly assigned by the network for coverage loss or DCR mode | Shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/zone-switch with assigning a new CRID in response to AAI-RNG-REQ or it shall be included when the network assigns a new CRID unsolicitedly to the AMS. |
| Emergency Service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | Shall be included when this message is sent in response to an AAI-RNG-REQ with Ranging Purpose Indication set to code 0b0101 |
| NS/EP service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | Shall be included when this message is sent in response to an AAI-RNG-REQ with Ranging Purpose Indication set to code 0b1101 |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x6~0xF: Reserved | Shall be included when this message is sent in response to an AAI- RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| If (Location Update Response == 0x0 ) { | - |  |  |
| Paging Group ID update | 32 | Old PG ID, New PG ID The Paging Group ID update specifies a new PGID that replaces an old PGID. | Shall be included when the Paging parameters are changed. |
| Paging Offset update | 24 | Old Paging Offset, New Paging Offset The Paging Offset update specifies a new Paging Offset that replaces an old Paging Offset. | Shall be included when the Paging parameters are changed. |
| New Paging Cycle | 4 | New Paging Cycle to be assigned to the AMS 0b0000-0b1111 | Shall be included when the new Paging parameters are assigned. |
| New Paging Group ID | 16 | New PGID of the AMS | Shall be included when the new Paging parameters are assigned. |
| New Paging Offset | 12 | New paging offset of AMS which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. | Shall be included when the new Paging parameters are assigned. |
| Deregistration Identifier (DID) | 18 | The new DID which the AMS shall maintain in idle mode. | Shall be included only in advanced network mode when DID is changed. |
| New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| For (i=0; i< M; i++) { | - | Number of Multicast Group ID and FID (M) to update in the T-ABS[1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| SMS | Variable | Short Message content up to the size of 140 bytes | May be included when SMS contents is sent in idle mode. |
| }//end of If (Location Update Response == 0x0 ) | - |  |  |
| Reentry Process Optimization | 5 | Reentry process optimization bitmap indicates which MAC control message transactions may be omitted during an attempted reentry (i.e. reentry during HO (including zone switching), and reentry from idle mode) A value of 1 in the bitmap indicates that the corresponding MAC control message transaction may be omitted, while a 0 indicates that the corresponding MAC control message transaction shall be completed. The AMS shall only commence Connected State with the T-ABS after completing all the required MAC control message transactions. Bit #0: Omit AAI-SBC-REQ and AAI-SBC-RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI-REG-REQ and AAI-REG-RSP message during reentry processing Bit #3: Omit higher layer protocol triggering for IP address refresh during reentry processing Bit #4: For the case of reentry during HO including zone switching, a 1 indicates to the AMS that the T-ABS has received the full service and operational states for static and dynamic context (including ARQ window parameters and state machines). For the case of reentry from Idle mode, a 1 indicates to the AMS that the T-ABS has received the static context of the AMS. The static context includesSFIDs and related description (QoS descriptors and CS classifier information) for all service flows that the AMS has currently established as well as any SAs with their related keying information | It shall be included when the AMS is attempting to perform network reentry (i.e. reentry during HO (including zone switching), and reentry from idle mode), and the T-ABS wishes to identify which MAC control message transactions may be omitted. |
| Activation Deadline | 6 | LSB bits of Superframe number after the AAI-RNG-RSP message to confirm the activation of pre-assigned secondary carrier by sending the AAI-CM-IND message. | Shall be included when the T-ABS has pre-assigned the secondary carriers through AAI-HO-CMD. |
| Neighbor station measurement report indicator | 1 | 1: Perform Neighbor station measurement report. | Optional. Identifies Neighbor station measurement report is required during current network entry for ARS. |
| Resource_Retain_Time | 8 | The default duration in units of 100 ms to which the ABS initializes the ABS Resource Retain Timer. time when the S-ABS discards AMS's context | May be included if Reentry Process Optimization Bit #3 is set to 1. |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs to be included here. The maximum number of N_FIDs are 24. | Shall be included if the ABS needs to update the AMS's existing flows. FIDs that are not presented here shall be regarded as guaranteed by the ABS. |
| SFID | 32 | SFID of the existing flow to be updated |  |
| FID | 4 |  |  |
| Update or delete | 1 | 0: delete the flow 1: updated QoS Info exists |  |
| Updated QoS Info | Variable | Following parameters shall be included if the parameter needs to be updated. The parameter which does not have to be updated is not included in this field. - Traffic Priority parameter - Maximum Sustained Traffic Rate parameter - Maximum Traffic Burst parameter - Minimum Reserved Traffic Rate parameter - Maximum Latency parameter - Paging Preference parameter - Tolerated Jitter parameter - UL scheduling type - Unsolicited Grant Interval parameter - Unsolicited Polling Interval parameter - Primary Grant Polling Interval parameter - Primary Grant Size parameter - Secondary Grant Polling Interval parameter - Secondary Grant Size parameter - Adaptation Method - Access Class - Differentiated BR Timer - Predefined BR index - BR action - BR size - Initial Backoff Window Size - Maximum Backoff Window Size - Backoff Scaling Factor - MAC in-order delivery indicator - Vendor ID | Present if needed |
| ROHC support | 1 | 0: not support 1: support | Present if needed |
| PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed |
| } |  |  |  |
| Unsolicited bandwidth grant indicator | 1 | 1: indicates an unsolicited BW grant will be available for transmission of BR header without request from AMS during network entry | Shall be included when AMS is attempting network entry |
| CLC Response | Variable | The information of co-located coexistence response (as defined in Table 695) | Shall be included when the AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before handover. |
| for (i=0; i<N_CSG_IDs; i++) { |  | N_CSG_IDs is the number of CSGIDs belong to the Femto ABS |  |
| CSGID | Variable | CSGID of the Femto ABS sending the AAI-RNG-RSP | Optional. If included, to help the AMS make the decision if the Femto ABS is accessible or not. |
| } |  |  |  |
| For (i=0; i<N_Redirect_Info; i++) { |  | N_Redirect_Info is the number of redirection information included here. ABSID, preamble index and center frequency for one or more neighbor ABSs of the S-ABS N_Redirection_Info: 1..8 | Sent by S-ABS to aid cell reselection in case the S-ABS does not allow the AMS to perform entry (due to various reasons such as high S-ABS load, non existence of CSG membership etc) |
| ABSID for neighbor ABS | 48 |  |  |
| Preamble index for neighbor ABS | 10 |  |  |
| Center frequency for neighbor ABS | 18 | center frequency (in unit of 50 kHz) |  |
| } |  |  |  |
| Ranging Request bit | 1 | 1 : AMS shall send the AAI-RNG-CFM message after a successful periodic ranging | Shall be included when S-ABS performs coverage loss detection procedure (16.2.26) |
| For (i=0; i<N_Invalid_FIDs; i++) { |  | N_Invalid_FIDs is the number of invalid FIDs listed here. The maximum number of N_Invalid_FIDs are 24. |  |
| FID | 4 |  | Shall be included if there is any invalid FID. Invalid FID is a FID whose FID change count in AAI-RNG-REQ is different from FID change count stored at the ABS. ABS and AMS may exchange the AAI-DSx-messages for invalid FIDs |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| SAID update bitmap | 16 | Bitmap for indicating the specific FID(s) that are being updated to SAID of AES-CTR | Shall be included if specific FID(s) are to be remapped to SAID of AES-CTR in case of Zone Switch from LZone to MZone |
| If(it is under network re-entry for HO){ |  |  |  |
| For (i=0; i< M; i++) { | - | Number of Multicast Group ID and FID (M) to update in the T-ABS[1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| }//end of If (it is under network re-entry for HO) | - |  |  |
| N_SFIDs | 5 | Number of SFIDs [1..24] supported in MZone when an AMS performs Zone Switching from LZone to MZone | Present if CID to FID mapping is done through the AAI-RNG-RSP message during Zone Switching operation. If this field is not present, all FIDs for the transport connection should be reestablished through the AAI-DSA exchanges after completion of network reentry in MZone. |
| For (i=0; i<N_SFIDs; i++) { |  |  |  |
| SFID | 32 | FID in MZone should be assigned as defined in section 16.2.6.4.1.3.1 per each DL/UL connections. |  |
| } |  |  |  |
| } // End of Else (Ranging Abort == 1) { |  |  |  |

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #3

Update the Notes column for the Resource Retain Time field in the AAI-REG-RSP message.

[Modify Line 5 at Page 124, as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

| Resource_Retain_Time | 16 | The default duration in units of 100 ms to which the ABS initializes the ABS-Resource-Retain-Timer. 0-65535: In units of 100 milliseconds Duration that the S-ABS shall retain the AMS context | Present if needed. |
| --- | --- | --- | --- |

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #4

Update the Notes column for the Resource Retain Time field in the AAI-HO-CMD message.

[Modify Line 32 at Page 129, Line 25 on page 133,  as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

| Resource_Retain_Time | 8 | The duration in units of 100 ms to which the T-ABS initializes the ABS-Resource-Retain-Timer. The time when the serving BS discards MS's context | Present if needed. Optional. If not included, the value included in the AAI-RNG-RSP or AAI-REG-RSP previously received at the S-ABS shall be used. |
| --- | --- | --- | --- |

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #5

Update the behavior related to maintaining the AMS’s resources in a mode 1 handover.

[Modify Line 23 on Page 322 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

The S-ABS shall retain discards theis AMS's key context (along with other MAC context associated with the AMS) upon until either the expiration of the ABS_Resource_Retain_Timer has expired or the AMS has completed network re-entry at the T-ABSa HO-Complete signaling from T-ABS via backbone.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #6

Update the behavior related to maintaining the AMS’s resources in a mode 0 handover.

[Modify Line 15 on Page 361 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------If HO_Reentry_Mode is set to 0, at the Disconnect Time, the S-ABS shall start the Resource Retain Timer

counting down from value Resource_Retain_Time provided by ABS in AAI-REG-RSP or AAI-HO-CMD

messages.If HO_Reentry_Mode is set to 0, the S-ABS shall start the ABS_Resource_Retain_Timer at Disconnect_Time.

The S-ABS shall use the default Resource_Retain_Time value sent to the AMS theindicated in the AAI-REG-RSP or AAI-RNG-RSP message is used to initialize the ABS_Resource_Retain_Timer unless the AAI-HO-CMD message includes the provides Resource_Retain_Time field, in which case it shall use the value sent in the . If AAI-HO-CMD includes Resource_Retain_Time, the value included in AAI-HO-CMD messageshall be used instead of the value included in AAI-REG-RSP. The S-ABS shall retain the context of the AMS including connections, states of MAC state machine, and untransmitted/unacknowledged data associated with the AMS for service continuation until the expiration of the ABS_Resource_Retain_Timer or the AMS has completed network re-entry or entry at another BS or ABS.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #7

Update the behavior related to maintaining the AMS’s resources in network re-entry.

[Modify Line 54 on Page 366 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

In the case of an uncoordinated handover, where AMS performs the contention based CDMA HO ranging at the T-ABS, the AAI-RNG-REQ message shall include the former previous serving BSID and previously used STID if the unless the AMS can determine that the ABS_Resource_Retain_Timer is not has expired. When ABS receives the AAI-RNG-REQ message and verifies tehthe CMAC, the ABS shall respond to the AAI-RNG-REQ message by transmitting an encrypted AAI-RNG-RSP message. The AAI-RNG-RSP message shall include a STID for the AMS if the STID is has not been pre-assigned by the AAI-HO-CMD message.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #8

Update the behavior related to maintaining the AMS’s resources and HO cancellation.

[Modify Line 23 on Page 367 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

After HO is initiated, the AMS may cancel the handover could be canceled by an AMS at any phase during the HO procedure. The

AMS may initiate HO cancellation is initiated before the expiration of the only when it can determine that the ABS_Resource_Retain_Timer has not expired. An AMS requests HO cancellation to the S-ABS by sending the AAI-HO-IND with HO Event Code 0b11 (HO cancel) with its current AK_COUNT after Disconnect Time. The S-ABS shall explicitly acknowledge to the HO cancellation message upon receiving it through the AAI-MSG-ACK message. The AAI-MSG-ACK message is triggered by

setting Polling bit to 1 in MCEH of the AAI-HO-IND message. After the HO cancellation is processed, the

ABS resumes communication with the AMS..

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #9

Update the behavior related to maintaining the AMS’s resources and zone switching.

[Modify Line 61 on Page 371 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

With zone switch based handover, the context management process from serving R1 BS to the LZone of the T-ABS follows is specified in section 6.3.21.2.8.1. The This following section and its sub-sections describes the context mapping from the LZone to the MZone of the T-ABS during network reentry procedure in the MZone in the case of zone switch based handover.

The ARQ context and state machine associated with serving R1 BS or LZone shall not be used in MZone and shall may be discarded upon expiration of the ABS_Resource_Retain_Time Timer or completion of network reentry completion into MZone whichever occurs first in both direct handover and zone switch cases.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #10

Update the behavior related to maintaining the AMS’s resources and Sleep Mode termination.

[Modify Line 49 on Page 489 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

16.2.17.3 Sleep Mode termination

Sleep Mode termination can be initiated by either the AMS or the ABS. If AMS-initiated, then the AMS shall send an AAI-SLP-REQ message with de-activation request and subsequently the ABS shall respond with the AAI-SLP-RSP message. The ABS may also send an unsolicited AAI-SLP-RSP message to de-activate Sleep Mode. In this case, the ABS shall request the AMS to send acknowledgment using AAI-MSG-ACK or MAEH to the AAI-SLP-RSP. When the AMS successfully exits from Sleep Mode, the AMS and ABS shall keep sleep mode context until the ABS_Resource_Retain_Timer expires and the AMS shall keep the Sleep Mode context until it can determine that the ABS_Resource_Retain_Timer has expired. Sleep Mode shall be implicitly terminated when an AMS successfully achieves idle mode, handover, or scanning mode transaction by explicit signaling.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #11

Update the behavior related to maintaining the AMS’s resources and coverage loss.

[Modify Line 49 on Page 524 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

The ABS shall send an unsolicited AAI-RNG-RSP message with the Ranging Request bit set to one to request the AMS to perform ranging using periodic ranging codes by setting the Ranging Request bit to one and . The ABS shall then start the T58 timer. Upon receiving an AAI-RNG-CFM message with the AMS’s STID, which indicates a successful periodic ranging initiated by this unsolicited AAI-RNG-RSP, the ABS shall restart the active_ABS_timer. If the ABS does not receive the AAI-RNG-CFM message upon the expiration of T58, the ABS shall start the ABS_Resource Retain Timer.

Once the ABS_Resource_Retain_Timer is started, the ABS shall not restart the active-ABS-timeractive_ABS_timer.

Upon expiration of the ABS_Resource_Retain_Timer, the ABS considers that the AMS is not in the network anymore, and may releases the AMS’s dynamic context and moves AMS’s static context to the network entity that stores AMS’s context.

If the ABS receives backbone context request for the AMS during coverage loss detection or before expiration of Resource Retain Time, the ABS considers that the AMS is performing network reentry due to link loss in AMS. The ABS in this case, shall provide the AMS’s context to the network, in response to the backbone context request.

------------------------------------------ Text End ---------------------------------------------------

[Modify Line 35 on Page 525 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

Whenever an AMS declares coverage loss, the AMS shall start the Resource Retain Time.

------------------------------------------ Text End ---------------------------------------------------

[Modify Line 54 on Page 525 as follows:]

------------------------------------------------ Text Start ---------------------------------------------------

In addition, if unless the Resource Retain time maintained at the AMS can determine that the ABS_Resource_Retain_Timer has does not expired, the AMS shall include its previous serving ABS ID and its previous STID in the AAI-RNG-REQ message.

------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #12

The ASN.1 code change is needed to allow most coverage loss operation to be implemented in the legacy network mode

[Modify Line 25 at Page 1001, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

-- ReentryFromDCR is used in the following places:

--    AAI-RNG-REQ

ReentryFromDCR ::=          SEQUENCE {

crid                        CRID,

stidInfo                    SEQUENCE {

previousServingBsid         BSID,

previousSTID                STID

} OPTIONAL,

akCount                     AKCount                     OPTIONAL,

fidList                     SEQUENCE (SIZE (1..15)) OF FidInfo OPTIONAL,

...

}

------------------------------------------------ Text End ---------------------------------------------------

2.3  Remedy #13

According to definition of CSGID in Table 678, the maximum length of CSGID is 24.

.

[Modify Line 44 at Page 996, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

CSGID ::= BIT STRING (SIZE (1..24, ...))

------------------------------------------------ Text End ---------------------------------------------------

2.4  Remedy #14

embsZoneInfo has been removed in D10 draft.

.

[Modify Line 52 at Page 998, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

SuccessOfLocationUpdate ::= SEQUENCE {

paginggroupidupdate BIT STRING (SIZE (32)) OPTIONAL,

pagingoffsetupdate BIT STRING (SIZE (24)) OPTIONAL,

newPagingCycle PgCycle OPTIONAL,

newPagingGroupID PGID OPTIONAL,

newPagingOffset PgOffset OPTIONAL,

deregistrationID DID OPTIONAL,

newPagingControllerID PCID OPTIONAL,

embsZoneInfo SEQUENCE (SIZE (1..8)) OF EMBSZoneInfoItem OPTIONAL,

multicastInfo SEQUENCE (SIZE (1..16)) OF SEQUENCE {

currentMulticastGroupID BIT STRING (SIZE (12)),

currentFID BIT STRING (SIZE (4)),

newMulticastGroupID BIT STRING (SIZE (12)),

newFID BIT STRING (SIZE (4))

} OPTIONAL,

smsMessage SMS OPTIONAL

}

------------------------------------------------ Text End ---------------------------------------------------

2.5  Remedy #15

embsZoneInfoList has been removed in D10 draft.

.

[Modify Line 24 at Page 999, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

RngRspForHoReentryInfo ::= SEQUENCE {

embsZoneInfoList SEQUENCE (SIZE (0..7)) OF EMBSZoneInfoInHandover,

multicastInfo SEQUENCE (SIZE (1..1516)) OF SEQUENCE {

currentMulticastGroupID BIT STRING (SIZE (12)),

currentFID BIT STRING (SIZE (4)),

newMulticastGroupID BIT STRING (SIZE (12)),

newFID BIT STRING (SIZE (4))

} OPTIONAL

}

------------------------------------------------ Text End ---------------------------------------------------

2.6  Remedy #16

The size of Paging Carrier update should be 6 bits, as per Table 678.

.

[Modify Line 49 at Page 1000, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

LocationUpdate ::=          SEQUENCE {

addressOrDID                AddressOrDID,

pagingControlInfo           PagingControlInfo,

pagingCycleChange           PgCycle                     OPTIONAL,

pagingCarrierUpdate         INTEGER (0..153163)           OPTIONAL,

akCount                     AKCount                     OPTIONAL,

amsMobility                 ENUMERATED {

slow,

medium,

fast,

null

}                           OPTIONAL,

smsMessage                  SMS                         OPTIONAL,

...

}

------------------------------------------------ Text End ---------------------------------------------------

2.7  Remedy #17

The ASN.1 code of ZoneSwitch is chnaged to aligne with Table 678.

.

[Modify Line 48 at Page 1001, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

ZoneSwitch ::=              SEQUENCE {

-- be selected for advanced network mode and AMSID privacy is enabled

zoneSwitchInfo              CHOICE {

receiveTSTID              SEQUENCE {

tstid                   STID,

amsidStarHashValue      MACAddress                  OPTIONAL,

},

receiveBsid               SEQUENCE {

servingBsid             BSID,

previousBasicCid        CID

}

}

akCount                     AKCount                     OPTIONAL,

...

}

------------------------------------------------ Text End ---------------------------------------------------

2.8  Remedy #18

The ASN.1 code of RangingResponse is changed to aligne with Table 678.

.

[Modify Line 48 at Page 1001, as following:]

------------------------------------------------ Text Start ---------------------------------------------------

RangingResponse ::=         SEQUENCE {

tempStidAndMAPMaskSeed SEQUENCE {

tempStidOrSTID          CHOICE {

temporarySTID         STID,

stid                  STID

mapMaskSeed BIT STRING (SIZE (15))

} OPTIONAL,

mapMaskSeed                 BIT STRING (SIZE (15)),

amsidOrMacAddress           CHOICE {

-- selected for advanced network mode and AMSID privacy is enabled

amsidStarHashValue          MACAddress,

-- selected for other cases

macAddress                  MACAddress

},

stid                        STID                        OPTIONAL,

mapMaskSeed                 BIT STRING (SIZE (15))      OPTIONAL,

crid                        CRID                        OPTIONAL,

-- response based on ringing purpose sent in AAI-RNG-REQ

ringingPurpose              CHOICE {

emergencyCallSetup          SEQUENCE {

emergencyServiceFID         INTEGER (2..15)

},

nsEpCallSetup               SEQUENCE {

nsEPServiceFID              INTEGER (2..15)

},

locationUpdatePowerDown     LocationUpdateResponse,

locationUpdateEmbsFlows     LocationUpdateResponse,

idleModeLocationUpdate      LocationUpdateResponse,

locationUpdateToDcrMode     LocationUpdateResponse,

dcrModeExtension            LocationUpdateResponse

},

-- bitmap for Reentry Process Optimization

reentryProcessOptimization  ReentryProOptimization      OPTIONAL,

activationDeadline          INTEGER (0..63)             OPTIONAL,

-- 1: perform neighbor station measurement report

nbrBsMeasurementRptIndicator BOOLEAN                    OPTIONAL,

resourceRetainTime          INTEGER (0..255)            OPTIONAL,

flowUpdating                SEQUENCE (SIZE (1..24)) OF SEQUENCE {

sfid                        BIT STRING (SIZE (32)),

updateOrDelete              BOOLEAN,

dlULIndicator               ENUMERATED {

dl,

ul

},

updatedQoSInfo              QosParameter                OPTIONAL,

rohc                        FeatureSupport              OPTIONAL,

phs                         INTEGER {

packetPhs                   (1)

} (0..1)                    OPTIONAL

} OPTIONAL,

unsolicitedBsGrantIndicator BOOLEAN                     OPTIONAL,

clcResponse                 CLCResponse                 OPTIONAL,

csgIdList                   SEQUENCE (SIZE (1..64, ...)) OF CSGID OPTIONAL,

nbrAbsRedirectInfoList      SEQUENCE (SIZE (1..8)) OF RedirectionInfo OPTIONAL,

rangingRequestBit           BOOLEAN                     OPTIONAL,

invalidFIDList              SEQUENCE (SIZE (1..24)) OF SEQUENCE {

FID,

dlULIndicator               ENUMERATED {

dl,

ul

}

} OPTIONAL,

saidUpdateBitMap            BIT STRING (SIZE (16))      OPTIONAL,

rngRspForHandoverReentry    RngRspForHoReentryInfo      OPTIONAL,

mzoneSfidList          SEQUENCE (SIZE (1..24)) OF BIT STRING (SIZE (32)),

...

}

------------------------------------------------ Text End ---------------------------------------------------