| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for network configuration (16.2.3) |  |
| Date Submitted | 2011-01-10 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.3 (MAC control message) |  |
| Abstract | This contribution proposes clean-up for the Section 16.2.3 of IEEE P802.16m/D10 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for network related terminology (16.2.3)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

YoungKyo Baek

Samsung Electronics

- Introduction

Instead of saying "advanced/legacy network mode" or “Network Configuration indicates ABS is attached to the advanced/legacy network”, it would be better to say “Network Configuration bit = 0b0/0b1” to keep consistence in the standard.

- Proposed Text for IEEE P802.16m/D10

============================ Start of Proposed Text ==================================

Suggested change #1:

[Modify table 678 in page 89, line 13 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| CMAC indicator | 1 | Indicate whether this message is protected by CMAC tuple 0b0: not protected 0b1: protected | Shall always be present |
| If (Ranging Purpose Indication == 0b0000) { |  | // Initial network entry |  |
| If (advanced network mode S-SFH Network Configuration bit == 0b0 and AMSID privacy is enabled) { |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to Table 16.2.5.3.1 |  |
| } else if (legacy network mode S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address |  |
| } |  |  |  |

Suggested change #2:

[Modify table 678 in page 89, line 51 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| if (legacy network mode S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only |
| } else{ |  |  |  |
| STID | 12 | The Station ID pre-assigned by the TABS |  |
| } |  |  |  |
| } |  |  |  |

Suggested change #3:

[Modify table 678 in page 90, line 27 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| if (legacy network mode S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only or in case that AMSID privacy is disabled |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 18 | The ID which the AMS is assigned for idle mode and currently maintains. | Presented in advanced network mode only. |
| } |  |  |  |

Suggested change #4:

[Modify table 678 in page 91, line 18 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| if (legacy network mode S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only or in case that AMSID privacy is disabled |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 18 | The ID which the AMS is assigned for idle mode and currently maintains. | Presented in advanced network mode only. |
| } |  |  |  |

Suggested change #5:

[Modify table 678 in page 93, line 10 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| if (advanced network mode S-SFH Network Configuration bit == 0b0 and AMSID privacy is enabled){ |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to Table 16.2.5.3.1 |  |
| } |  |  |  |

Suggested change #6:

[Modify table 679 in page 96, line 32 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| if (advanced network mode S-SFH Network Configuration bit == 0b0){ |  |  |  |
| AMSID* | 48 |  | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected when the AMS has not been assigned its STID. |
| } else { // legacy network mode |  |  |  |
| AMS MAC address | 48 |  | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected when the AMS has not been assigned its STID. |
| } |  |  |  |

Suggested change #7:

[Modify table 679 in page 98, line 19 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Deregistration Identifier (DID) | 18 | The new DID which the AMS shall maintain in idle mode. | Shall be included only in advanced network mode when the S-SFH Network Configuration bit == 0b0 and DID is changed. |

Suggested change #8:

[Modify table 699 in page 166, line 33 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Deregistration ID | 18 | Used to indicate Deregistration ID used to identify the AMS in idle mode 0..218-1 | Present when the S-SFH Network Configuration indicates ABS is attached to the advanced network bit == 0b0 |

Suggested change #9:

[Modify table 699 in page 168, line 21 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Deregistration ID | 18 | Used to indicate Deregistration ID used to identify the AMS in idle mode 0..218-1 | Present when the S-SFH Network Configuration indicates ABS is attached to the advanced network bit == 0b0 |

Suggested change #10:

[Modify table 700 in page 169, line 34 with the following changes:]

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Deregistration ID | 18 | Used to indicate Deregistration ID for the AMS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218-1 | Present if the S-SFH Network Configuration indicates ABS is attached to the advanced network bit == 0b0 |
| MAC Address Hash | 24 | used to identify the AMS to be paged | Present if the S-SFH Network Configuration indicates ABS is attached to the legacy network bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved | Present if the S-SFH Network Configuration indicates ABS is attached to the advanced network bit == 0b0 |

Suggested change #11:

[Adopt the following changes in page 314, line 61:]

…, when ABS is operating in mixed mode or when the Network Configuration bit in the S-SFH is set to 0b1 ABS is in legacy ASN support mode. In particular, when the Network Configuration bit in the S-SFH is set to 0b1 in legacy ASN support mode, CMAC_KEY_COUNTB (AK_COUNTB) and CMAC_KEY_COUNTN are maintained in the network as defined in WirelessMAN OFDMA reference system.

Suggested change #12:

[Adopt the following changes in page 497, line 62:]

If the Network Configuration bit in the S-SFH is set to 0b1 In legacy ASN support mode operation, the ABS receives the 16e paging parameters (paging cycle and paging offset) for an idle mode AMS from paging controller.

Suggested change #13:

[Adopt the following changes in page 498, line 55:]

If the Network Configuration bit in the S-SFH is set to 0b1ABS is attached to the legacy network, the AMS MAC Address Hash as used in the legacy network shall be used for paging the AMS.

Suggested change #14:

[Adopt the following changes in page 500, line 13:]

When the Network Configuration bit in the S-SFH is set to 0b1ABS is connected to the legacy network elements, AMS MAC address replaces AMS deregistration identifier in the formula above.

Suggested change #15:

[Adopt the table 700 following changes in page 650, line 5:]

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Network Configuration | 1 | Indicates configuration of the ABS network element to which ABS is attached. 0b0: AAI network configuration Advance network mode 0b1: WirelessMAN-OFDMA network configuration Legacy network mode |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D10. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, November 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.