| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Error recovery from false paging indication (16.2.18.2.2) |  |
| Date Submitted | 2010-01-10 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Youngsoo Yuk, and JinSam Kwak LG Electronics | giwon.park@lge.com +82-31-450-1879 |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | The contribution proposes to include the text about ‘error recovery from false alarm paging’ in section 16.2.18.2.2. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Error recovery from false paging indication

Giwon Park, Kiseon Ryu, Youngsoo Yuk and JinSam Kwak

LG Electronics

- Introduction

The DIDs are assigned to idle mode AMS by the paging controllers (PCs). One or more PCs manage each paging group. Thus, when two different PCs assign the DIDs to different idle mode MSs of the same paging group, there is a possibility that they assign the same DID to two different AMSs. This is because the DID assignment of each PC is independent of the other ones. If both the MS that have the same DID also have the same paging cycle and paging offset, then these AMSs have the same identification. This leads to false paging message indication as the paging message for one of these AMSs also results in unwanted paging indication. This results in unwanted paging operation and unnecessary signaling overhead.

- Problem

If the false paging indication was sent to the AMS, AMS terminates idle mode and reenters the network. However, CMAC verification of AMS can be failed and AMS can perform the full network reentry. This is improper paging operation, because the AMS that received the false paging indication shall continuously maintain the idle mode.

Figure 1. False paging indication

- Solution

If the false paging indication was sent to the AMS, AMS terminates idle mode and reenters the network. However, CMAC verification of AMS can be failed and AMS can perform the full network reentry. This is improper paging operation, because the AMS received the false paging indication it shall continuously maintain the idle mode.

When the ABS receives the AAI-RNG-REQ message (action code: reentry from idle mode) from the AMS (false paging indication), it can transmit the AAI-RNG-RSP message to the AMS to maintain the idle mode.

Figure 2. Error recovery from false paging indication

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Add the following text in the line 29 on page532, as follows:]

An AMS shall terminate idle mode and reenter the network if it decodes a paging message that contains the AMS's identification and action code 0b0 (Re-enter Network). When the ABS receives the AAI-RNG-REQ message (i.e., Re-enter Network), it validates the CMAC of the AMS. If the CMAC verification of AMS was failed and AMS was paged wrong (i.e., false paging indication), the ABS shall transmit the AAI-RNG-RSP message including the Maintain Idle Mode parameter which is set to 1 to the AMS. When the AMS decodes the AAI-RNG-RSP message that contains the Maintain Idle Mode parameter which is set to 1, it shall maintain the idle mode without performing the reentry from the idle mode.

[Modify the Table 679 on page 6, as follows:]

| Table 679—AAI-RNG-RSP Message Field Description |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-RNG-RSP message_format() { | — | — |  |
| … | … | … | … |
| Maintain Idle Mode | 1 | Maintain Idle Mode field indicates to the AMS that the recovery from false paging indication shall be performed. If this field is set to 1, AMS shall maintain the idle mode without performing the reentry from the idle mode. | It shall be included when the CMAC verification was failed and AMS was paged wrong. |
| … |  |  |  |
| } |  |  |  |

-------------------------------  Text End  ---------------------------------------------------