| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to D11 – Table 782 |  |
| Date Submitted | 2011-02-10 |  |
| Source(s) | Max Riegel Nokia Siemens Network | E-mail: maximilian.riegel@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: |  |  |
| Abstract | Table 782 mixes up bit- and byte-sizes. The contribution corrects the values to the bit-sizes. |  |
| Purpose | For editorial update |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table 782.—Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| fullNumE-MBSIdArr[1..8] | variable |  |
| A) E-MBS_Zone_ID | 7 | Indicates an E-MBS Zone ID where the connection for associated service flow is valid |
| B) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to |
| C) Number of E-MBS ID_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings in the EMBS_Zone_ID to add |
| D) fullEMBSIdFidMappingArr[0...16] |  |  |
| D.1) E-MBS ID and FID | Variable (16xN) | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings |
| Quick Access Predefined BR Index Parameter | variable | Refer to 16.2.12.12.4 |
| SCID | 4 | Sleep cycle ID |
| SDU Size | 8 | Number of bytes Default=49 |
| Non_ARQ_reordering_timeout | 6 | 0 > and <= 32. Unit is PHY frame (5ms) |
| Classification Rule Priority | 8 | //packet classification rule parameter 0-255 |
| Protocol field | 8 | //packet classification rule parameter Protocol |
| IP Masked Source Address | 64(IPv4) or 256 (IPv6) | //packet classification rule parameter src, smask |
| IP Masked Destination Address | 64(IPv4) or 256 (IPv6) | //packet classification rule parameter dst, dmask |
| Protocol Source Port Range | 32 | //packet classification rule parameter sportlow, sporthigh |
| Protocol Destination Port Range field | 32 | //packet classification rule parameter dportlow, dporthigh |
| Associated PHSI | 8 | //packet classification rule parameter Index value |
| Packet Classification Rule Index | 16 | //packet classification rule parameter Packet Classification Rule Index |
| IPv6 Flow Label field | 20 | //packet classification rule parameter Flow Label |
| Classification Action Rule | 8 | //packet classification rule parameter Bit 0: 0 = none. 1 = Discard packet Bit 1-7: Reserved. |

Table 782.—Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| IP Type of Service | 8 | //packet classification rule parameter Bit 0-Bit 1: reserved. Shall be set to 0b00 Bit 2-Bit 7: DSCP value |
| PHS DSC Action field | 2 | //packet classification rule parameter 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |
| PHSI field | 8 | //packet classification rule parameter Index value |
| PHSS field | 8 | //packet classification rule parameter Number of bytes in the suppression string |
| PHSF field | PHSS e 8 | //packet classification rule parameter String of bytes suppressed |
| PHSM field | PHSS e 8 | //packet classification rule parameter Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |
| PHSV field | 1 | //packet classification rule parameter 0: Verify 1: Don't verify |
| Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier |
| IEEE 802.3/Ethernet Destination MAC Address parameter | 96 | dst.msk |
| IEEE 802.3/Ethernet Source MAC Address parameter | 96 | src, msk |
| Ethertype/IEEE 802.2 SAP | 24 | type, eprot1, eprot2 |
| IEEE 802.1D User Priority field | 16 | pri-low, pri-high Valid range: 0-7 for pri-low and pri-high |
| IEEE 802.1Q VLAN ID field | 16 | vlan_id1, vlan_id2 |
| Number of HARQ Channels | 4 | Indicates the number of HARQ channels which are intended to the flow. |
| fullNumHarqChArr[0..15] |  |  |
| A) HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel. |