9. Frame formats

9.3 Control frame format

9.3.2 Control frame fields

Insert the following value 0C16 information field in Table 9.4, after the 0B16 entry.

| 0C16 | CT_OAM_PIRC_STATUS | OAM PIRC status notify frame |
| --- | --- | --- |

11. Topology discovery and protection

11.3 Frame formats

11.3.5 ATD frame format

Insert the following value 8 information field in Table 11.7, after the 7 entry.

| 8 | ATT_PIRC_SET | 2-100 | 11.4.8 | C | PIRC station settings |
| --- | --- | --- | --- | --- | --- |

11.4 Defined ATT encodings

Insert the following new section 11.4.8 after 11.4.7. Renumber as necessary.

11.4.8 PIRC settings ATT

The PIRC settings ATT is identified by its distinctive type field value of ATT_PIRC_SET (see 11.3.5).The PIRC settings ATT encodes several protection group settings of the PIRC station, as specified in Figure 11.23. The PIRC settings ATT is only meaning to the PIRC stations. Only PIRC station sends the ATD frame with PIRC settings ATT. If there is a PIRC settings ATT field in an ATD frame, the frame should be generated by a PIRC station. And only PIRC station can recognize and process the PIRC settings ATT.

lb

sr

res

1

MSB

LSB

Protection Group ID 1

1

lb

sr

res

1

…

Protection Group ID N

1

Figure 11.23—PIRC settings attDataUnit format

11.4.8.1 Protection Group ID: Identify a protection group. A PIRC station can be a member of N different protection groups. Different protection groups’ settings can be different. In one PIRC settings ATT, There may be multiple protection group ID and corresponding settings.

11.4.8.2 res: A 4-bit reserved field.

11.4.8.3 sr: Two station role bits identifies the role of the PIRC station in the corresponding protection group. In active/standby mode, it identifies the PIRC station is active or standby. In load balancing mode, it identifies the PIRC station forwards VLAN set 1 or VLAN set 2, as specified in Table 11.9. (Methods such as hashing, manual configuration etc. can be use to decide which set a VLAN belongs to.)

Table 11.9—station role field value encoding

| value | Name | description |
| --- | --- | --- |
| 00 | VLAN set 1 | VLAN set 1 is forwarded by this PIRC station, but in active/standby mode, it means standby. |
| 01 | VLAN set 2 | VLAN set2 is forwarded by this PIRC station, but in active/standby mode, it means active. |
| 10-11 | - | Reserved |

11.4.8.4 lb: Three load balancing bits mark if the station is working in load balancing mode in the corresponding protection group, and what load balancing method is used, as specified in Table 11.10.

Table 11.10—load balancing field value encoding

| Value | Name | Description |
| --- | --- | --- |
| 000 | Active/Standby | Load balancing is not used. |
| 001 | VLAN Hashing | Load balancing is used, and VLAN based Hashing method is used to achieve it. |
| 010 | VLAN Configuration | Load balancing is used, and VLAN based configuration method is used to achieve it. |
| 011-111 | - | Reversed. |

When it is received by a PIRC station, if there are protection groups which the PIRC station belongs to, the corresponding information is copied to the pircMateMac, pircMateMode and pircMateRole fields in corresponding group in the topology database (see 11.5).

11.5 Topology database

11.5.1 Topology database structure

11.5.1.1 Logical representation of topology database

Add the following item to the end of the list.

d) Protection Groups Information (see Table 11.12): pircInfo.fieldname

Add the following figure to Figure 11.25

Protection Group[1]

…

Protection Group[N]

Insert the Table 11.12 after Table 11.11

Table 11.12—Topology database for the protection groups: pircInfo

| Variable | Description | Example values |  |  |
| --- | --- | --- | --- | --- |
|  |  | GroupID[1]=01 | GroupID[2]=0A | GroupID[3]=1B |
| ATD-derived information |  |  |  |  |
| pircMode | Load balancing mode | Active/Standby | VLAN Configuration | VLAN Hashing |
| pircRole | Station role in the protection group | 00 | 01 | 00 |
| pircMateMac | Mate station’ s MAC address | 00-10-A4-97-A8-B2 | 00-10-A4-97-A8-AC | 00-10-A4-97-A8-58 |
| pircMateMode | Load balancing mode | Active/Standby | VLAN Configuration | VLAN Hashing |
| pircMateRole | Station role in the protection group | 01 | 00 | 01 |
| PIRC-derived information |  |  |  |  |
| pircStatus | Station’s status | No Request | Protection | No Request |
| pircMateStatus | Mate station’s status | No Request | Failure | No Request |

12. Operations, administration, and maintenance (OAM)

12.1 Overview

Add the following item to the end of the list.

f) Provide a mechanism to exchange and notify status messages between the mate stations on the ring.

12.1.2 OAM functions supported by RPR

Add the following additional OAM frame type supported to the end of the list.

e) PIRC notify—For status exchange and notify between the mate PIRC stations.

Insert the following new section, 12.1.8, after 12.1.7.

12.1.8 PIRC notify operations

A PIRC frame allows one PIRC station to notify its status to its mate PIRC station in the same protection group. The PIRC frame is broadcast over the rings with a service class of subclassA0.

PIRC frames are received by all stations, processed only by the mate PIRC stations in the same protection group. The PIRC station’s status may be changed according to the information received in the PIRC notify frames.

12.3 OAM frame formats

Insert the following section, 12.3.5, after 12.3.4.

12.3.5 PIRC frame

The PIRC frames are identified by the controlType field value of CT_OAM_PIRC_STATUS and a controlVersion value of 0. The PIRC frame is only sent by the PIRC station. The PIRC frame contents include information about the sending station’s status. Figure 12.13 illustrates the PIRC frame payload with the controlDataUnit portion of the frame indicated. The header fields (specified in 9.3, but not illustrated here) are additionally constrained as specified in Table 12.5.

Device ID

Status

Protection Group ID

controlVersion

controlType

4

1

1

1

1

controlDataUnit

Figure 12.13—PIRC frame payload format

12.3.5.1 Protection Group ID: Identify a protection group.

12.3.5.2 Status: The status field is illustrated in the figure12.14, and Table 12.6 shows the encoded status values

status

res

MSB

LSB

Figure 12.14—Status value format

Table 12.5—PIRC frame header field-value restrictions

| Field | Sub-Field | Value | Description |
| --- | --- | --- | --- |
| ttl | - | MAX_Stations | Allow PIRC frames to propagate through all stations. |
| baseControl | sc | CLASS_A0 | Sent as subclassA0. |
|  | fe | FAULT | Not fairness eligible. |
| da | - | FF-FF-FF-FF-FF-FF | Allow PIRC frames to be received by any station. |

Table 12.6—status values

| Value | Status description |
| --- | --- |
| 000 | No Request |
| 001 | Manual Switch |
| 010 | Failure |
| 011 | Protection |
| 100 | Forced Switch |
| 101-111 | Reserved |

12.3.5.3 Device ID: The PIRC station’s device ID. It can be used for management, such as to help decide the station role in a protection group.