P802.17/D0.1, November 2001

Proposed Draft Standard for Resilient Packet Ring Access Method & Physical Layer Specifications

(OAM&P and Layer Management)

Draft 0.3

Comments on this proposal can be directed to the contributing editors:

Costantinos Bassias
Lantern Communications

Italo Busi
Alcatel
Via Trento, 30
20059 Vimercate (MI)
Italy
Phone: +39 039 686 7054
FAX: +39 039 686 3590
Email: italo.busi@alcatel.it

Wai-Chau Hui
Nortel Networks

Angela Faber
Telcordia

Table of contents

1. Introduction	7

1.1 Overview of RPR systems	8

1.2 Overview of OAM for RPR	8

1.3 Structure of the document	9

2. Operation Administration and Maintenance (OAM)	10

2.1 Generic OAM Frame Format	10

2.2 RPR Ping function	10

2.2.1 Ping Request OAM RPR Format	11

Ping Reply OAM message	12

3. Layer Management	13

3.1 Overview of the management model	13

3.2 Generic management primitives	13

3.3 MLME SAP interface	14

3.3.1 RPR interface configuration	15

3.3.2 Topology discovery monitoring	15

3.3.3 Protection switching	15

3.3.3.1 MLME-SWITCHTOPROTECTION.request	15

3.3.3.2 MLME- SWITCHTOPROTECTION.confirm	16

3.3.4 Performance and Accounting Measurements	17

3.3.4.1 Topology Discovery statistics	17

3.3.5 Notifications and Fault Management	17

3.3.6 RPR Ping Management	18

3.3.6.1 MLME-PING.request	18

3.3.6.2 MLME- PING.confirm	18

3.4 Link Aggregation	19

3.5 PLME SAP interface	19

3.5.1 The Ethernet PHY	19

3.5.2 The SONET PHY	20

3.5.2.1 The GFP Adaptation Layer	20

Annex 3A RPR SNMP MIB Definition	21

Abbreviations

MAC	Medium Access Control

MAU	Medium Attachment Unit

PHY	Physical Interface

SAP	Service Access Point

References

- IEEE 802.3 – 2000 Edition

Carrier sense multiple access with collision detection (CSMA/CD) MAC and physical layer specification.

- ITU-T G.774 Series

SDH Management Information Model for the Network Element View

- ITU-T G.783

Characteristics of Synchronous Digital Hierarchy (SDH) Equipment Functional Blocks

- ITU-T G.7041

Generic Framing Procedure

- RFC 1471

The Definitions of Managed Objects for the Link Control Protocol of the Point-to-Point Protocol

- RFC 2863

The Interfaces Group MIB

# Introduction

- This section provides OAM&P (Operation, Administration, Maintenance and Provisioning) and Layer Management functionalities supported by RPR equipment.

- Management Functional Areas pertinent to RPR are:

- Configuration management

- Fault management

- Performance management

- Security management (not addressed in this draft)

- Configuration Management exercises control over, identifies, collects data from, and provides data to Network Elements (NEs) and the connections between NEs. Configuration Management is responsible for the installation of NEs, their interconnection into a network (configuration) and provisioning. [Note that it does not check and report configuration anomalies, this is fault management. Depending on the system used to configure the NE, it may not allow the user to enter an invalid value, but it cannot be used for checking configuration errors.]

- Performance Management evaluates and reports upon the behavior of NEs and the effectiveness of the network and NEs for the support of services. Performance Management provides mechanisms to measure service quality, by monitoring the system performance. It also reports statistics information to the management system

- Fault (or Maintenance) Management enables the detection, isolation, and correction of abnormal operation of the NEs and its network. It is responsible to detect and process any faults as well as to report it to the management system.

- In order to improve fault and performance management capability, e.g. to allow fault detection, some in-band OAM functions are envisaged.

- [How about add something about the Layer Management and how RPR will handle management?]

## Overview of RPR systems

- The IEEE 802.17 MAC is a single MAC layer supporting two physical interfaces on the ring: the east interface and the west interface (see Figure 1.1 and Figure 1.2).

- Physical interfaces are usually bi-directional interfaces thus each span interface receives from one ringlet and transmits on the opposite ringlet. With a reference to Figure 1.1, the west span interface receives frames from the counterclockwise ringlet and transmits frames on the clockwise ringlet; the opposite applies to the east interface.

              - – RPR Ring View

              - – RPR Functional Model

- The system has at least three interfaces: two (i.e. east and west) span interfaces (PHY-SAPs) and one RPR interface (MSAP) to the upper layers. The OAM&P requirements for the east and west span interfaces depend on the medium (i.e. Ethernet or Sonet/SDH) used while the OAM&P requirements of the RPR interface are independent on the physical medium.

  - Overview of OAM for RPR

- SONET/SDH has in-band OAM functionality specified in ITU G.783 [B3]. Based on G.783 specification, it is possible to detect Signal Fail (SF) in each span (SONET SPE or SDH VC). SONET/SDH interfaces used in RPR systems should support the OAM requirements as specified in Recommendation G.783 [B3].

- The IEEE 802.3 standards for 1 GbE (802.3ac) and 10 GbE (802.3ae) LAN PHY interfaces have no in-band OAM functionality defined. The only information that is known is if the physical interface is available or not available. The unavailability of the physical interface directly translates into a Signal Fail (SF) condition for the MAC layer.

- In-band OAM functionality for IEEE 802.3 standard 10 GbE (802.3ae) WAN PHY interface is TBD.

- In addition to some OAM mechanisms provided at the RPR layer, IEEE 802.17 RPR re-uses existing in-band OAM mechanisms provided by the PHY layers.

- Some OAM flows in the RPR MAC layer are defined in section 2. These are OAM functionality for the RPR layer, and are independent on the PHY technology used.

- In-band OAM functionality used in upper layers is outside the scope of the IEEE 802.17 MAC layer.

## Structure of the document

- Section 2 defines some OAM flows that are useful to help RPR fault management. In particular it defines a loopback mechanism only because RPR is a connectionless technology. This section will likely become an autonomous clause in the IEEE 802.17 standard.

- Section 3 defines the Layer Management requirements for the RPR. It will likely become an autonomous clause in the IEEE 802.17 standard.

- Section Annex 3A defines the SNMP MIB objects for the RPR MAC layer. It will likely become an annex to the Layer Management clause in the IEEE 802.17 standard.

# Operation Administration and Maintenance (OAM)

- The current OAM mechanisms supported by the IEEE 802.17 MAC are used for:

- Fault Management

- Activation/Deactivation

- The fault management mechanisms are useful to check the reachability at the MAC layer between two RPR nodes on the ring, especially when there are some failures (e.g. one station on the ring steals packets addressed to some other stations) that are not detected at layer 1.

- This section defines two Fault Management mechanisms.

- The first mechanism is an on-demand in-service Ping mechanism (see section 2.2) used for troubleshooting the RPR network (reactive mechanism).

- The second mechanism is a Continuity Check (CC) that is used for fault detection (proactive mechanism) by continuously running and warning the operator when there are some problems (see section 2.3).

- As part of the Fault Management, a Remote Defect Indication (RDI) mechanism is also defined. It is used together with the CC mechanism to inform the source node of a flow that the destination node has detected a failure on that flow (see section 2.4). Currently, the only failure that can be notified is the Loss of Continuity (LOC).

- The CC mechanism is an optional mechanism and should be activated in both the source and the destination station. The activation/deactivation of the CC mechanism on both stations can be done by the management system either on both stations or on only one of them. In the former case, the coordination between the beginning of the transmission and reception of CC frames is under the responsibility of the management system. In the latter case, an Activation/Deactivation mechanism is defined to coordinate the activation between the source and destination stations (see section 2.5).

- [Editor’s note – The need to have the Activation/Deactivation OAM mechanism is still an open issue to discuss.]

          - – OAM Type values

  - RPR Ping function

- The IEEE 802.17 allows the management system to request an echo operation to a specified destination in order to check the reachability of an RPR station.

- The management system can specify if the request must follow the shortest path (using the topology discovery), the clockwise or the counterclockwise ringlet. By default, if nothing is specified, the RPR ping request is sent on the shortest path.

- It can specify also the CoS that should be used to ping the destination. By default, the highest priority CoS will be used.

- It can require the addressed station to reply either on the shortest path, or on the same ringlet it received the request, or on the opposite ringlet, or on the counterclockwise or on the clockwise ringlet. By default, if nothing is specified, the addressed station is required to reply on the shortest path.

- After the request is made on the source station, it will output an RPR OAM Ping Request frame to the addressed station, together with an indication over which ringlet that station should answer by using the Request Type field (see section 2.1.1).

- When an RPR OAM Ping Request message addressed to this station is received, an RPR OAM Ping Reply message is sent as a response to the request’s source station. The reply is sent over the ringlet selected in the request message.

- When the RPR OAM Ping Reply message is received, the management system is notified about the success of the procedure. If the reply is not received within a pre-defined time interval, either because the request cannot reach the addressed station or because the reply cannot came back to the source station, the ping is declared failed and the management system is notified about this failure.

## RPR Continuity Check (CC)

- Continuity Check allows detecting per flow failures, such as one Station “stealing” the frames from another Station. It can also be used to verify connectivity in the protection path.

- Continuity Check frames transmission and reception can be activated either using Activation/Deactivation frames or by configuration. In order to avoid meaningless alarms, it is required that the CC frame insertion is activated in the source station before activating the CC frame detection in the destination station.

- CC frames are sent with a periodicity of nominally 1 frame per second. CC frames use the highest priority Class Of Service.

- When the sink Station does not receive any CC frame within a time interval of 3.5+0.5 seconds, it will declare a Loss Of Continuity (LOC) defect. LOC shall be removed when a CC frame is detected.

- Each side of the Station can have separate CC capability (ringlet specific CC). In this case the source station always sends the CC frames on the specified ringlet. The destination station will declare LOC on per ringlet basis.

- It is also possible to activate CC without specifying the side (ringlet agnostic CC). In this case the source station sends the CC frames on the shortest path and steers them during ring failures. The destination station will declare LOC on per station basis.

## RPR Remote Defect Indication (RDI)

- The Station detecting a Loss Of Continuity (LOC) defect shall generate and send back to the source station of the failed flow, a RDI frame. The RDI frame shall be transmitted through the shortest path and steered in case of failure. The SA should be the detecting Station MAC address and the DA should be the MAC address of the Station sourcing the failed flow, the ringlet in which the LOC was detected shall be indicated in the Defect Location field.

- The RDI frame shall be generated and transmitted as soon as possible after detection of the LOC defect, and shall be periodically transmitted during the defect condition. The generation frequency of the RDI frame shall be one frame per second.

- The RDI frame generation shall be stopped as soon as the defect indication is removed.

- The RDI frames shall be detected at the respective flow sourcing Station. The RDI state shall be declared at the RDI frame detecting Station as soon as a RDI frame is received. The RDI state is released when RDI frames are absent for 2.5+0.5 seconds.

## RPR Activation/Deactivation of OAM

- Since Continuity Check can be activated at any time, an initialization procedure is needed between the two endpoints of the flow to properly initialize the OAM process. Specifically, this initialization procedure may serve the following purposes:

- To coordinate the beginning or end of the transmission and reception of CC

- To establish the type of procedure

- To specify relevant parameters (if required)

- The initialization procedure may be performed either via configuration, or using the Activation/Deactivation frames.

- If no response is received for an Activation/Deactivation frame within 5 seconds, the frame shall be resend. If no response is received after 3 attempts the operation shall be declared as failed.

- A Station that does not support Continuity Check may respond to the relevant Activation messages with Activation Request Denied, or silently discard the Activation/Deactivation frame.

## Generic OAM Frame Format

- [Editor’s note – The RPR frame format should allow the identification of RPR OAM frames. Some text will be added to recall that mechanism when the frame format group defines it.]

- RPR OAM frames are RPR Control frames (with the Payload Type field equal to XXX and the Message Type field equal to XXX), as defined in clause 7.

- The format of the control type specific part of the OAM frames is defined as show in Figure 2.1.

              - – Generic OAM frame control type specific information

- The following fields are defined.

- OAM Type – Specifies which type this OAM message is. All the allowed codes are defined in Table 2.2.

- Specific Fields – The length and the structure of this part depends on the OAM message type. The format of this structure is defined with the relevant OAM message type.

- Checksum – The last 16 bits of the OAM frame payload contain the checksum (CRC-16) calculated on the entire OAM message payload, starting from the OAM type and ending at the last specific field (if present). When an OAM frame is received with a bad checksum, it is silently discarded.

| OAM Type | Description |
| --- | --- |
| 0x00 | RPR ping request |
| 0x01 | RPR ping reply |
| 0x02 | RPR Continuity Check |
| 0x03 | RPR Remote Defect Indication |
| 0x04 | RPR CC Activation/Deactivation |
| All the others | Reserved |

          - – OAM Type values

- Note – All the received OAM frames that are addressed to the station and have a reserved OAM Type must be silently discarded.

### Ping Request OAM RPR Format

- The format of the control type specific part of the Ping Request OAM RPR frame is shown in Figure 2.1.

              - – RPR Ping Request OAM format

- The following fields are defined:

- OAM Type – Represents a ping request type (i.e. 0x00 value, as in Table 2.2)

- Request Type – It is used to ask the addressed station how to respond, if using the shortest path, the same or the opposite ringlet the request was receive, the counterclockwise or the clockwise ring. Valid values are defined in Table 2.3.

- Identifier and Sequence Number – They are used by the application to correlate the requests with the replies. The meaning of these two fields and how they are used to make the correlation are implementation dependent.

- The Ping Request OAM RPR frame header fields are defined as follow:

- The destination address is filled with the value passed by the LME representing the station to be echoed.

- The Payload Type field is set to XXX (RPR Control Frame), as defined in clause 7.

- The CoS field is filled with the value requested by the upper layer.

| Request Type | Description |
| --- | --- |
| 0x00 | The addressed station replies on the ringlet chosen by the shortest path (via topology discovery) |
| 0x01 | The addressed station replies on the counterclockwise ringlet |
| 0x02 | The addressed station replies on the clockwise ringlet |
| 0x03 | The addressed station replies on the same ringlet it has received the request |
| 0x04 | The addressed station replies on the opposite ringlet it has received the request |
| All the others | Reserved |

          - – Request Type values

- Note – Only the 0x00 request type value must be supported, all the other types are optional. All the requests that are received with an unsupported or a reserved request type value must be discarded.

### Ping Reply OAM message

- The format of the control type specific part of the Ping Reply OAM RPR frame is shown in Figure 2.1.

              - – RPR Reply OAM format

- The following fields are defined:

- OAM Type – Represents a ping reply type (i.e. 0x01 value, as in Table 2.2)

- Reserved – This field is not used. It must be fixed to 0x00 in transmission and ignored in reception.

- Identifier and Sequence Number – They are copied from the request message and used by the application to correlate the requests with the replies. The meaning of these two fields and how they are used to make the correlation are implementation dependent.

- The Ping Reply OAM RPR frame header fields are defined as follow:

- The destination address copied from the request’s source address.

- The Payload Type field is set to XXX (RPR Control Frame), as defined in clause 7.

- The CoS field is copied from the loopback request frame.

### Continuity Check OAM message

- There are no control type specific fields in the CC OAM RPR frame.

- The following field is defined:

- OAM Type – Represents a continuity check type (i.e. 0x02 value, as in Table 2.2)

- The CC OAM RPR frame header fields are defined as follow:

- The destination address is filled with the value passed by the LME.

- The Payload Type field is set to XXX (RPR Control Frame), as defined in clause 7.

- The CoS field is set to the highest value (111).

### Remote Defect Indication OAM message

- The format of the control type specific part of the RDI OAM RPR frame is shown in Figure 2.2.

              - – RPR RDI OAM format

- The following fields are defined:

- OAM Type – Represents a remote defect indication type (i.e. 0x03 value, as in Table 2.2)

- Reserved – These fields are not used. They must be fixed to 0x00 in transmission and ignored in reception.

- Defect Type – It indicates the kind of defect the RDI frame is indicating. Valid values are defined in Table 2.4.

- Defect Location – It indicates in which ringlet the defect has been detected. If the defect detection is not applicable to a specific ringlet (e.g. a LOC due to a “ringlet agnostic CC” mechanism), a reserved code (i.e. the “1111 1111” value) will indicate the “unspecified ringlet”.

- The RDI OAM RPR frame header fields are defined as follow:

- The destination address depends on the failure. For LOC reporting, it is the MAC address of the station that is supposed sending the CC frames that are not being received.

- The Payload Type field is set to XXX (RPR Control Frame), as defined in clause 7.

- The CoS field is set to the highest value (111).

| Defect Type | Description |
| --- | --- |
| 0x00 | Loss of Continuity (LOC) |
| 0xFF | Defect not specified |
| All the others | Reserved |

          - – Defect Type values

### CC Activation/Deactivation OAM message

- The format of the control type specific part of the CC activation/deactivation OAM RPR frame is shown in Figure 2.1.

              - – RPR CC Activation/Deactivation OAM format

- The following fields are defined:

- OAM Type – Represents a CC activation/deactivation type (i.e. 0x04 value, as in Table 2.2)

- Reserved – These fields are not used. They must be fixed to 0x00 in transmission and ignored in reception.

- Identifier and Sequence Number – They are used to correlate the commands with responses. In the commands, the meaning of these fields and how they are used to make the correlation are implementation dependent. In the responses, these fields are copied from the commands.

- Message Type – It is used to define which type of message it is (activation or deactivation; command or response). Valid values are defined in Table 2.5.

- Ringlet ID – It is used to indicate which ringlet (clockwise or counterclockwise) the ringlet specific CC mechanism should be activated/deactivated. A reserved value (i.e. “1111 1111”) is used to activate the ‘ringlet agnostic CC’.

- The CC AD OAM RPR frame header fields are defined as follow:

- The destination address copied from the request’s source address.

- The Payload Type field is set to XXX (RPR Control Frame), as defined in clause 7.

- The CoS field is copied from the loopback request frame.

| Message Type | Description |
| --- | --- |
| 0x01 | Activate (Command) |
| 0x02 | Activation Confirmed (Response) |
| 0x03 | Activation Denied (Response) |
| 0x04 | Deactivate (Command) |
| 0x05 | Deactivation Confirmed (Response) |
| All the others | Reserved |

          - – Message Type values

# Layer Management

## Overview of the management model

- Both RPR MAC and all applicable PHY layers conceptually include management entities, called MAC sub layer management and PHY layer management entities (MLME and PLME, respectively). These entities provide the layer management service interfaces through which layer management functions may be invoked.

- In order to provide correct RPR MAC operation, a station management entity (SME) must be present. The SME is a layer-independent entity that may be viewed as residing in a separate management plane. The exact functions of the SME are not specified in this standard, but in general this entity may be viewed as being responsible for such functions as the gathering of layer-dependent status from the various layer management entities, and similarly setting the value of layer-specific parameters. SME would typically perform such functions on behalf of general system management entities and would implement standard management protocols. Figure 3.1 depicts the relationship among management entities.

- The management SAPs within this model are the following:

- SME-MLME SAP

- SME-PLME SAP

- In this fashion, the model reflects what is anticipated to be a stackable implementation approach in which PLME functions are controlled by SME. In particular, different PHY implementations are required to have separate interfaces with the SME. The interfaces of the SME with the different PHYs are not part of this standard and are specified in the respective standard documents that specify the management primitives and MIBs for the different PHYs.

              - – RPR MAC Management SAPs

## Generic management primitives

- The management information specific to each layer is represented as a management information base (MIB) for that layer. The MAC and PHY layer management entities are viewed as “containing” the MIB for that layer. The generic model of MIB-related management primitives exchanged across the management SAPs is to allow the SAP user-entity to either GET the value of a MIB attribute, or to SET the value of a MIB attribute. The invocation of a SET.request primitive may require that the layer entity perform certain defined actions.

- The GET and SET primitives are represented as REQUESTs with associated CONFIRM primitives. These primitives are prefixed by MLME or PLME depending upon whether the MAC or PHY layer management SAP is involved.

- In the following, XX denotes MLME or PLME:

- XX-GET.request (MIBattribute)

- Requests the value of the given MIBattribute.

- XX-GET.confirm (status, MIBattribute, MIBattributevalue)

- Returns the appropriate MIB attribute value if status = “success,” otherwise returns an error indication in the Status field. Possible error status values include “invalid MIB attribute”.

- XX-SET.request (MIBattribute, MIBattributevalue)

- Requests that the indicated MIB attribute be set to the given value. If this MIBattribute implies a specific action, then this requests that the action be performed.

- XX-SET.confirm (status, MIBattribute)

- If status = “success,” this confirms that the indicated MIB attribute was set to the requested value, otherwise it returns an error condition in status field. If this MIBattribute implies a specific action, then this confirms that the action was performed. Possible error status values include “invalid MIBattribute” and “attempt to set read-only MIB attribute.”

- Additionally, there are certain requests (with associated confirms) that may be invoked across a given SAP that do not involve the setting or getting of a specific MIB attribute. Each SAP supports one of these, as follows:

- XX-RESET.request: where XX is MLME or PLME as appropriate

- XX-RESET.confirm

- This service is used to initialize the management entities, the MIBs, and the data path entities. It may include a list of attributes for items to be initialized to non-default values. The corresponding.confirm indicates success or failure of the request.

- Other SAP-specific primitives are in the following sections.

## MLME SAP interface

- The services provided by the MLME to the SME are specified in this section. These services are described in an abstract way and do not imply any particular implementation or exposed interface. MLME SAP primitives are of the general form ACTION.request followed by ACTION.confirm. The SME uses the services provided by the MLME through the MLME SAP.

- According to the IETF layering principles, the RPR interface should be stacked over the two west and east span interfaces.

### RPR interface configuration

- The RPR ring interface can be activated/deactivated for administrative purposes. Its activation/deactivation allows/forbids the upper layers to send packets on the ring. It can be activated if and only if at least one of the underlying span interfaces is activated.

- The RPR interface has its own operational state that goes down when both the span interfaces go down.

- The RPR MAC entity requires the MAC address. This is fixed by the vendor and can be only read for maintenance purposes.

### Topology discovery monitoring

- The configuration management should allow monitoring, for maintenance purposes, the state of the auto-configuration and topology discovery protocols. It should also allow disabling the support of some features, even if supported by all the stations on the ring.

- The detailed configuration requirements for the auto-configuration and topology discovery protocols depend from the actual mechanism that will be used and are now for further study.

- If some misconfiguration condition is detected, a notification is sent to the management system for maintenance purposes. For example, the topology discovery mechanism can discover that two stations on the ring have the same MAC address.

### Protection switching

- The protection management mechanisms that will be defined in this standard are based on information known to the MAC entity. PHY layer protection mechanisms are independent and are addressed in the appropriate PHY specifications.

- The configuration management should allow monitoring, for maintenance purposes, the state of the protection switching.

- It should also allow activating/deactivating the RPR protection as well as activating/deactivating the usage of the Signal Degrade condition as switching criteria.

- When the RPR protection is deactivated, the RPR MAC never switches under failure conditions. When the usage of the SD as a switching criteria is disabled, the RPR MAC never switches because of a signal degrade condition detected on the physical interface.

- In order to support multi-layer protection mechanisms, a hold-off timer should be configured for each span interface, because it highly depends on the kind of physical network you have between two adjacent RPR nodes, if any.

- In order to ensure robustness against unstable alarms, a wait time to restore (WTR) should be configured for each node, because there is no need to have a granular configuration of it.

- The usage of the hold-off timer and of the wait to restore is specified in the protection switching section.

- It should be possible to force a switching event for operational purposes.

- This mechanism deals with the modes under which an RPR MAC switches to protection.

#### MLME-SWITCHTOPROTECTION.request

- This primitive requests a change in the protection mode of an RPR ring.

- The primitive parameters are as follows:

- MLME-SWITCHTOPROTECTION.request (

- ReversionMode,

- Side

- )

- This primitive is generated by the SME to implement a user request for a node to switch to protection at the east or the west side.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ReversionMode | Enumeration | FORCED, AUTOMATIC-REVERTIVE, AUTOMATIC-NONREVERTIVE | An enumerated type that describes the desired way to revert from protection |
| Side | Enumeration | WEST, EAST | An enumerated type that describes the side of the ring relative to the station that will switch |

Effect of receipt

- This request sets the reversion mode of the protection switch that occurs at the specified side. The MLME subsequently issues a MLME- SWITCHTOPROTECTION.confirm that reflects the results of the protection switch request.

#### MLME- SWITCHTOPROTECTION.confirm

- This primitive confirms the change in protection switching mode.

- The primitive parameters are as follows:

- MLME- SWITCHTOPROTECTION.confirm (

- ResultCode

- )

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, NOT_SUPPORTED | Indicates the result of the MLME- SWITCHTOPROTECTION.request |

- This primitive is generated by the MLME as a result of an MLME- SWITCHTOPROTECTION.request by the SME. It is not generated until the switch is completed.

Effect of receipt

- The SME is notified of the protection switch.

### Performance and Accounting Measurements

- The following statistics should be kept for the RPR interface.

  - RCF performance monitoring.

  - How many frames/octets have been inserted on the ring (by the upper layer)

  - How many frames/octets have been delivered to the upper layer

  - How many frames, received from the west interface, have been stripped because originated by the node itself

  - How many frames, received from the east interface, have been stripped because originated by the node itself

  - How many pass-through frames, received from the west interface, have been discarded because of the TTL expiration

  - How many pass-through frames, received from the east interface, have been discarded because of the TTL expiration

  - How many frames, received from the west interface, have been discarded because of a bad FCS (this counter is fixed to 0 when the cut-through method is implemented)

  - How many frames, received from the east interface, have been discarded because of a bad FCS (this counter is fixed to 0 when the cut-through method is implemented)

  - How many frames addressed to the node have been discarded because of a bad FCS (this counter is fixed to 0 when the store and forward method is implemented)

  - How many frames, addressed to the node, have been discarded because of an unknown or unsupported protocol.

  - How many frames addressed to the node have been discarded even if no error has been detected (e.g. because of buffer congestion). This counter has an implementation specific meaning and in some implementations it may be always equal to 0.

  - How many frames originated by the node have been discarded (e.g. because of buffer congestion). This counter has an implementation specific meaning and in some implementations it may be always equal to 0.

  - How many frames, received from the west interface, correctly passed-through the MAC entity.

  - How many frames, received from the east interface, correctly passed-through the MAC entity.

  - How many pass-through frames, received from the west interface, have been discarded even if no error has been detected (e.g. because of buffer congestion). This counter has an implementation specific meaning and in some implementations it may be always equal to 0.

  - How many pass-through frames, received from the east interface, have been discarded even if no error has been detected (e.g. because of buffer congestion). This counter has an implementation specific meaning and in some implementations it may be always equal to 0.

- For the counters defined in points 1, 2, 9, 11, 12, 13, 14, 15 and 16, there is one counter for each QoS class and a global counter.

#### Topology Discovery statistics

- The exact requirements depend on the mechanism that is used and then are for further study.

### Notifications and Fault Management

- TBD

### RPR Ping Management

#### MLME-PING.request

- This primitive requests the station to loopback another station on the RPR ring.

- The primitive parameters are as follows:

- MLME-PING.request (

- Addressed Station,

- Ringlet,

- Request Type,

- CoS,

- Timer

- )

- This primitive is generated by the SME to implement a user request for a node to perform a ping operation.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Addressed Station | MAC Address | Any valid unicast MAC address | The MAC address of the RPR station to be ping |
| Ringlet | Enumeration | SHORTEST, COUNTERCLOCKWISE, CLOCKWISE | The ringlet over which the RPR ping request message should be sent (see section 2) |
| Request Type | Enumeration | SHORTEST, SAME, OPPOSITE, COUNTERCLOCKWISE, CLOCKWISE | The ringlet over which the addressed station should send the RPR ping reply message (see section 2) |
| CoS | Integer | (0…7) | The CoS to be used in the RPR OAM frames carrying the RPR ping request and reply messages. |
| Timer | Integer | (0..65535) | The number of seconds the source station should wait for the reply before declaring the ping failed. |

Effect of receipt

- This requests causes the station to send an OAM RPR ping request message.

#### MLME- PING.confirm

- This primitive notifies the success or the failure of a ping operation.

- The primitive parameters are as follows:

- MLME- PING.confirm (

- ResultCode

- )

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, FAILURE | Indicates the result of the MLME- PING.request |

- This primitive is generated by the MLME as a result of an MLME- PING.request by the SME. It is not generated until the OAM RPR ping reply frame is received or the timer expires.

Effect of receipt

- The SME is notified of the success or of the failure of the ping procedure.

## Link Aggregation

- [Editor’s note – If Link Aggregation for RPR Links is supported then Link Aggregation Objects and a Link Aggregation MIB will be defined. For this purpose the IEEE 802.3 link aggregation objects defined in the LAG-MIB (IEEE 802.3 – Annex 30C) might serve as a guide.]

- Link aggregation can be managed as an interface stacked over multiple RPR interfaces

- A request for a new ifType for the aggregation should be forwarded to the IANA

## PLME SAP interface

- Each span interface can be activated/deactivated separately for administrative purposes. Its activation/deactivation allows/forbids the MAC layer to send packets on that span.

- In order to avoid inconsistent configurations, a span interface can be deactivated only if the RPR interface has been deactivated or the other span interface is still active.

- Each span interface has its own operational state that can be read for maintenance purposes.

- The management of Sonet/SDH PHY as well as of Ethernet physical interfaces are already defined in the relevant standard recommendations. The IEEE 802.17 will reuse the already defined PLME SAP primitives to manage the Sonet/SDH and Ethernet PHY layers.

- The number frames that are received or transmitted on each span interface are counted as span interface statistics representing the number of frames that the span interface has delivered or has received from the upper layer.

### The Ethernet PHY

- For the Ethernet PHY we will use the 802.3ae LAN and WAN PHY primitives. Also we will use the MIB objects that are defined in the MAU-MIB (RFC 2668) for the Ethernet PHY objects and the updated MIB in preparation (draft-ietf-hubmib-mau-mib-v3-00) for management of 10 Gb/s LAN PHY. In addition we will use the MIB objects defined in  (draft-ietf-hubmib-wis-mib-00) to manage the10 Gb/s WAN PHY – called ETHER-WIS-MIB.

- The Ethernet Reconciliation Sublayer management should be defined in the IEEE 802.17 specification.

- According to the IETF layering principles, the RPR interface, when working over Ethernet interfaces, should be stacked over the west and east “Ethernet” interfaces.

- An Ethernet span interface can be activated at any time, because it is the lowest level interface on the system. Its operational state goes down when the media is unavailable.

### The SONET PHY

- Sonet/SDH interfaces are layered interfaces and are managed as a set of stacked interfaces. The physical medium, the section and the line layers are managed as a single layered interface. The recommended IANA ifType is sonet (39). The Path layer is managed as a stacked interface over the Medium/Section/Line interface. The recommended IANA ifType is sonetPath (50). For the SONET PHY the SONET/SDH PHY objects are defined in the SONET-MIB (RFC 2558).

#### The GFP Adaptation Layer

- An additional GFP interface might be stacked over the Path interface to represent the GFP adaptation layer.

- The GFP management is outside the scope of IEEE 802.17.

- According to the IETF layering principles, the RPR interface, when working over Sonet/SDH interfaces, should be stacked over the west and east GFP interfaces.

- A GFP span interface can be activated only if the underlying Sonet/SDH interface is activated. Its operational state goes down when a signal fail condition is detected on the Sonet/SDH path or if there is a payload mismatch (the value in the received C2 byte is different than the GFP code, i.e. [TBD] value) or there is a loss of frame alignment.

- [Editor’s note – The signal label value to be put in the C2 byte of Sonet/SDH interfaces has not yet been assigned by ITU-T.]

- RPR SNMP MIB Definition

  - Introduction

- This section defines a portion of the Management Information Base (MIB) for use with network management protocols in the TCP/IP base Internets. In particular it defines objects for managing the IEEE 802.17 RPR interfaces.

- This section includes a MIB module that is SNMPv2 SMI compliant.

  - The SNMP Management Framework

- The SNMP Management Framework presently consists of five major components:

    - An overall architecture, described in RFC 2571 [1].

    - Mechanisms for describing and naming objects and events for the purpose of management. The first version of this Structure of Management Information (SMI) is called SMIv1 and described in STD 16, RFC 1155, STD 16, RFC 1212 and RFC 1215. The second version, called SMIv2, is described in STD 58, RFC 2578, STD 58, RFC 2579 and STD 58, RFC 2580.

    - Message protocols for transferring management information. The first version of the SNMP message protocol is called SNMPv1 and described in STD 15, RFC 1157. A second version of the SNMP message protocol, which is not an Internet standards track protocol, is called SNMPv2c and described in RFC 1901 and RFC 1906.  The third version of the message protocol is called SNMPv3 and described in RFC 1906, RFC 2572 and RFC 2574.

    - Protocol operations for accessing management information. The first set of protocol operations and associated PDU formats is described in STD 15, RFC 1157. A second set of protocol operations and associated PDU formats is described in RFC 1905.

    - A set of fundamental applications described in RFC 2573 and the view-based access control mechanism described in RFC 2575.

- Managed objects are accessed via a virtual information store, termed the Management Information Base or MIB.  Objects in the MIB are defined using the mechanisms defined in the SMI.

- This Annex specifies a MIB module that is compliant to the SMIv2. A MIB conforming to the SMIv1 can be produced through the appropriate translations. The resulting translated MIB must be semantically equivalent, except where objects or events are omitted because no translation is possible (use of Counter64). Some machine readable information in SMIv2 will be converted into textual descriptions in SMIv1 during the translation process. However, this loss of machine readable information is not considered to change the semantics of the MIB.

  - Structure of the MIB

  - Relationship to the Interfaces MIB

- The Interface MIB [B6] requires that any MIB, which is an adjunct of the Interface MIB, clarify specific areas within the Interface MIB.

- These areas were intentionally left vague in the Interface MIB to avoid over constraining the MIB, thereby precluding management of certain media-types.

- Section 3.3 of [B6] enumerates several areas which a media-specific MIB must clarify.  Each of these areas is addressed in a following subsection.  The implementor is referred to [B6] in order to understand the general intent of these areas.

    - Layering Model

- Any RPR interface is stacked over two lower-layer interfaces, representing the two spans.

- The layering relationship between the RPR interface and the lower-layer interfaces is defined by the ifStackTable defined in [B6]. The RPR MIB module should define a way for the management system to designate which of the two span interface is the east and which is the west interface.

    - ifRcvAddressTable

- This table contains all the IEEE 802.17 addresses, unicast, multicast or broadcast, for which this interface will receive packets and forward them up to a higher layer entity for local consumption. The format of the address, contained in the ifRcvAddressAddress object, is the same as for ifPhyAddress.

- In the event that the interface is part of a MAC bridge, this table does not include unicast addresses, which are accepted for possible forwarding out some other port.  This table is explicitly not intended to provide a bridge address filtering mechanism.

    - ifPhyAddress

- This object contains the IEEE 802.17 address, which is placed in the source-address field of any RPR frames that is originated at this interface.  Usually this will be kept in ROM on the interface hardware.  Some systems may set this address via software.

- If the address cannot be determined, an octet string of zero length should be returned.

- The address is stored in binary in this object.  The address is stored in "canonical" bit order, that is, the Group Bit is positioned as the low-order bit of the first octet.  Thus, the first byte of a multicast address would have the bit 0x01 set.

    - ifType

- This MIB applies to interfaces which have the following ifType value:

- ieee80217Rpr (TBA)

- A request for a new ifType for the RPR interface should be forwarded to the IANA

    - Specific Interface MIB Object

- Table 3.1 provides specific guidelines for applying the interface group generic objects to the RPR media.

| Object | Definitions |
| --- | --- |
| ifIndex | A unique value is allocated to each RPR interface by the local system. It is interpreted as defined in [B6]. |
| ifDescr | Refer to [B6]. |
| ifType | Refer to section 3A.4.5. |
| ifMTU | TBD octets. This is the MTU as seen by the MAC client. |
| ifSpeed | The current operational speed of the interface. Both the east and west span interfaces are assumed to have the same speed. |
| iPhysAddress | Refer to section 3A.4.4. |
| ifAdminStatus | Refer to [B6]. |
| ifOperStatus | It goes “down” when both the span interfaces’ operation status is “down”. It is “up” when either one of the span interfaces’ operational status is “up”. |
| ifLastChange | Refer to [B6]. |
| ifInOctets | Refer to [B6]: the number of octets in valid MAC frames received on this RPR interface, including the MAC header and FCS. This does not include the number of octets in valid MAC control frames received on this interface, because they are not passed to any higher layer protocol. |
| ifInUCastPkts | Refer to [B6]. This does not include the MAC control frames received on this interface, because they are not passed to any higher layer protocol. |
| ifInDiscards | Refer to [B6]. |
| ifInErrors | TBD |
| ifInUnknownProtos | Refer to [B6]. |
| ifOutOctets | Refer to [B6]: the number of octets in valid MAC frames transmitted on this RPR interface, including the MAC header and FCS. This does not include the number of octets in valid MAC control frames transmitted on this interface, because they are not generated by any higher layer protocol. |
| ifOutUcastPkts | Refer to [B6]. This does not include the MAC control frames transmitted on this interface, because they are not generated by any higher layer protocol. |
| ifOutDiscards | Refer to [B6]. |
| ifOutErrors | TBD |
| ifName | Refer to [B6]. |
| ifInMulticastPkts | Refer to [B6]. This does not include the MAC control frames received on this interface, because they are not passed to any higher layer protocol. |
| ifInBroadcastPkts | Refer to [B6]. This does not include the MAC control frames received on this interface, because they are not passed to any higher layer protocol. |
| ifOutMulticastPkts | Refer to [B6]. This does not include the MAC control frames transmitted on this interface, because they are not generated by any higher layer protocol. |
| ifOutBroadcastPkts | Refer to [B6]. This does not include the MAC control frames transmitted on this interface, because they are not generated by any higher layer protocol. |
| ifHCInOctets ifHCOutOctets | Refer to [B6]. |
| ifHCInUcastPkts ifHCInMulticastPkts ifHCInBroadcastPkts ifHCOutUcastPkts ifHCOutMulticastPkts ifHCOutBroadcastPkts | Refer to [B6]. |
| ifLinkUpDownTrapEnable | Refer to [B6]. Default is ‘disabled’ because the RPR interface is not the lowest-level one. |
| ifHighSpeed | Refer to [B6]. |
| ifPromiscuousMode | Refer to [B6]. |
| ifConnectorPresent | This will always be ‘false’, because it is not a lowest-level interface. |
| IfAlias | Refer to [B6]. |
| ifCounterDiscontinuityTime | Refer to [B6]. A discontinuity in the Interface MIB counters may also indicate a discontinuity in some or all of the counters in this MIB that are associated with tat interface/ |
| ifStackHigherLayer ifStackLowerLayer ifStackStatus | Refer to section 3A.4.2. |
| ifRcvAddressAddress ifRcvAddressStatus ifRcvAddressType | Refer to section 3A.4.3. |

          - – ifTable element definitions for an RPR interface

  - Definitions for the RPR MIB

-- *******************************************************************

-- Textual Conventions

-- *******************************************************************

InterfaceSide ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The interface side on the ring."

SYNTAX INTEGER {

east (1),

west (2) }

-- *******************************************************************

-- the RRP MAC Interface group

-- *******************************************************************

rprIfTable OBJECT-TYPE

SYNTAX  SEQUENCE OF RprIfEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"The RPR interface table."

::= { rprMAC 1 }

rprIfEntry OBJECT-TYPE

SYNTAX  RprIfEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"There exists one such entry for every interface

in the ifTable which has an ifType of RPR interface.

Each of these entries are indexed by the value

of ifIndex as defined in RFC2863."

INDEX   { ifIndex }

::= { rprIfTable 1 }

RprIfEntry ::=

SEQUENCE {

rprNodesOnTheRing       Integer32,

rprWaitToRestoreTimer	Integer32,

rprIfTimeElapsed        Integer32,

rprProtectionState	INTEGER,

rprIfValidIntervals     Integer32

}

-- *******************************************************************

-- The dual counter rotating ring topology map

-- *******************************************************************

rprRingTopologyMapTable OBJECT-TYPE

SYNTAX  SEQUENCE OF RprRingTopologyMapEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"A list of RPR MACs that form a dual counter rotating ring. This list represents the topology of the ring as viewed from a specific station."

::= { rprMAC  2 }

RprRingTopologyMapEntry OBJECT-TYPE

SYNTAX  RprRingTopologyMapEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"A topology entry containing information

specific to a particular station on the ring.

The distance is computed counterclockwise on the ring."

INDEX   { ifIndex, rprStationDistance }

::= { rprRingTopologyMapTable 1 }

RprRingTopologyMapEntry ::=

SEQUENCE {

rprStationDistance

Unsigned32,

rprStationMACAddress

MacAddress,

rprStationMACName

DisplayString

}

-- *******************************************************************

-- The PRP MAC to PHY side table

-- *******************************************************************

rprMACSideTable OBJECT-TYPE

SYNTAX  SEQUENCE OF rprMACSideEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"A table of information about the sides of RPR interfaces."

::= { rprMAC  3 }

rprMACSideEntry OBJECT-TYPE

SYNTAX  RprMACSideEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"A list of information specific to a particular

side of a particular RPR interface."

INDEX   { ifIndex, rprMACInterfaceSide }

::= { rprMACSideTable 1 }

RprMACSideEntry ::=

SEQUENCE {

rprMACInterfaceSide

InterfaceSide,

rprMACNeighborAddress

MacAddress,

rprMACReversionMode

ReversionMode,

rprMACAutoDetectionMode

DetectionMode,

rprMACTopologyTimer

Integer32

}

-- *******************************************************************

-- The RPR MAC RCF Performance Monitoring Table

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Protection switch thresholds Table

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Protection switch thresholds Table

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Interval Counters

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Interval Traffic Counters

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Interval Error Counters

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Counters

-- *******************************************************************

-- *******************************************************************

-- The RPR MAC Notifications

-- *******************************************************************

- The protection primitive comes from a Layer Management Interface and it includes parameters such as ( Protect CID, from station to station).

- The load balancing function also can come from the Layer management and it includes the parameters (Send this flow on such ringlet ID).