|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 5D/TEMP/6(Rev.1)-E |
|  |  | 31 January 2008 |
|  |  | English only |

Working Party 5D

Liaison statement to IEEE and WiMAX Forum

REQUEST for CLARIFICATION on OFDMA-TDD-WMAN
BS and MS ACS VALUES

At its meeting held in Geneva, 28 January-1 February, WP 5D discussed the values of the Adjacent Channel Selectivity (ACS) for OFDMA-TDD-WMAN base stations (BS) and mobile stations (MS) given in Report ITU-R M.2116 and those calculated with the ACR defined in ITU-R Recommendation M.1457-7 based on Document 5D/70.

WP 5D has noted significant differences, given in the table below, between the ACS values given in Report ITU-R M.2116 and those derived from Recommendation ITU-R M.1457-7.

The calculated values were derived using the following formula:

ACS=PN=SNRmin+3 dB+AdjacentAlternate Channel R ejection

|  | ACS in ITU-R Report M.2116 |  | ACS calculated with ACR defined in ITU-R Rec. M.1457-7 |  |
| --- | --- | --- | --- | --- |
|  | Base station | Mobile station | Base station | Mobile station |
| ACS_1 (dB) | 70 | 40 | 26.7 (16QAM-3/4) 23.9 (64QAM-2/3) | 26.7 (16QAM-3/4) 23.9 (64QAM-2/3) |
| ACS_2 (dB) | 70 | 59 | 45.7 (16QAM-3/4) 42.9 (64QAM-2/3) | 45.7 (16QAM-3/4) 42.9 (64QAM-2/3) |

WP 5D kindly requests assistance from the IEEE and the WiMAX Forum in helping to solve the differences between the ACS values given in Report ITU-R M.2116 and those derived from Recommendation ITU-R M.1457-7, noting that the next meeting of WP5D is scheduled for 
24 June  - 1 July 2008.

Status:		For response

| Contact: | Colin Langtry | Tel: +41 22 730 6178 Email: colin.langtry@itu.int |
| --- | --- | --- |

______________