IEEE P802.11
Wireless LANs

Usage Models

Date:	July 11, 2003

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Tomer Bentzion | Metalink | Yakum Business Park, 60972 Yakum Israel | +972 9 960 5365 | +972 9 960 5399 | tomerb@metalink.co.il |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Mary Cramer | Agere Systems | 1110 American Parkway NE, Allentown, PA 18109-9138, USA | +1 610-712-6112 | +1 610 712 1182 | mecramer@agere.com |
| Javier Delprado | Philips |  |  |  | javier.delprado@philips.com |
| Paul Feinberg | Sony |  |  |  | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Sanjeev Sharma | Samsung | 75 W. Plumeria Dr. , San Jose, CA, 95134 | +1 408- 544 5978 |  | ssharma@sisa.samsung.com |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |

Abstract

This document covers the process of determining usage models for the High Throughput Study Group.  The process includes establishing target usage cases, determining the applications and environments used for the cases, narrowing down and grouping the applications and environments, and developing usage models to be targeted by simulations for verifying the performance of various HTSG technical proposals.

# Revision History

| ### Revision | ### Comments | ### Date | ### Commenter |
| --- | --- | --- | --- |
| R0 Draft 0 | This document is in its beginning phases. The initial target is to generate a concept that can be reviewed and commented on. The first draft has definitions of some terms along with an initial stab at a few usage cases. This is intended to start discussion and review. | July 8, 2003 | Mary Cramer |
| R0 Draft 1 | APS additions | July 9, 2003 | Adrian Stephens |
| R0 | Merged in comments and changes from the group of authors and made public via the .11 reflector. | July 11, 2003 | Adrian Stephens |

# Introduction

To support the definition of a higher data rate standard within the IEEE, this document attempts to define usage models based on various market based usage cases.  The usage models are intended to support the definition of simulations that will be performed by various companies to compare the performance of proposed high throughput solutions.

Note - These usage models we develop here need to satisfy the following constraints (remove note eventually):

- They are relevant to the expected uses of the technology

- They require higher throughput than cannot be achieved with existing 802.11 technology

- They are capable of being turned into an unambiguous simulation scenario

# Process going forward

We (the usage model committee) need to:

- Brainstorm use cases (current mode of operation)

- Select representative use cases to create simulation scenarios (ideally before next IEEE meeting)

# Definitions

We’ve had a lot of confusion over terminology, with the same term meaning different things to different people. This section defines some of the terms used in this document.

Usage Model – Everything in this document!

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial description such as home, large office, small office, stadium, shopping mall, New York city street, train, car are acceptable.  Details should follow when channel model assumptions are made and should include information regarding: indoor, outdoor, construction material, size of rooms, number of floors.

Initial examples:  home, large office

### Usage Case – A usage case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A usage case includes an application in a deployment environment with details regarding the user activity and both sides of the link.

### Examples: Watching television remote from the cable or set-top box within the home.  Talking on the telephone remote from one’s desk at work.

### Simulation Scenario – A simulation scenario is a description of a usage case that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage case to the simulation scenario: environment linked to a channel model, position of the AP (console or ceiling mounted), position of STAs w.r.t. AP, uplink and downlink traffic (# packets, size of packets, interference (number and types of users on the same WLAN channel – adjacent cells, the same cell, number and types of users on alternate channels, BT, baby monitors, GPRS or other systems).

# Mappings between Application, Environment, Usage Case and Simulation Scenarios

(Adrian's note.  In our telecons to date, we assumed a 1:1 mapping between usage case and simulation scenario.  Mary has changed this so that usage cases are related to individual applications.  I don't have any problem with that,  but it changes the multiplicity of the mappings between them).

Each environment may support multiple usage cases.

Each usage case may be specified in multiple channel models (e.g. LOS domestic, NLOS domestic) and will eventually reference one or more applications.

Applications will eventually be described in terms of a traffic profile of some kind.   TBD – we may lump together multiple use cases to create a synthetic traffic profile that is representative of all the activity on a particular device.

Each Simulation Scenario will consist of multiple STA, each of with specified position and channel model type to other STA.  Each STA is specified in terms of the applications it is running.

# Future Work

The output of the Usage Model special committee is the set of environments, applications, usage cases and simulation scenarios.

We probably can map the usage cases to simulation scenarios in time for the September meeting.

It is also necessary to describe each application in terms of its traffic profile so that the simulation scenarios unambiguously specify a traffic pattern.   This may take place within this group,  or more likely a “son of Usage Models” group operated under the authority of the 802.11n task group.

# Environments

The list of environments we are considering is shown in the following table.

| Environment | Includes | Applicable Channel Models |
| --- | --- | --- |
| Residential, Domestic or Home | Intra-room. Room to room. Indoor to outdoor ?? Large multi-family Dwelling?? | TBD - Vinko, can you add please? |
| Small Enterprise | Enclosed offices Meeting room / Conference Room Classroom |  |
| Large Enterprise | Enclosed offices Meeting room / Conference Room Classroom Sea of cubes Multi-story office environment Campus |  |
| Hotspot | Airport Library Convention Center Hotel Shopping Mall Train Station / bus terminal Drive-in window |  |
| Outdoor | Outdoor sport event Campus City Square Public Park |  |
| Industrial | Indoor Large factory floor Hospital Warehouse Concert Hall / Auditorium Movie Theatre |  |
| Other custom environments | Wireless Backhaul Fixed Wireless Access |  |
| Mobile | Train Bus Plane Curb-to-car (is this stationary?) Car-to-car (is this stationary?) |  |

# Usage Cases

Eventually, the entries in the ”Application” column should be drawn from a well-defined set of applications listed in the above mentioned “Applications” table.

TBD – should we represent interferers (e.g. 2.4GHz cordless phone, Bluetooth) as a “usage case”?

TBD – should we represent adjacent channel and out-of-band effects within our “usage case”s?

(e.g. DECT, 900 MHz DSSS)?

TBD – should we require that simulations use realistic analogue filters, or ideal filters?

| Number | Usage Case | Application | Environment |
| --- | --- | --- | --- |
|  | One personal phone everywhere – home, office, car. Each person has a phone that works everywhere, home, office, car – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small, Car, etc. |
|  | Multiplayer Internet gaming anywhere within the home / Internet Café. | Streaming Video and Audio? | Residential/small enterprise (internet cafes) |
|  | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | Streaming Video and Audio with some messaging for control | Residential |
|  | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | Streaming Video and Audio | Residential |
|  | Watch a movie of your choice, when you want it, it your hotel room. | Streaming Video and Audio | Hotel – TBD needs to be mapped into an existing environment |
|  | Watch a clear replay of an event from your seat in a sporting arena. | Streaming Video | Arena |
|  | Remotely located security cameras transmitting video signal to a monitoring location. | Streaming Video | Outside/Inside Residential, Small office building |
|  | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | Streaming Audio | Residential |
|  | Net meeting in a conference/class room to share someone’s display. 30 participants/students | File Transfer? Or White-board Applications | Conference room/class room |
|  | Reconfigurable/temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, etc. | File Transfer | Enterprise – sea of cubes |
|  | Download video, music and other data files to a device in an automobile in the home garage or driveway.Broadband file transfer – HT rates. | File Transfer | Residential / Outdoor |
|  | Backup/transfer files between PCs located throughout the home. Access point router. | File Transfer | Residential |
|  | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train |
|  | Download digital pictures and home movies to a PC/AV-server | File transfer | Residential – same room |
|  | Exchange files between PCs – ad hoc (no access point). Using IBSS mode of operation. | File Transfer | Residential, Enterprise |
|  | Update inventory from the warehouse and the retail floor. | File Transfer | Large open building. |
|  | Surfing the web, email, instant messaging, online gaming, content download all through a broadband connection to the home/hotel located remotely. | File Transfer | Residential/Hotel |
|  | Access of networked software from the classroom. 30 participants, simultaneously signing on. | File Transfer | Conference room/class room |
|  | Update/view medical records from patient rooms. | File Transfer | Large building, multiple walled rooms. |
|  | Obtain real time interactive player and game stats from your seat at a sporting event. | Content Downloading | Arena |
|  | Interactive multi-person gaming – ad hoc. | Streaming video and audio, file transfer | Car, home, train station, office  |
|  | Point-point link for wireless backhaul | Aggregated traffic | Arena? Hotspot? |
|  | Point-multipoint link for wireless backhaul | Aggregated traffic. Small number of links. | Arena? Hotspot? |
|  | Point-multipoint link for Fixed Wireless Access | Aggregated traffic. Large number of links controlled by geography | Outdoor |
|  | Mixed mode AP has legacy and HT STA | File transfer | Small Enterprise |
|  | Co-channel legacy BSS interference | File transfer | Small Enterprise |
|  | Legacy mode operation in legacy BSS | File transfer | Small Enterprise |
|  | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams | Streaming video (should probably be moved up to 9) | Hospital, same as 19. |
|  | Online distance learning/broadcasting locally | File Transfer, Streaming Audio/Video | Residential, small/large enterprises |
|  | Video conferencing with headset | Streaming Audo | Small Enterprise |
|  | Enterprise high stress | 25 users each demanding an average of at least 4 Mb/s of real bi-directional throughput. email, ftp, websurfing | Large Enterprise |

# Simulation Scenarios

The purpose of these scenarios is to merge representative use cases to create a small number (we agreed 5 was a nice small number) of simulation scenarios.  The scenarios have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some TBD subset of the use cases that we identify to be priorities.

At the moment, these are just examples!  They have to be validated against our use cases!

Eventually this table should link specific environments and usage cases and only use terms already defined in the above tables.

It is TBD whether the simulation scenario defines one scenario with fixed topology (e.g. wireless TV display at (5,4)m wrt AP),  or a probabilistic topology (e.g. TV display randomly within 20m).

Eventually, there will be enough information here to unambiguously simulate against.

| Scenario | Application mix |  |
| --- | --- | --- |
| Residential | 3 x (20Mbps) HDTV, 1 x VoIP phone, 3 x ftp clients, 1 x (128kbps) audio-stream |  |
| Small Enterprise | 1 AP, 6 STA. Mix of web, ftp, real-time audio and audio/video streaming (10-15Mbps) additional 4 STA low rate IP traffic (1. to 3 Mbps) Aggregate max at 100 Mbps. |  |
| Large Enterprise | 1 AP, 6 STA, cellular frequency re-use . Each STA doing email, web, ftp, real-time streaming audio and audio/video. Suggestion: 1 AP 4 STA each @ 20 Mbps plus (HT applications: BB file transfer, instantaneous database sync, server-based applications etc), additional 4 STA IP traffic 1-4Mbps (email, browsing) aggregate max at 100Mbps Also, see footnote number Error: Reference source not found. |  |
| Public park/Outdoor space | 1 AP, 50 STAs, cellular frequency reuse. Each station doing a mix of web, email and real-time audio/video streaming applications. (Note, offered load must exceed existing .11a rates to make this a valid scenario) | See comment to this simulation scenario |
| Mixed-mode BSS | 1 HT AP, 3 legacy, 3 HT STA. 1 streaming (20Mbps) HDTV,two other doing ftp uplink and ftp downlink |  |
| Co-channel legacy BSS | 1 legacy AP and 6 STA. 1 HT AP and 6 STA. Cochannel. All STA doing ftp uplink and ftp downlink. |  |

6File Transfer at HT (close to 100Mbps) should be a requirement for these applications. We may want to emphasize the differentiation between regular FTP and “Broadband FTP” (e.g. very short download time for audio files, “large” email message instantaneous sync, enterprise-class FTP etc)

This means that we collect the statistics off a “central” BSS, but surround it by identical BSSs that act solely as interferers at the distance appropriate to the frequency re-use plan implied by the band of operation and channel width used.