3GPP TSG-RAN1 Meeting #84bis 	R1-163502

- Busan, South Korea, 11th – 15th April

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 232 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPDCCH procedures for LAA FS 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-04-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Typo |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected reference to the right section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect reference in spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 6.8A	Enhanced physical downlink control channel

### 6.8A.1	EPDCCH formats

The enhanced physical downlink control channel (EPDCCH) carries scheduling assignments. An enhanced physical downlink control channel is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A. The number of ECCEs used for one EPDCCH depends on the EPDCCH format as given by Table 6.8A.1-2 and the number of EREGs per ECCE is given by Table 6.8A.1-1.Both localized and distributed transmission is supported.

An EPDCCH can use either localized or distributed transmission, differing in the mapping of ECCEs to EREGs and PRB pairs.

A UE shall monitor multiple EPDCCHs as defined in 3GPP TS 36.213 [4]. One or two sets of physical resource-block pairs which a UE shall monitor for EPDCCH transmissions can be configured. All EPDCCH candidates in EPDCCH set Xm use either only localized or only distributed transmission as configured by higher layers. Within EPDCCH set Xm in subframe i, the ECCEs available for transmission of EPDCCHs are numbered from 0 to NECCE,m,i−1 and ECCE number n corresponds to

-	EREGs numbered nmodNECCERB+jNECCERB in PRB index nNECCERB for localized mapping, and

-	EREGs numbered n/NRBXm+jNECCERB in PRB indices n+jmax1,NRBXm/NEREGECCEmodNRBXm  for distributed mapping,

where j=0,1,...,NEREGECCE−1, NEREGECCE is the number of EREGs per ECCE, and NECCERB=16/NEREGECCE is the number of ECCEs per resource-block pair. The physical resource-block pairs constituting EPDCCH set Xm are in this paragraph assumed to be numbered in ascending order from 0 to NRBXm−1.

Table 6.8A.1-1: Number of EREGs per ECCE, NEREGECCE

| Normal cyclic prefix |  |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Special subframe, configuration 1, 2, 6, 7, 9 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |  |

Table 6.8A.1-2: Supported EPDCCH formats

| EPDCCH format | Number of ECCEs for one EPDCCH, NECCEEPDCCH |  |  |  |
| --- | --- | --- | --- | --- |
|  | Case A |  | Case B |  |
|  | Localized transmission | Distributed transmission | Localized transmission | Distributed transmission |
| 0 | 2 | 2 | 1 | 1 |
| 1 | 4 | 4 | 2 | 2 |
| 2 | 8 | 8 | 4 | 4 |
| 3 | 16 | 16 | 8 | 8 |
| 4 | - | 32 | - | 16 |

Case A in Table 6.8A.1-2 is used when the conditions corresponding to case 1 in clause 9.1.4 of 3GPP TS 36.213 [4] are satisfied, otherwise case B is used. The quantity nEPDCCH for a particular UE and referenced in 3GPP TS 36.213 [4] is defined as the number of downlink resource elements (k,l) available for EPDCCH transmission in a physical resource-block pair configured for possible EPDCCH transmission of EPDCCH set X0 and fulfilling all of the following criteria:

-	they are part of any one of the 16 EREGs in the physical resource-block pair, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 unless other values for these parameters are provided by clause 9.1.4.3 in 3GPP TS 36.213 [4], and-

-	they are assumed by the UE not to be used for transmission of CSI reference signals, where the positions of the CSI reference signals are given by clause 6.10.5.2 with the configuration for zero power CSI reference signals obtained as described in clause 6.10.5.2 unless other values are provided by clause 9.1.4.3 in 3GPP TS 36.213 [4], and with the configuration for non-zero power CSI reference signals obtained as described in clause 6.10.5.2, and

-	for frame structure type 1 and 2, the index l in the first slot in a subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause 9.1.4.1 of 3GPP TS 36.213 [4], and

-	for frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates ‘s07’ and if the downlink transmission starts in the second slot of a subframe

-	the index l in the second slot in the subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause 7.1.6.4 of 3GPP TS 36.213  [4],

-	otherwise

-	the index l in the first slot in the subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause  7.1.6.4 of 3GPP TS 36.213  [4].