## LTE Release 13

R1-156964	Introduction of LAA	Ericsson LM

R1-157322	Introduction of Rel 13 feature of LAA	Huawei

R1-157492	Introduction of LAA (PHY layer aspects)	Motorola Mobility UK Ltd.

Revision of R1-156547

Endorsed.

R1-157493	LAA Carrier Access Procedure 213 Draft CR	Motorola Mobility UK Ltd.

Revision of R1-156620

Endorsed.

R1-157426	Introduction of LAA	Ericsson

Agreed in R1-157842 by changing “resource elements” to “resource blocks”

R1-157517	Introduction of LAA	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Agreements:

In Section 4 in TS36.211 CR, following is agreed.

Downlink, uplink and sidelink transmissions are organized into radio frames with Tf=307200×Ts=10 ms duration. 
Three radio frame structures are supported:

-	Type 1, only applicable to FDD,

-	Type 2, only applicable to TDD,

-	Type 3, only applicable to LAA secondary cell operation

R1-157743	Introduction of LAA	ALU

Endorsed as baseline for final CR

R1-157565	[DRAFT] LS on CR to 36.201 for Introduction of LAA	ALU

Agreements:

Agreed in R1-157744 with following update

RAN1 respectfully informs RAN2 that the attached draft CR to 36.201 has been endorsed as a baseline for the introduction of LAA. RAN1 provides this to RAN2 for information in case it may help RAN2 in their preparation of the corresponding Stage 2 CR for 36.300.

RAN1 does not expect RAN2 to make changes to their agreed CRs.

RAN1 leaves the final choice of terminology in RAN2 specs up to RAN2.

R1-157745	Introduction of LAA	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Agreed in R1-157899 (Rev. 3) with following frame structure type 3 descriptions

The radio frame structure type 3 is only applicable to LAA secondary cell operation. It has a duration of 10ms and consists of 20 slots with a slot duration of 0.5ms. Two adjacent slots form one subframe of length 1ms. Any subframe may be available for downlink transmission, and the eNB shall perform the channel access procedures as specified in [4] prior to transmitting. A downlink transmission may or may not start at the subframe boundary, and may or may not end at the subframe boundary.

R1-157900	LS on CR to 36.201 for Introduction of LAA	RAN1

R1-157890	Introduction of Rel-13 feature of LAA	Huawei

Agreed in R1-157898 (Rev.2) with following update

Else

- Subframe configuration for LAA – 4 bits as defined in section x.y.z of [3]

Reserved information bits are added until the size is equal to that of format 1C used for very compact scheduling of one PDSCH codeword

R1-157901	Channel access procedures for LAA	Motorola Mobility

Endorsed.

Email approval of LS to RAN4 about R1-157901 (Sorour (Ericsson))

Email approval of Rel-13LAA 36.211 and 36.213 CRs until 30th Nov. (editor will provide draft CR until 24th Nov.)

Companies are encouraged to have early comments for above CRs in the email discussion.

### Licensed-Assisted Access to Unlicensed Spectrum

WID in RP-151045.

R1-157622	LS on L1 parameters for LAA	Ericsson, Huawei

Agreed in R1-157705

Conclusion:

No more RRC signaling in Rel-13 LAA in addition to R1-157705

Email approval of LS to RAN2/RAN4 about the RAN1 agreements until 23rd Nov. (Havish (Ericsson)/Xia Yuan (Huawei)

#### Channel access framework for LAA

Remaining details of LBT procedure and parameter sets

R1-157627	WF on LAA DL LBT ED Threshold Adaptation Rule		Cisco Systems, Broadcom, CableLabs, Ruckus Wireless

Intel can support it except for Y value

R1-157626	WF on LAA DL LBT ED Threshold Adaptation Rule (Functionality of Detecting On-going 802.11 a/n/ac tx)	Cisco Systems, Broadcom, CableLabs, Ruckus Wireless

R1-157641	WF on ED Threshold Adaptation	Ericsson

R1-157531	On Energy Detection Threshold for LAA	Ericsson

Revision of R1-157255

R1-157625	WF on Identifying LAA eNB Coexistence Capabilities	Cisco Systems, Broadcom, AT&T, CableLabs, BlackBerry, Orange, Intel, Ruckus Wireless

R1-157555	WF on Regulation and MCOT	Ericsson, Intel, CATT,

R1-157556	WF on MCOT and CWS for LBT priority classes	Ericsson, ALU, ASB,

R1-157628	WF on MCOT for LBT priority classes	Cisco Systems, Broadcom, CableLabs, BlackBerry, Orange

R1-157875	Status of LBT Priority Class Proposals in LAA	BlackBerry

Agreements:

- If a DL transmission burst with PDSCH is transmitted, for which channel access has been obtained using LBT priority class X (1...4), the eNB shall ensure that:

  - The transmission duration shall not be longer than the minimum possible duration needed to transmit all available buffered traffic corresponding to LBT priority classes ≤ X

  - The transmission duration shall not be longer than the MCOT for priority class X

  - Additional traffic corresponding to LBT priority classes >X may only be included in the DL transmission burst once inclusion of traffic corresponding to LBT priority classes ≤ X has been exhausted.  In such cases, the eNB should maximise occupancy of the remaining transmission resources in the DL transmission burst with this additional traffic.

- The above requirements shall be captured within RAN2 specifications.

Agreements:

- Rel-13 LBT supports four LBT priority classes in the RAN1 specifications

- For the LBT priority classes 1 to 4, LAA supports the LBT parameter values shown in the table below

| LBT priority class | CWmin | CWmax | n | MCOT | Set of CW sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 7 | 1 | 2ms | {3,7} |
| 2 | 7 | 15 | 1 | 3ms | {7,15} |
| 3 | 15 | 63 | 3 | 10 or 8 ms | {15,31,63} |
| 4 | 15 | 1023 | 7 | 10 or 8 ms | {15,31,63,127,255,511,1023} |

Agreements:

- If the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), the maximum channel occupancy time for LBT priority classes 3 and 4 is for 10 msec, otherwise, maximum channel occupancy time for LBT priority classes 3 and 4 is for 8 msec

Agreement:

- No more than one random backoff counter per carrier is used by eNB

Ageements:

- The LBT parameters used can only be modified at the start of an extended CCA operation requiring the generation of a new random backoff counter.

- CW size is maintained and updated per LBT priority class

- For adjustment of the contention window sizes per carrier the following are supported:

  - The outcome of the HARQ-based CW adjustments, i.e. increase or reset the CW size, is applied to all the LBT priority classes

Agreements:

- Note: In Japan on 5 GHz unlicensed carrier, the maximum continuous transmission time after sensing the channel to be idle is limited to 4ms by regulation.

- For LAA operation in Japan, when the DL transmission burst is greater than 4 ms, the eNB may do the following:

  - After the initial 4 ms of the DL transmission burst, the eNB can sense the channel to be idle for a single continuous interval of duration 34 µs before immediately continuing its transmission with the maximum continuous transmission time limit.

    - The total sensing time and the transmission time should not be more than the  MCOT + floor(MCOT/4ms )*34 µs  where MCOT is the maximum channel occupancy time corresponding to the LBT priority class used for accessing the channel.

- Note that if Japan regulation will change, above agreements needs to be changed

R1-157670	WF on MCOT for LBT priority classes	Samsung, Intel, Cisco, Broadcom, CableLab

R1-157639	WF on Contention Window Adaptation	Intel

Agreements:

- Confirm following working assumption as an agreement with the FFS point removed.

- Working assumption:

  - For LBT operation for PDSCH, the CW size (CWS) is adjusted based on HARQ ACK/NACK feedback.

- The eNB considers HARQ-ACK feedback (ACK, NACK, or DTX) only for the self-scheduled TB(s) with DTX and ‘any’ state (defined for TDD channel selection case) counted as NACK

- Do not consider “Whether the CWS is reset to the minimum value if there has been no DL transmission by the eNB for a duration of at least T.”

R1-157711	WF on HARQ-ACK based contention window size adaptation	Huawei, HiSilicon, CATT, DCM, CMCC, CATR

R1-157833	WF on Contention Window Adaptation	Intel

Agreements:

  - The CWS is increased if at least 80 % of the HARQ-ACK feedback values for a reference subframe set are NACK. Otherwise, the CWS is reset to the minimum value.

    - Reference subframe set: the first DL subframe of the latest DL data burst for which HARQ-ACK feedback is available.

    - Working assumption: Initial partial subframe in addition to the first full DL subframe when initial partial subframe is transmitted

R1-157550	WF on HARQ-ACK based contention window size adjustment		LG Electronics, WILUS, Intel

Agreements:

- For HARQ-ACK based CWS adjustment, following rules should be adopted.

  - HARQ-ACK DTX is considered as NACK with the exception of following cases.

    - HARQ-ACK DTX from a non-scheduled TB is ignored.

    - HARQ-ACK DTX from the TB which is scheduled by licensed carrier is ignored.

    - Note: HARQ-ACK DTX means explicit DTX response or implies that eNB does not detect any HARQ-ACK response corresponding to a PDSCH transmitted in an unlicensed carrier.

  - HARQ-ACK response(s) for two codewords transmitted on a PDSCH is(are) considered separately per each codeword.

  - Bundled HARQ-ACK across M subframes is considered as M HARQ-ACK responses.

R1-157013	Remaining Details of Single-Carrier LBT	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

Agreements:

- If the absence of IEEE802.11n/11ac devices sharing the carrier can not be guaranteed on a long term bases (e.g., by level of regulation),

  - For Rel13, if the maximum number of unlicensed channels that the LAA eNB can simultaneously transmit on is equal to or less than 4

    - It is strongly recommended that maximum frequency separation between any two channel centers is less than or equal to 62MHz

- Capture the above in 36.300

Agreements:

- For multi-Carrier LBT on a group carriers

  - Alt1: eNB performs Cat-4 based LBT on only one unlicensed carrier

    - The eNB shall choose the carrier requiring Cat-4 based LBT uniformly randomly before each transmission burst or fix the carrier at least for 1 sec

    - The energy detection threshold used on channels not performing Cat-4 based LBT is same as the one used on channel performing Cat-4 LBT

    - A single backoff counter should be used for the carrier on which Cat-4 LBT is performed.  Working assumptions: eNB can use the following Option 1 or Option 2:

      - Option 1: One CW size is updated based on HARQ feedback for all the carriers.

      - Option 2: CW size is updated independently per carrier. The largest CW size among the carriers is used to draw the backoff counter.

  - Alt2: eNB performs Cat-4 based LBT on more than one unlicensed carriers

    - The eNB is allowed to transmit DL data burst(s) on the carriers that has completed Cat-4 based LBT with potential self-deferral (including idle sensing for a single interval) to align transmission over multiple carriers.

    - eNB can perform CW window update independently for Cat-4 based LBT on more than one unlicensed carriers

      - eNB can use independent backoff counters or can use a common backoff counter for multiple carriers.

        - When a common backoff counter is used, the common random number shall be drawn based on the largest CW size of the carriers which were used in prior transmission

      - If the absence of any other technology sharing the carrier cannot be guaranteed on a long term basis (e.g. by level of regulation)

        - If a common random counter is used, then after transmission of DL data burst(s) over one or multiple carriers, reset the extended CCA counter(s) for all channel(s) on which LBT was performed

        - If independent random counters are used, after the transmission of DL data burst(s) over one or multiple carriers, do one of the following

          - Wait for a duration of 4 CCA slots before resuming countdown of counters for all the activated LAA Scell(s) for the UE(s) which were scheduled in the burst

          - reset the counters for all the activated LAA Scell(s) for the UE(s) which were scheduled in the burst

          - Note: The decision between the two options above can be made by the eNB dynamically (e.g. after drawing a random number) on a per carrier basis

  - LAA supports both Alt. 1 and Alt. 2

R1-157602		WF on LAA DL Multi-carrier Allocation Limitations CableLabs, Broadcom Corporation, Cisco Systems, Ruckus Wireless

R1-157643	WF on some details on LBT Intel

Agreements:

For the DRS without PDSCH transmission LBT procedure used by LAA:

- If the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), the maximum energy detection threshold used by LAA is

  - Min(X, Y)

    - X = -75 dBm/MHz+ 10*log10(BWMHz) + 10 dB

    - Y =

      - Maximum energy detection threshold defined by regulatory requirements when such requirements are defined

      - Y = X, otherwise

- Otherwise, LAA uses the maximum energy detection threshold of TH;

  - TH = max(-72 dBm (20MHz), min(Tmax, Tmax – 5 dB + (PH – PTX)))

- If an LAA eNB uses channel access signals of other technologies for the purpose of LAA channel access, it shall continue to meet the LAA maximum energy detection threshold requirement. (Captured in 36.300)

- The parameters used in the above proposal are defined as the following:

  - PH is a reference power equaling 23 dBm,

  - PTX is the configured maximum transmit power for the carrier in dBm

    - Uses the configured maximum transmission power over a single carrier irrespective of whether single carrier or multi-carrier transmission is employed

  - Tmax is defined as the following:

    - Tmax = -75 dBm/MHz+ 10*log10(BWMHz).

    - BWMHz is the channel bandwidth in MHz

R1-157642	WF on the possible use of FD-MIMO with LAA 	Mediatek

R1-157658	WF on the possible use of FD-MIMO with LAA	Mediatek, Broadcom, Cablelabs

R1-157644	WF on ED Threshold for LAA Frequency Reuse	Samsung

R1-157708 	WF on ED Threshold for LAA Frequency Reuse	Samsung, ZTE

Agreements:

For the category 4 LBT procedure used by LAA:

- If the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), the maximum energy detection threshold used by LAA is

  - Min(X, Y)

    - X = -75 dBm/MHz+ 10*log10(BWMHz) + 10 dB

    - Y =

      - Maximum energy detection threshold defined by regulatory requirements when such requirements are defined

      - Y = X, otherwise

- Otherwise, LAA uses the maximum energy detection threshold of TH;

  - TH = max(-72 dBm (20MHz), min(Tmax, Tmax – 10 dB + (PH – PTX)))

- If an LAA eNB uses channel access signals of other technologies for the purpose of LAA channel access, it shall continue to meet the LAA maximum energy detection threshold requirement. (Captured in 36.300)

- The parameters used in the above proposal are defined as the following:

  - PH is a reference power equaling 23 dBm,

  - PTX is the configured maximum transmit power for the carrier in dBm

    - Uses the configured maximum transmission power over a single carrier irrespective of whether single carrier or multi-carrier transmission is employed

  - Tmax is defined as the following:

    - Tmax = -75 dBm/MHz+ 10*log10(BWMHz).

    - BWMHz is the channel bandwidth in MHz

Conclusion:

For multi-carrier operation, recommended modes of operation can be captured in 36.300.

Agreements:

For initial CCA,

- If an eNB has not transmitted a transmission including PDSCH on a LAA Scell after step 4 in the procedure below, the eNB may transmit a transmission including PDSCH on the LAA Scell, after sensing the channel to be idle at least in the slot durations of an additional defer duration, Td.

- Modify next paragraph as follows

  - The eNB may transmit a transmission including PDSCH on the LAA Scell, after sensing the channel to be idle during the slot durations of a defer duration, Td ; and after the counter  is zero in step 4 of the procedure below.

Agreements:

For defer period,

- The defer duration Td  consists a fixed duration Tf = 16 us immediately followed by  m consecutive slot durations where each slot duration is Ts = 9us, where the fixed duration of Tf = 16us includes an idle slot duration, Ts, at the start of Tf.

R1-157874	WF on ED threshold for LAA only scenario	Huawei, HiSilicon, LGE, ZTE

R1-157172	On the LAA ED threshold	Intel Corporation

R1-156622	Experimental Results on Impact of Energy Detection Threshold for DL LAA	National Instruments Corp.

R1-156761	Remaining details of HARQ-ACK based CWS adaptation	Samsung

R1-156438	Parameters for ACK/NACK-based contention window size adaptation	Huawei, HiSilicon

R1-157009	Further Discussion on LAA DL Multi-channel LBT	Broadcom Corporation

R1-157258	On Channel Access Solutions for LAA Multi-Carrier Transmission	Ericsson

R1-157379	Downlink LBT Priority Classes	BlackBerry UK Limited

R1-156983	Considerations on LAA-LTE UL Transmissions without LBT	CableLabs

R1-156858	UL LBT design in LAA	LG Electronics

R1-156621	Coexistence simulation results for DL only LAA	Wi-Fi Alliance

R1-156437	Adaptation rules of energy detection threshold	Huawei, HiSilicon

R1-156489	Views on Energy Detection Threshold Adaptation 	Cisco Systems France

R1-156490	Views on Multi-Channel Access for LAA	Cisco Systems France

R1-156510	Remaining details on LBT	Intel Corporation

R1-156511	Remaining details on HARQ feedback based CW adaptation	Intel Corporation

R1-156574	Remaining details of DL LBT for LAA	CATT

R1-156575	Design of UL LBT for LAA	CATT

R1-156576	DL LBT for LAA multi-carrier transmission	CATT

R1-156577	Support of higher frequency reuse for LAA DL	CATT

R1-156701	LAA CW size adjustment methodologies	Sony Corporation

R1-156702	LAA energy detection adaptation rules	Sony Corporation

R1-156733	Contention window size adaptation for LAA	Lenovo (Beijing) Ltd

R1-156762	ED threshold adaptation for LAA	Samsung

R1-156763	LAA multi-channel LBT	Samsung

R1-156764	Further discussion on frequency reuse support in LAA	Samsung

R1-156856	DL LBT design in LAA	LG Electronics

R1-156857	Multi-carrier DL TX in LAA	LG Electronics

R1-156914	Remaining LBT parameters for LAA DL	Huawei, HiSilicon

R1-156981	Limitations of Multi-carrier Allocation Scheduling	CableLabs

R1-156982	CW size considerations for UL LAA-LTE	CableLabs

R1-156984	Contention Window Adaptation based on HARQ-ACK feedback	ZTE

R1-156985	Discussion on remaining issues of  DRS LBT design	ZTE

R1-156986	Frequency Reuse on Unlicensed Carrier	ZTE

R1-156993	Discussion on the UL LBT for LAA	ZTE

R1-156994	UL framework for LAA	ZTE

R1-157010	Discussion on remained issues of LAA LBT Category 4 Design for DL	Broadcom Corporation

R1-157011	Further Discussion on LAA UL LBT Design	Broadcom Corporation

R1-157014	Multi-carrier LBT operation for LAA	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

R1-157036	Adaptive ED threshold setting	Qualcomm Inc.

R1-157037	Remaining details of UL LBT operation	Qualcomm Inc.

R1-157038	Remaining details of window adaptation trigger mechanism	Qualcomm Inc.

R1-157039	Multi-carrier LBT operation	Qualcomm Inc.

R1-157099	Remaining details of CWS adjustment based on HARQ-ACK feedback	ETRI

R1-157113	HARQ-ACK based LAA contention window size adjustment	Sharp

R1-157114	Transmit timing alignment for LAA frequency reuse	Sharp

R1-157130	On LAA DL LBT design enabling frequency reuse	Nokia Networks

R1-157131	Subframe type coordination for forward compatible DL/UL operation	Nokia Networks

R1-157166	LAA LBT design	Mediatek Inc.

R1-157167	Directional transmission and channel access	Mediatek Inc.

R1-157219	Views on contention window size adaptation mechanisms	NTT DOCOMO, INC.

R1-157220	Discussion on channel access framework for potential LAA UL	NTT DOCOMO, INC.

R1-157256	Details of Contention Window Size Adjustment Based on HARQ Feedback	Ericsson

R1-157257	On Remaining Details of DL LAA LBT Procedures and Parameters	Ericsson

R1-157259	On the Number of UEs for DL-only LAA Using a Bandwidth of 80 MHz	Ericsson

R1-157260	Coexistence Performance of DL-only LAA Networks Using a Bandwidth of 80 MHz	Ericsson

R1-157261	Coexistence Evaluation Results for Video Conference Services	Ericsson

R1-157281	Discussion on adaptation rules of the maximum energy detection threshold in LAA coexistence	HTC

R1-157305	CWS adaptation for DL LAA LBT	Nokia Networks

R1-157306	Multi-carrier LBT for DL LAA	Nokia Networks

R1-157329	Remaining issues on CWS adaptation for LBT on LAA	WILUS Inc.

R1-157330	Consideration on Multicarrier LBT for LAA	WILUS Inc.

#### RRM measurements and reporting

DRS design and UE support for RRM measurements

R1-156766	Remaining details of UE-reported RSSI measurement	Samsung

R1-157143	On LAA RSSI measurement	ZTE Corporation

R1-157213	Further details on new RSSI measurement Framework	Motorola Mobility

Agreement:

- No OFDM symbol offset for UE reported RSSI measurement is specified

Working assumptions:

- “measurement duration” for RSSI

  - Note: values 1 and 70  (in units of L1 averaging duration) are already supported

  - value 14 is also supported

Agreements:

- Parameter “Subframe offset for UE reported RSSI measurement duration” is made optionally configurable for inter-frequency measurement

  - When the parameter is configured

    - UE measures according to the configured offset

  - When the parameter is not configured

    - For inter-frequency measurements, UE [should be allowed to flexibly] chooses a random starting offset for “RSSI measurement duration”

Agreement:

- Periodicity of UE RSSI measurement duration has a values of 40, 80, 160, 320, 640 msec

Agreements:

- L1 averaging duration of UE-reported RSSI measurement is 1 OFDM symbol

  - Note: Above agreements override past agreements

  - L1 averaging duration of UE-reported RSSI measurement does not need to be configured to UE

- Value range of Measurement duration of UE-reported RSSI measurement is 1, 14, 28, 42, 70 (in unit of L1 averaging duration)

R1-157621	WF on UE-reported RSSI	Samsung, Intel

Agreements:

- L1 provides to higher layer a single RSSI sample value for every “L1 averaging duration of UE-reported RSSI measurement” in a “Measurement duration of UE-reported RSSI measurement”

  - No additional L1 filtering is applied

- Example note

  - L3 averaging window is only for an example illustration

  - L1 averaging duration = 1 OFDM symbol, UE-reported RSSI measurement duration = 70

Agreement:

- No additional UE behaviour is required when a RSSI measurement instance overlaps with the transmission of the serving cell

R1-157577	WF on DRS transmission	LG Electronics, Nokia Networks, Huawei, HiSilicon

R1-156578	Remaining details of LAA DRS design	CATT

R1-156997	Further discussion on DRS design	CMCC

R1-156440	Remaining details of DRS design for LAA	Huawei, HiSilicon

R1-156513	On transmission and sequence generation of PSS/SSS/CRS for LAA	Intel Corporation

R1-156579	Remaining details of LAA RSSI measurement	CATT

R1-156765	Remaining details of DRS design	Samsung

R1-156859	RRM measurement in LAA	LG Electronics

R1-156860	DRS design in LAA	LG Electronics

R1-157015	Remaining Details of RRM Measurement for LAA	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

R1-157040	DRS design details	Qualcomm Inc.

R1-157041	Discovery and RRM procedure for LAA	Qualcomm Inc.

R1-157042	Procedures for PLMN ID detection	Qualcomm Inc.

R1-157100	Remaining details of DRS design for LAA	ETRI

R1-157115	Multiplexing of DRS and PDSCH for LAA	Sharp

R1-157132	Remaining details on DRS Occasion Design for LAA	Nokia Networks

R1-157142	Discussion on remaining issues of LAA DRS	ZTE Corporation

R1-157168	DRS-PDSCH Multiplexing in a DL Transmission burst	Mediatek Inc.

R1-157221	Remaining details on DRS transmission and RRM measurement for LAA	NTT DOCOMO, INC.

R1-157222	Discussion on inter-frequency RSSI measurement for LAA	NTT DOCOMO, INC.

R1-157262	Remaining Details on RRM Measurements and Reporting for LAA	Ericsson

R1-157263	On DRS and PSS/SSS Transmissions	Ericsson

R1-157307	Remaining details of LAA RRM and RSSI measurements	Nokia Networks

R1-157331	Consideration on Multiplexing DRS and PDSCH for LAA	WILUS Inc.

#### Downlink Transmissions

Discontinuous transmissions including initial signal, transmission modes, transmission bursts, PDSCH transmission techniques including modifications to TTI, Scheduling and other details of DL transmissions

R1-157045	Remaining details of physical layer signaling	Qualcomm Inc.

R1-157267	On Signaling in DL LAA Subframes	Ericsson

R1-157215	Further details on UE operation and related signalling for LAA	Motorola Mobility

R1-157296	Starting and Ending positions for DL discontinuous transmission	Innovative Technology Lab Co.

Agreements:

- RRC signaling provides the list of possible starting positions of transmission in the first subframe of the DL transmission burst

  - Possible starting positions are {0}, {0,7}. Other starting positions are not considered

  - Candidate starting positions in the first subframe are excluding the reservation signal and initial signal (if any)

R1-157552	WF on LAA starting partial subframe design	Huawei, HiSilicon

Agreements:

- Possible starting positions are {0}, {0,7}.

- When the starting position is 7,

  - Same CRS/DMRS resource mapping as in the second slot

    - All the configured CRSs are included in the partial subframe

  - If the configured CSI-RS/CSI-IM pattern is in the first slot

    - UE assumes CSI-RS/CSI-IM does not exist in the partial subframe

  - If the configured CSI-RS/CSI-IM pattern is in  the second slot

    - UE assumes CSI-RS/CSI-IM does not exist in the partial subframe

  - PDCCH/PDSCH/PCFICH/PHICH resource mapping as in the first slot of LAA regular subframe

    - UE does not rate match around PSS/SSS REs in the partial subframe in subframe 0/5

    - PSS/SSS is not contained in the partial subframe

  - Slot number of the second slot is used for sequence generation

  - UE does not assume to receive PHICH for ack/nack

Agreements:

  - EPDCCH resource mapping as in the first slot of LAA regular subframe

  - In initial partial subframe, the start symbol for the EPDCCH if configured, is offset additionally by 7 OFDM symbols

  - The number of available REs for the EPDCCH is the actual number of available REs for the EPDCCH transmission in the initial partial subframe

Agreements:

- RRC signaling indicates which subframe has one or two symbol CRS structure or all symbols CRS structure

  - Up to 8 subframes (i.e., exclude 0th and 5th subframe in the frame) can be configured as MBSFN per frame

  - Note: DRS can be transmitted in either of the above subframe type

  - Note: This is not intended to preclude DRS and PDSCH multiplexing options

  - MBSFN configuration is not applied to a partial subframe

R1-157804	Way Forward on signaling the remaining number of expected DL subframes	Qualcomm, ZTE, Ericsson

R1-157846	WF on LAA signaling design	Mediatek

Agreements:

- If the UE detects common PDCCH DCI referring to subframe n in subframes n-1 or n, the UE may assume the number of OFDM symbols in subframe n according to the detected DCI

- If the UE does not detect common PDCCH DCI in subframe n and the UE does not detect common PDCCH DCI in subframe n-1, the UE is not required to use the subframe n for updating the CSI measurement

| 0 | Next subframe is 3 OFDM symbols |
| --- | --- |
| 1 | Next subframe is 6 OFDM symbols |
| 2 | Next subframe is 9 OFDM symbols |
| 3 | Next subframe is 10 OFDM symbols |
| 4 | Next subframe is 11 OFDM symbols |
| 5 | Next subframe is 12 OFDM symbols |
| 6 | Next subframe is full (14 Symbols) |
| 7 | Current subframe is partial 3 OFDM symbols |
| 8 | Current subframe is partial 6 OFDM symbols |
| 9 | Current subframe is partial 9 OFDM symbols |
| 10 | Current subframe is partial 10 OFDM symbols |
| 11 | Current subframe is partial 11 OFDM symbols |
| 12 | Current subframe is partial 12 OFDM symbols |
| 13 | Current subframe is full (14 Symbols) and end of transmission |
| 14 | Reserved |
| 15 | Reserved |

Agreement:

- DCI format 1C is used to indicate this LAA common signaling

R1-157549	Way Forward on Signaling in LAA DL Subframes		Qualcomm

R1-157559	Way Forward on Signaling in LAA Downlink Subframes	Qualcomm, Kyocera

Agreements:

- If the end subframe is partial subframe, then the end partial subframe configuration of a DL transmission burst is indicated to the UE in the end subframe and the previous subframe

  - Note: The UE may expect that the information signaled in both the above subframes is consistent

  - If the UE receives an indication of an end partial subframe in the current subframe but does not receive this signaling in the previous subframe, then the UE is not required to further process the subframe

- If the end subframe is a full subframe, then such signaling may or may not be present

  - Note: This is to be resolved based on further discussion on common signaling

- In a DL transmission burst, the UE may assume that an end partial subframe does not immediately follow an initial partial subframe

Agreements:

- The linkage between periodic CSI-RS for channel measurement and CSI-RS configured in DRS for RRM is not introduced

  - No RRC impact

  - Periodic CSI-RS for channel measurement and CSI-RS configured in DRS for RRM are independently configured

    - CSI-RS configured as part of DRS is not enhanced and utilize existing signaling/configuration

Agreements:

- The linkage between periodic CSI-RS for channel measurement and the subframe in which the DRS is transmitted is not introduced

  - No RRC impact

  - Periodic CSI-RS configured for CSI measurement is not transmitted in the last 2 OFDM symbols for the subframe containing DRS without PDSCH

Agreements:

- For indicating the end partial subframe configuration,

  - Common DL information is carried by common PDCCH on an unlicensed carrier, the following design is used

  - Even if UE is configured with cross-carrier scheduling, it should detect the common PDCCH on unlicensed carrier

  - Information bit width of PDCCH format 1C or 1A is reused

  - The common DCI consists of:

    - X information bits

    - Y reserved bits

    - 16 CRC bits

  - If other common signaling is introduced, the same PDCCH DCI is reused

Agreements:

- For common DL information carrying common PDCCH on an unlicensed carrier, the following design is used

  - The first PDCCH candidate in a PDCCH search space corresponding to aggregation level 4 and 8is used

  - Single RNTI value is applied for the common PDCCH

    - RAN2 may decide the RNTI value

  - For the information bitwidth, decide between DCI format 1C and 1A after all the contents of common PDCCH are decided

R1-157690	Way Forward on Transmitting Common Control Signaling on PHICH	ALU, ASB, Ericsson

Agreements:

- Confirm the working assumption

  - UE may assume that the CRS and CSI-RS transmission has a constant power in each subframe within a DL transmission burst, and the UE shall not assume that the CRS or CSI-RS transmission power is the same across transmission bursts

- Continue to discuss whether there is any problem with AGC

R1-157691	WF on CRS and CSI-RS power indication	Huawei, HiSilicon

R1-157620	WF on LAA channel measurement	Huawei, HiSilicon, ALU, ASB, ZTE, Nokia Networks, Qualcomm

Also supported by Intel, Kyocera, DOCOMO

Agreements:

- For CSI channel measurement,

  - UE should not average CRS/CSI-RS measurements across transmission bursts

  - It is up to UE implementation whether to average CRS/CSI-RS measurements across subframes or not

R1-157579	WF on CSI measurement in LAA	LG Electronics, Sharp, Samsung

Also supported by Ericsson, Lenovo, Motorola Mobility

Agreements:

For aperiodic and periodic CSI reporting,

- For CSI report in subframe n, UE’s CSI channel measurement is restricted within the transmission burst to which the CSI reference resource belongs

- For CSI report in subframe n, CSI reference resource in time domain is defined as subframe n-nCQI_ref

- Subframe n-nCQI_ref is latest valid subframe which satisfies nCQI_ref  X

  - X follows existing specifications (e.g., 4 as default, 5 when multiple CSI processes are configured, etc.)

- Definition of valid subframe in addition to the current definition of valid subframe:

  - A subframe is not valid subframe if it is not part of a transmission burst

  - For a UE configured with TM9 or TM10, a subframe is not valid subframe if it doesn’t consist of CSI-RS configured for CSI measurement

  - Initial partial subframe and end partial subframe are not valid subframe

  - Outside a configured DMTC, a subframe where the CRS scrambling sequence doesn’t follow corresponding subframe index is not valid subframe

  - Within a configured DMTC, a subframe where the CRS scrambling sequence doesn’t follow corresponding subframe index is not valid subframe

R1-157551	WF on end of DL TX burst in LAA	LG, Sharp, ITL, Samsung

Agreements:

- In LAA SCell, DwPTS structure with at least {3,6,9,10,11,12} OFDM symbols is supported for the last subframe of a DL TX burst.

Agreement:

- Clarification: LAA only supports normal CP

Conclusion:

- The decision on whether CSI-RS/IM is supported or not in DwPTS does not have any RRC impact.

R1-157692	Way forward on TBS determination in initial partial subframes	Panasonic

R1-157803	Way forward on TBS determination in initial partial subframes	Panasonic

No consensus in RAN1

Conclusion:

- UE determines the TBS size for starting partial subframe in the same way as in full subframe

R1-157693	Way Forward on signaling the number of subframes after the duration during which UE does not monitor DL transmission	Qualcomm, Ericsson, Nokia Networks

R1-157528	LAA initial signal design	Mediatek

Agreements:

- On an LAA SCell, DRS and other downlink channels including PDSCH/(E)PDCCH can be multiplexed and transmitted within a subframe, at least in subframes 0 and 5, where the PDSCH is part of the transmission and the eNB follows the Cat4 LBT scheme defined for PDSCH

  - Note: This clarifies part of a previous working assumption

Agreements:

  - DRS cannot be multiplexed with EPDCCH/PDSCH/PDCCH in subframes other than subframe 0 or 5.

- Scrambling sequences of PSS/SSS/CRS/CSI-RS composing DRS are generated using subframe index 0 when transmitted in subframe 0~4, and using subframe index 5 when transmitted in subframe 5~9.

  - Note: This applies to CSI-RS configured as part of the DRS. CSI-RS configured for CSI measurements follows the slot index.

Agreements:

- PSS/SSS is not transmitted in SF0 and SF5 if the number of transmitted OFDM symbols in the subframe is less than 12 OFDM symbols

- UE should assume that the configured CSI-RS/IM resources are not present in partial starting subframes.

Agreements:

- UE should assume that the configured CSI-RS/IM resources are not present in partial ending subframes

R1-157578	WF on scheduling in LAA	LG Electronics, Sharp, ITL,

Agreements:

- Cross-carrier scheduling is supported in LAA Scell DL

  - A UE is not expected to be configured with an unlicensed carrier to schedule another unlicensed carrier or licensed carrier

  - For the cross-carrier scheduling, if UE receives DL grant for LAA SCell, the UE can assume the corresponding PDSCH is present, which is same as Rel-12 UE behavior

  - A UE is not expected to be cross-carrier scheduled for an unlicensed carrier with an initial partial subframe in the unlicensed carrier

Agreement:

- If the UE is configured with CSI-RS for RRM using DRS, the UE assumes the presence of CSI-RS for RRM in first detected subframe containing PSS/SSS/CRS within DMTC

R1-156439	LAA cell detection and synchronization for support of partial subframe	Huawei, HiSilicon

R1-156514	Performance evaluation of CRS presence detection	Intel Corporation

R1-156515	On the starting and ending positions for DL LAA burst	Intel Corporation

R1-156516	On the LAA DL signaling	Intel Corporation

R1-156517	Further design details on partial subframe	Intel Corporation

R1-156546	DL Transmission Design for partial subframe	Kyocera Corporation

R1-156580	Utilization of first partial subframe in DL transmission burst	CATT

R1-156581	Utilization of ending partial subframe in DL transmission burst	CATT

R1-156582	Scheduling methods for LAA DL	CATT

R1-156675	Discussion on LAA DL transmission	NEC

R1-156676	Discussion on downlink scheduling for LAA	NEC

R1-156677	Discussion on LAA synchronization	NEC

R1-156703	DL power variation for LAA	Sony Corporation

R1-156722	Signalling for 1/2 or 4/6 OFDM symbol CRS	NEC Corporation

R1-156734	DL scheduling for LAA Scells	Lenovo (Beijing) Ltd

R1-156767	LAA control signaling details	Samsung

R1-156768	LAA UL LBT	Samsung

R1-156769	Impact of LAA dynamic power control	Samsung

R1-156861	DL synchronization in LAA	LG Electronics

R1-156862	Reservation signal in LAA	LG Electronics

R1-156863	Starting partial TTI structure in LAA	LG Electronics

R1-156864	Ending partial TTI structure in LAA	LG Electronics

R1-156865	Layer 1 signaling in LAA	LG Electronics

R1-156866	DL scheduling and control signaling in LAA	LG Electronics

R1-156904	Downlink signaling for LAA transmission burst related information	Huawei, HiSilicon

R1-156905	LAA synchronization signals for one-shot detection	Huawei, HiSilicon

R1-156906	CCA measurement to support frequency reuse	Huawei, HiSilicon

R1-156915	Discussion and comparison between partial subframe and floating subframe	Huawei, HiSilicon

R1-156958	Indication methods for Partial subframe	Panasonic

R1-156959	Impact of LAA Beamforming on LBT Behaviour	Panasonic

R1-156960	TBS Selection for Downlink Partial Subframes	Panasonic

R1-156961	Frequency scheduling options for unlicensed carriers	Panasonic

R1-156962	Self-scheduling and cross-scheduling options for unlicensed carrier access	Panasonic

R1-156987	LAA cell detection and synchronization	ZTE

R1-156988	Details of Design on DL frame structure and signaling for LAA	ZTE

R1-156989	Discussion on Scheduling Combinations for LAA	ZTE

R1-156998	Further discussion on Partial Subframe Transmission	CMCC

R1-157016	Support of Initial Partial Subframe in LAA	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

R1-157017	Downlink control signaling in LAA	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

R1-157043	Remaining details of subframe types	Qualcomm Inc.

R1-157044	Adaptive frame structure and DL-UL configuration for LAA	Qualcomm Inc.

R1-157046	Reservation signal design for LAA	Qualcomm Inc.

R1-157047	LAA DRX operation and PHY layer aspects	Qualcomm Inc.

R1-157101	Discussion on the first subframe of LAA DL transmission burst	ETRI

R1-157102	Modified compact initial signal designs for cell detection	ETRI

R1-157116	Consideration on partial TTI	Sharp

R1-157117	Synchronization and cell detection in LAA carrier	Sharp

R1-157121	Consideration on scheduling methods for LAA SCell	Sharp

R1-157133	On DL/UL burst and subframe indication for LAA	Nokia Networks

R1-157169	Design Considerations of Ending Partial Subframe	Mediatek Inc.

R1-157170	LAA signaling design	Mediatek Inc.

R1-157171	LAA Initial signal design	Mediatek Inc.

R1-157214	PDSCH Transmission options for LAA	Motorola Mobility

R1-157216	Activation/Deactivation procedures for LAA Scells	Motorola Mobility

R1-157223	Support of partial subframe transmission for LAA	NTT DOCOMO, INC.

R1-157224	Discussion on L1 signaling design for LAA	NTT DOCOMO, INC.

R1-157235	Discussions on the design of downlink transmissions for LAA	CATR

R1-157240	Discussion and details on data transmission	ETRI

R1-157264	On an Initial Signal for LAA	Ericsson

R1-157265	On LAA Burst Detection and Fine Synchronization	Ericsson

R1-157266	On Support of Multiple DL Data Transmission Starting Points	Ericsson

R1-157268	On the configuration of the last subframe of a DL burst	Ericsson

R1-157269	Carrier Selection for LAA	Ericsson

R1-157280	Dynamic carrier selection between LAA CCs	HTC

R1-157308	On LAA DL scheduling and HARQ operation	Nokia Networks

R1-157309	Remaining details of synchronization in LAA	Nokia Networks

R1-157310	On the need for initial signal specification	Nokia Networks

R1-157311	On data transmission in partial subframe	Nokia Networks

R1-157332	Partial Subframe for PDSCH on LAA Scell	WILUS Inc.

R1-157353	Discussion on power sharing for multicarrier operation for LAA	China Unicom

#### CSI measurements and reporting

CSI measurement and reporting design corresponding to the supported DL transmission modes.

R1-157579	WF on CSI measurement in LAA	LG Electronics, Sharp, Samsung

R1-156770	Remaining details of CSI measurement	Samsung

R1-157049	CSI measurements and reporting for LAA	Qualcomm Inc.

R1-156867	CSI measurement in LAA	LG Electronics

R1-156441	Support of CSI measurement for LAA	Huawei, HiSilicon

R1-156518	Discussion on CSI- measurement and reporting for LAA (need more internal discussion)	Intel Corporation

R1-156583	CSI measurement and reporting for LAA	CATT

R1-156771	LAA partial subframe	Samsung

R1-156772	LAA CSI reporting	Samsung

R1-157018	CSI Measurement and Reporting for LAA	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

R1-157086	On the CSI reporting for LAA	ITRI

R1-157134	On CSI reporting for LTE LAA	Nokia Networks

R1-157147	On remaining issues of LAA CSI measurement	ZTE Corporation

R1-157225	Discussion on CSI measurement and reporting for LAA	NTT DOCOMO, INC.

R1-157270	On Carrier Power and CSI Measurements in LAA	Ericsson

R1-157271	On CSI Measurements and Reporting in LAA	Ericsson

R1-157297	CSI measurement and reporting for LAA	Innovative Technology Lab Co.

#### Other

R1-156584	Frame structure for LAA Scell	CATT

R1-156735	Frame structure for LAA SCells supporting both DL and UL transmissions	Lenovo (Beijing) Ltd

R1-156907	Transmission bandwidth for LAA	Huawei, HiSilicon