IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-1-4 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Broadband | 12670 High Bluff Drive San Diego CA 92130 USA | 1 858 480 3100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for the US.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Table of contents

1	Table of contents	2

2	Document version release notes	3

3	Introduction and document usage	3

4	Scenarios	3

5	Environmental factors	4

5.1	Large-scale fading	4

5.1.1	General	4

5.1.2	Outdoor-to-outdoor and outdoor-to-indoor model	4

5.1.3	Amendment for outdoor-to-indoor propagation model	5

5.1.4	Indoor model	5

5.2	Small-scale fading	5

5.2.1	Outdoor propagation	5

5.2.2	Indoor propagation	6

6	Device parameters	6

6.1	General	6

6.1.1	Channel bandwidth	6

6.1.2	Nominal antenna gains	6

6.1.3	Horizontal radiation patterns	6

6.1.4	Cabling loss	8

6.1.5	Maximum EIRP	9

6.1.6	Typical EIRP	9

6.1.7	Power Control	9

6.1.8	Link adaptation	9

6.1.9	Interference thresholds for frame error calculation	10

6.2	802.11y specific parameters	10

6.2.1	General	10

6.2.2	802.11y feature support for 3.65GHz	10

6.2.3	Receiver sensitivity	10

6.2.4	CCA and medium access parameters	11

6.3	802.16h specific parameters	13

6.3.1	General	14

6.3.2	802.16h feature support for 3.65GHz	14

6.3.3	802.16 parameters	15

6.3.4	Receiver sensitivity	15

7	External driving parameters	16

7.1	Traffic model	16

7.1.1	Unimodal traffic model	17

7.1.2	Multimodal traffic model	17

7.1.3	VoIP traffic model	18

7.1.4	Fragmentation	18

8	Coexistence metrics	18

9	Regulatory requirements	19

10	References	21

11	Abbreviations	21

12	Definitions	22

13	Annex 1	22

14	Annex 2	23

14.1	Beacon period	23

14.2	Beacon transmission	23

14.3	Quiet intervals	24

# Document version release notes

| Date | Author | Action | Notes |
| --- | --- | --- | --- |
| 30 Nov 2007 | Paul Piggin | Creating r11 | Following from 802.19 CC 29 November 2007: > Addition of OFDMA subchannelisation gain in the 802.16h uplink budget. > Modification to the traffic model definition. > Addition of a coexistence metrics section ready to reference a companion coexistence metrics document. |
| 2 Jan 2008 | Paul Piggin | Creating r12 | Following from 802.19 CC 13 December 2007: > Modifications to section 6.1.7 and 6.1.8. > Modifications to section 7.1. > Added reference to Up-link OFDMA subchannel allocation. |
| 4 Jan 2008 | Paul Piggin | Creating r13 | Following from 802.19 CC 3 January 2008: > Add a specific reference for 802.16 in the references section and modified text accordingly. > Added in section 7.1.1. values for offered load based on the packet sizes and packet inter-arrival times. |

# Introduction and document usage

This document provides simulation parameters for a number of deployment scenarios for use in Coexistence Assurance analysis between 802.16h [1] and 802.11y [2] systems in the 3.65-3.7GHz band in the US.

In order to ensure a reasonably concise and representative set of simulation cases for Coexistence Assurance the scenarios in section 4 are marked as either mandatory or optional. A scenario marked as mandatory is required for inclusion in any Coexistence Assurance analysis, whilst an optional scenario can be used to provide additional supplementary information. Parameters within the document are either associated with scenario (in which case these parameters are implicitly mandatory or optional) or stand alone. The stand alone parameters are explicitly labelled as either mandatory or optional, and where appropriate to which scenarios they apply. In this way it is possible to narrow parameter selection for a given scenario where a number of possible parameters may apply.

All algorithms are optional. However some algorithms are suggested to facilitate comparisons specifically where the algorithm in question is not the main focus of a given study.

# Scenarios

The scenarios are described in this section with the following sections providing reference to which parameters are applicable to which scenario. It is possible to combine scenarios; it is also possible to combine scenarios where a scenario is specific to a given technology.

The scenarios are described thus:

Scenario A – MANDATORY

Outdoor only scenario with fixed subscribers

This scenario considers an outdoor deployment with fixed outdoor subscribers only.

Scenario B - OPTIONAL

Outdoor only scenario with mobile subscribers

This scenario considers an outdoor deployment with mobile outdoor subscribers.

Scenario C - MANDATORY

Outdoor to indoor scenario with portable and mobile subscribers

This scenario considers an outdoor deployment with indoor portable (within a standalone enclosure) and mobile subscribers.

Scenario D - OPTIONAL

Indoor only scenario with portable and mobile subscribers

This scenario considers an indoor only deployment with indoor portable and mobile subscribers.

Radiation safety limits need to be observed in this scenario. The notional maximum transmitter power of 1W/MHz EIRP maximum PSD for Base stations is expected to require capping.

Scenario E - OPTIONAL

A combined scenario

This scenario considers a combination of all the above scenarios.

# Environmental factors

## Large-scale fading

This section describes the model for calculation of pathloss caused by obstructions within the propagation path from transmitter to receiver. These obstructions can be attributed to terrain and/or building clutter.

### General

Carrier frequency = 3675MHz (mid band) MANDATORY (Scenarios A - E)

### Outdoor-to-outdoor and outdoor-to-indoor model

The pathloss model is based on the Stanford University Interim (SUI) model [4]. This model is for outdoor operation with a correction for building penetration. In this document a set of propagation models applicable to the multi-cell architecture is presented. [4] enables the evaluation of equation [1] using the following range of parameters:

- Cells are < 10 km in radius, variety of terrain and tree density types

- Under-the-eave/window or rooftop installed directional antennas (2 – 10 m) subscriber devices

- 15-40 m base station antennas

- High cell coverage requirement (80-90%)

The general pathloss expression is:

PathlossTotal = Pathlossoutdoor-outdoor + SM + BPL						[1]

Specific parameters to calculate Pathlossoutdoor-outdoor are:

Scenarios A (MANDATORY), Scenarios B, D, E (OPTIONAL):

Terrain type B (suburban)

Base Station height: 25m

Subscriber Station height:

Fixed Station: 10m

Portable Station: 2m

Mobile Station: 2m

Specific parameters to calculate PathlossTotal are:

Scenarios A (MANDATORY), B (OPTIONAL):

Standard Deviation = 8-10dB, page 4 [4].

Shadow Margin (SM) = 6dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 0dB

### Amendment for outdoor-to-indoor propagation model

Specific parameters to calculate PathlossTotal are:

Scenarios C (MANDATORY), E (OPTIONAL):

Standard Deviation = 10-12dB.

Shadow Margin (SM) = 8dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 6dB (at window), 12dB (elsewhere)

### Indoor model

Scenarios D (OPTIONAL), E (OPTIONAL):

The 802.11n indoor propagation model is applicable, clause 2 [8].

Base Station height: 2m

Subscriber Station height:

Portable Station: 2m

Mobile Station: 2m

## Small-scale fading

This section describes the model for calculation of fading caused by obstructions and movement of obstructions within the propagation environment.

### Outdoor propagation

Scenario A (MANDATORY):

Fixed Station: 10m

Ricean K factor = 12dB (Multipath fade margin = 2dB)

Scenarios B (OPTIONAL), C (MANDATORY):

Portable Station: 2m

Mobile Station: 2m

Rayleigh fading.

Multipath fade margin:

802.11 (1Tx/1Rx: Downlink + Uplink) = 13dB [Note: Selection combining derived]

802.16 (2Tx/2Rx: Downlink) = 5dB [Note: Combined array transmitted power not to exceed regulatory requirements]

802.16 (1Tx/2Rx: Uplink) = 10dB

### Indoor propagation

Scenarios D (OPTIONAL), E (OPTIONAL):

The 802.11n indoor propagation model is applicable, clause 2 [8].

# Device parameters

## General

### Channel bandwidth

The following channel bandwidths are supported:

5MHz – MANDATORY (Scenarios A - E)

10MHz – OPTIONAL (Scenarios A - E)

20MHz – OPTIONAL (Scenarios A - E)

[Note: 802.11 5MHz transmission mode is mandated as part of the standard [7]]

### Nominal antenna gains

Nominal bore sight antenna gains; for omni directional antennas this is the bore sight gain in elevation and for directional antennas the bore sight gain in azimuth and elevation.

Base Station:

Base Station (outdoor): 18dBi (directional) – OPTIONAL (Scenarios A, B, C, E)

Base Station (outdoor): 10dBi (omni directional) – MANDATORY (Scenarios A, B, C, E)

Base Station (indoor): 0dBi (omni directional) – OPTIONAL (Scenarios D, E)

Subscriber Station:

Mobile Station: 2dBi (omni directional) – MANDATORY (Scenarios B, C, E)

Portable Station: 2dBi (omni directional - indoor) – OPTIONAL (Scenarios C, D, E)

Portable Station: 8dBi (directional - indoor) – MANDATORY (Scenarios C, D, E)

Fixed Station: 5dBi (omni directional) – OPTIONAL (Scenarios A, D, E)

Fixed Station: 18dBi (directional - outdoor) – MANDATORY (Scenarios A, E)

### Horizontal radiation patterns

This section is for information only.

This section is divided into the three antenna types specified above, namely:

- Directional base station,

- Directional subscriber station,

- Omni directional.

Each antenna type is described in turn using representative values. Specific scenarios can be addressed using the nominal maximum antenna gains given above.

Directional base station

A reference base station antenna is given in [12] and also [15]. For angle θ (relative to the boresight direction), the antenna has a gain (dB) of:

Gθ=Gmax+max−12θθ3dB2,−GFB						[2]

Where θ3dB is the 3dB beamwidth (70 degrees), θ≤180, Gmax is the maximum antenna gain (boresight direction, ~18 dBi) and GFB is the front-to-back ratio ( ~25 dBi).

This antenna is expected to be deployed in a 3 sector configuration, each sector covering 120 degrees. A diagrammatic representation is given in Figure 2.

Figure 1 An example sector beam pattern for base station (directional - outdoor) antenna [12] and [15].

Directional subscriber station

Maximum antenna gain Gmax = 18dBi.

3dB beamwidth, θ3dB =~ 20 degrees. This is in both azimuth and elevation planes.

Front-to-back ratio, GFB = 25dB.

A diagrammatic representation of an idealised radiation pattern is given in Figure 2.

Figure 2 An example radiation pattern for a directional subscriber station antenna.

Omni directional

Maximum antenna gain Gθ=Gmax.

Front-to-back ratio, GFB = 0dB.

Elevation discrimination ranges from ~6 degrees for higher gain base station antennas to upwards of ~45 degrees for lower gain subscriber antennas.

### Cabling loss

Base Station: 1dB (802.16), 1dB (802.11) – MANDATORY (Scenarios A - E)

Subscriber Station (connector loss):

Fixed Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios A, E)

Portable Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios C, D, E)

Mobile Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios B, C, D, E)

### Maximum EIRP

Maximum EIRP values are based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz: – MANDATORY (Scenarios A - E)

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz: – OPTIONAL (Scenarios A - E)

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz: – OPTIONAL (Scenarios A - E)

Base/Fixed stations =20W (43dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

### Typical EIRP

Typical transmit powers are presented in this section.

5 MHz: – MANDATORY (Scenarios A - E)

802.16 outdoor Base Station transmits at maximum power.

802.11 outdoor Base Station transmits at maximum power.

802.16 and 802.11 Fixed, Portable and Mobile subscribers transmit at 17dBm.

### Power Control

802.16-based systems shall follow the power control rules as described in clause 8.4.10 of [5] and as amended in [6].

### Link adaptation

Link adaptation is intended to maintain a consistent power density at the Base Station by setting an appropriate modulation and FEC rate at each subscriber station. The Base Station may change the subscriber station’s transmit power through power correction messages.

The simplest scheme for link adaptation within a simulation is to provide a link budget assessment (with an appropriate margin) and the selection of the highest permissible MCS for the link based on pathloss and equipment parameters. For 802.16-based systems transmit power may be selected for a given MCS in the range of powers supported for that MCS. It should be noted that TPC is specified for 802.11-based systems but is not expected to be implemented in this simulation. In this case 802.11-based systems will transmit at maximum power for the selected MCS.

For 802.16-based systems MCS selection shall be implemented as described in [18]. This is part of the uplink OFDMA subchannel allocation scheme.

A 2dB margin (MANDATORY (Scenarios A - E)) shall be imposed at the MCS transition points for both 802.16 and 802.11 systems. That is to say, if during MCS selection the link is within 2dB of the next highest modulation scheme the lower modulation scheme shall be selected. This ensures a link runs in an optimal state.

### Interference thresholds for frame error calculation

The interference threshold for frame error calculation is a method used to determine if a frame is received in error. The way to determine if a frame is in error is by simulating the wireless channel and the receiver. A simplified method may use the criteria below, which shall be true for the full duration of the received frame:

S(I+N)>SNR+FM									[3]

where:

S is the Signal power,

N is the Noise level,

I is the Interference level,

FM is the interference Fade margin. The value is 2dB – MANDATORY (Scenarios A - E)

## 802.11y specific parameters

### General

Based on the OFDM PHY for the 5GHz band (clause 17 [7]) specified in [2].

### 802.11y feature support for 3.65GHz

Maximum frame transmission duration between CCA = 4ms (subclause I.1 [2])

Use of an enabling signal DSE (subclause 11.14 [2])

RTS/CTS: enabled

### Receiver sensitivity

- Number of MCS = 8

- AWGN

Figure 3 Typical receiver sensitivities for OFDM 5GHz PHY [7]; Note: an Implementation Margin of 5dB, and a Noise Figure of 10dB is implicitly assumed in these sensitivity values.

20MHz data rates: 54, 48, 36, 24, 18, 12, 9, 6 Mbps – OPTIONAL (Scenarios A - E)

10MHz data rates: 27, 24, 18, 12, 9, 6, 4.5, 3 Mbps (for -3dB on sensitivity for 20MHz) – OPTIONAL (Scenarios A - E)

5MHz data rates: 13.5, 12, 9, 6, 4.5, 3, 2.25, 1.5 Mbps (for -6dB on sensitivity for 20MHz) – MANDATORY (Scenarios A - E)

### CCA and medium access parameters

Taken from table 147 [7].

Figure 4 Time domain representation of medium access for 802.11.

- HCF (Hybrid Coordination Function) is specified in the 802.11e amendment [7].

- HCF consists of EDCA (Enhanced Distributed Channel Access, distribution function) and HCCA (HCF Controlled Channel Access, centralized function).

- WMM (Wi-Fi Multimedia) certifies the EDCA and TXOP (Transmit Opportunity) features.

- EDCA and TXOP features enhance the QoS support in 802.11.

- EDCA introduces 4 AC (Access Categories) that prioritizes traffic class access to the air interface.

- TXOPs are used to provide a station with a time period in which to transmit in a non-contended manner.

Figure 5 Values for EDCA 4 AC parameters [10].

For analysis the highest priority (AC_VO) and one of the lowest priority (AC_BE) ACs is recommended for use.

Range dependency on propagation time

Subclause 7.3.2.9 Country information element of [7] states:

The Country information element contains the information required to allow a station to identify the regulatory domain in which the station is located and to configure its PHY for operation in that regulatory domain.

The Coverage Class field of the regulatory triplet specifies the aAirPropagationTime characteristic used in basic service set (BSS) operation, as shown in Table 27. The characteristic aAirPropagationTime describes variations in actual propagation time that are accounted for in a BSS and, together with maximum transmit power level, allow control of BSS diameter.

From Table 27 [7]:

Coverage class value = 0 for aAirPropagationTime ≤1µs

Coverage class value = 1 for aAirPropagationTime 3µs

Coverage class value = 2 for aAirPropagationTime 6µs

…

Coverage class value = 31 for aAirPropagationTime 93µs

Subclause 9.8.4 of [2] states:

Radio waves propagate at 300 m/μs in free space, and, for example, 3μs would be the ceiling for BSS maximum one way distance of ~450 m (~900m round trip).

MANDATORY Scenarios A, B, C, E:

Outdoor case: Coverage class value = 4, (12µs), giving a cell radius of 1400m, round trip 2800m (9.3µs). The maximum cell radius as calculated in Annex 1.

MANDATORY Scenarios D, E:

Indoor case: Coverage class value = 0, (≤1µs), giving a cell radius of 150m, round trip 300m.

Timing values

Reference 9.2.10 DCF timing relations [7] and Figure 4.

(Legend 20MHz (OPTIONAL All scenarios)/10MHz (OPTIONAL All scenarios)/5MHz (MANDATORY All scenarios))

SIFS = 16/32/64µs

AIFS[AC] = SIFS + AIFSN[AC].aSlotTime

From Figure 170 [7]

aSlotTime = aCCATime + aRTTXTurnaroundTime + aAirPropagationTime + aMACProcessingTime

aCCATime (CCA) = 4/8/16µs

aMACProcessingDelay (M2) = 2/2/2µs

aRXTXTurnaroundTime (Rx/Tx) = 2/2/2µs

aAirPropagationTime (D2) = 	12µs – MANDATORY (Scenarios A, B, C, E) (all bandwidths)

aAirPropagationTime (D2) = 	1µs – MANDATORY (Scenarios D, E) (all bandwidths)

|  | AC_VO AIFSN[AC_VO] = 2 [Contention Window = 3, 7] | AC_BE AIFSN[AC_BE] = 3 [Contention Window = 15, 1023] |
| --- | --- | --- |
| Outdoor MANDATORY Scenarios A, B, C, E | aSlotTime = 20/24/32µs AIFS[AC_VO] = 56/80/128µs | aSlotTime = 20/24/32µs AIFS[AC_BE] = 76/104/160µs |
| Indoor MANDATORY Scenarios D, E | aSlotTime = 9/13/21µs AIFS[AC_VO] = 34/58/106µs | aSlotTime = 9/13/21µs AIFS[AC_BE] = 43/71/127µs |

Figure 6 aSlotTime and AIFS[AC] durations based on AC and deployment assumptions.

CCA-CS threshold

-82dBm (20MHz) – OPTIONAL (Scenarios A - E)

-85dBm (10MHz) – OPTIONAL (Scenarios A - E)

-88dBm (5MHz) – MANDATORY (Scenarios A - E)

CCA-ED threshold

-72dBm (20MHz) – OPTIONAL (Scenarios A - E)

-75dBm (10MHz) – OPTIONAL (Scenarios A - E)

-78dBm (5MHz) – MANDATORY (Scenarios A - E)

These thresholds are intended to be no higher (less sensitive) than the values stated.

These threshold values are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

## 802.16h specific parameters

### General

Based on the OFDMA PHY (subclause 8.4 [5] and as amended in [6]) and profiled in WiMAX Forum Mobile WiMAX System Profile [3] specified in [1].

### 802.16h feature support for 3.65GHz

[1] specifies a number of features for coexistence, these are described in this section.

#### Uncoordinated Coexistence Protocol (UCP)

Uncoordinated Coexistence Protocol (UCP) [1] is specified as a set of features to provide coexistence in the 3.65GHz band. UCP is specified in the following sub-clauses:

UCP (Uncoordinated Coexistence Protocol) subclause 6.4.2.3 [1]:

DCS (Dynamic Channel Selection) subclause 6.4.2.3.2 [1]

LBT (Listen-Before-Talk) subclause 6.4.2.3.7 [1]

Given below are the parameters for the DMA (Dynamic Medium Acquisition) aspect of LBT 6.4.2.3.7 [1]. This is shown diagrammatically in Figure 7.

(Legend 20MHz (OPTIONAL All scenarios)/10MHz (OPTIONAL All scenarios)/5MHz (MANDATORY All scenarios))

MINFRST = TCMA + TFRAME_END_OFFSET

TCMA = 4/8/16μs

TFRAME_END_OFFSET = 50μs (minimum time to switch Rx/Tx and send FRS (Frame Reservation Signal), all bandwidths)

MINFRST = 54/58/66μs

MAXFRST = 4000µs

K = 1

Figure 7 Representation of parameters used in DMA when an attempt is made to reclaimed the medium. DMA operation is shown for two distinct cases Tmedium_quiet > FRSTn and Tmedium_quiet < FRSTn . The timings in the figure are not to scale.

CMA-CS threshold

-82dBm (20MHz) – OPTIONAL (Scenarios A - E)

-85dBm (10MHz) – OPTIONAL (Scenarios A - E)

-88dBm (5MHz) – MANDATORY (Scenarios A - E)

CMA-ED threshold

-72dBm (20MHz) – OPTIONAL (Scenarios A - E)

-75dBm (10MHz) – OPTIONAL (Scenarios A - E)

-78dBm (5MHz) – MANDATORY (Scenarios A - E)

These thresholds are intended to be no higher (less sensitive) than the values stated.

These threshold values are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

#### Coordinated Contention Based Protocol (CX-CBP)

Coordinated Contention Based Protocol (CX-CBP) [1] is specified as a set of features to provide coexistence in the 3.65GHz band. CX-CBP is specified and supported in the following sub-clauses:

CX-CBP (Coordinated Contention Based Protocol) subclause 15.4.1.3 [1]

Provision for WirelessMAN-CX procedure subclause 15.2 [1]

Annex 2 details behavior, in the form of recommended parameters, required by 802.11 systems when working in a coexistence environment with CX-CBP. Specifically Annex 2 describes recommendations for required 802.11 quiet intervals which respect the 802.16 frame structure. 802.11 systems are also required to maintain beacon transmissions aligned with the 5ms frames of 802.16 systems.

### 802.16 parameters

MANDATORY (Scenarios A - E) Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration = 5ms

47 symbols per frame

- 28 in the downlink (1 for preamble)

- 19 in the uplink

- Representing the nearest fit to a 60% downlink, 40% uplink asymmetry

103µs symbol duration

TTG > 50µs (RTD and Timing Advance dependant)

RTG = 60µs

SSTTG = SSRTG = 50µs

### Receiver sensitivity

- Number of MCS = 7

- Permutation = PUSC

- Coding = CTC

- No HARQ

- AWGN

|  |  | Sensitivity (dBm) |  |  | Data rates (kbps) (per subchannel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz |  | 10MHz |  | 20MHz |  |
|  | Eb/No (dB) | 5MHz | 10MHz | 20MHz | DL | UL | DL | UL | DL | UL |
| QPSK1/2 | 2.9 | -91.1 | -88.1 | -85.1 | 150.4 | 153.6 | 150.4 | 153.6 | 150.4 | 151.4 |
| QPSK3/4 | 6.3 | -88.8 | -85.8 | -82.8 | 225.6 | 230.4 | 225.6 | 230.4 | 225.6 | 227.1 |
| 16QAM1/2 | 8.6 | -84.7 | -81.7 | -78.7 | 300.8 | 307.2 | 300.8 | 307.2 | 300.8 | 302.8 |
| 16QAM3/4 | 12.7 | -83.6 | -80.6 | -77.6 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM1/2 | 13.8 | -80.5 | -77.5 | -74.5 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM2/3 | 16.9 | -79.4 | -76.4 | -73.4 | 601.6 | 614.4 | 601.6 | 614.4 | 601.6 | 605.6 |
| 64QAM3/4 | 18 | -77.5 | -74.5 | -71.5 | 676.8 | 691.2 | 676.8 | 691.2 | 676.8 | 681.3 |
| Subchannels available: |  |  |  |  | 24 | 17 | 48 | 35 | 96 | 70 |

Figure 8 802.16h sensitivity and data rate typical values taken from WiMAX Forum RCT [11], Note: An Implementation Margin of 5dB, and a Noise Figure of 8dB is implicitly assumed in these sensitivity values.

Note: 20MHz channels – OPTIONAL (Scenarios A - E), 10MHz channels – OPTIONAL (Scenarios A - E), 5MHz channels – MANDATORY (Scenarios A - E).

Uplink OFDMA subchannel allocation for 802.16h systems is described in [18].

# External driving parameters

External driving parameters are suggested in this section. Detail of these processes, external to the simulation, require description to enable a meaningful comparison of different simulation cases.

## Traffic model

This section provides information on traffic models used in simulation. The details herein are not intended to be exhaustive. Further background information is contained in [16]. It is assumed throughout that the packet sizes are defined at the network layer 3; appropriate MAC overheads are required where applicable.

802.11y ACK SDU offered to the PHY 14bytes: MANDATORY (Scenarios A - E)

The traffic models included are summarized thus:

Unimodal traffic model (section 7.1.1):

Fixed packet size: MANDATORY (Scenarios A - E)

Packet size derived from a uniformly distribution random variable: OPTIONAL (Scenarios A - E)

Fixed inter arrival time: MANDATORY (Scenarios A - E)

Inter arrival time derived from a uniformly distribution random variable: OPTIONAL (Scenarios A - E)

Multimodal traffic model (section 7.1.2):

OPTIONAL (Scenarios A - E)

VoIP traffic model (section 7.1.3):

OPTIONAL (Scenarios A - E)

Fragmentation (section 7.1.4):

MANDATORY (Scenarios A - E)

The number of subscribers per AP/BS (802.16 and 802.11):

4: MANDATORY (Scenarios A - E)

Otherwise: OPTIONAL (Scenarios A - E)

The traffic models are described in more detail in the following sections.

### Unimodal traffic model

A unimodal traffic model based on a single packet size is defined.

| Probability (%) | Packet size (bytes) |
| --- | --- |
| 100 | 1500 |

Figure 9 Unimodal traffic model based on Internet traffic observations.

Packet inter arrival time varies as required and defines the offered traffic load. Suitable inter arrival times are 100ms, 75ms, 50ms, 25ms, 20ms, 10ms, 7.5ms, 5ms, 2.5ms, 2ms, 1ms, 0.75ms, 0.5ms.

| Inter arrival time (ms) | Offered load (kbps) | MANDATORY (Scenarios A - E) |
| --- | --- | --- |
| 100 | 120 | Yes |
| 75 | 160 | Yes |
| 50 | 240 | Yes |
| 25 | 480 | Yes |
| 20 | 600 | Yes |
| 10 | 1200 | Yes |
| 7.5 | 1600 | Yes |
| 5 | 2400 | Yes |
| 2.5 | 4800 | Yes |
| 2 | 6000 | Yes |
| 1 | 12 000 | Yes |
| 0.75 | 16 000 | Yes |
| 0.5 | 24 000 | Yes |

Figure 10 Packet inter arrival times with corresponding values of offered load.

### Multimodal traffic model

A multimodal traffic model based on aggregated internet traffic observations is defined. The observations are from one day’s CAIDA (www.caida.org) based measurements within a municipal wi-fi mesh network.

| Probability (%) | Packet size (bytes) |
| --- | --- |
| 20 | 1500 |
| 20 | 554 (majority MTU discovery) |
| 60 | 114 |

Figure 11 Multimodal traffic model based on Internet traffic observations.

Packet inter arrival time varies as required and defines the offered traffic load. Suitable inter arrival times are 100ms, 75ms, 50ms, 25ms, 20ms, 10ms, 7.5ms, 5ms, 2.5ms, 2ms, 1ms, 0.75ms, 0.5ms.

### VoIP traffic model

A VoIP traffic model is suitable for the appropriate 802.11e Access Category.

Figure 12 VoIP traffic model [12].

From [13]:

AMR with Header Compression IPv4(IPv6)

Voice payload:

Active: 33 bytes

Inactive: 7bytes

Protocol header:

2 bytes (4 bytes)

### Fragmentation

Maximum transmission period of 4ms between medium sensing for 802.11y.

# Coexistence metrics

Coexistence metrics are discussed in [17]. This section makes a statement on which of these metrics shall be mandatory and which optional.

This section is at present incomplete. This section is to include:

- Occupancy metric is Optional

- ‘Successful’ occupancy metric is mandatory

- Rules are necessary to define what constitutes a hidden node and how this is related to FER.

# Regulatory requirements

3.65-3.7GHz

Channel width = 5, 10, 20MHz

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band [14].

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

# References

[1] IEEE P802.16h/D3: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D5.0: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)

[4] IEEE 802.16.3c-01/29r4: Channel Models for Fixed Wireless Applications, Erceg et al., IEEE 802.16 Broadband Wireless Access Working Group, January 2001.

[5] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed Broadband Wireless Access Systems. IEEE Std 802.16-2004; October 2004.

[6] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands. IEEE Std 802.16e-2005 and IEEE Std 801.6-2004/Cor1-2005; February 2006.

[7] Standard for Information Technology— Telecommunications and information exchange between systems— Local and metropolitan area networks— Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.

[8] TGn channel models, IEEE 802.11-03/940r4, May 2004.

[9] Broadband Wireless Internet Forum White Paper, BWIF - Bringing Broadband Wireless Access Indoors, Document Number WP-4_TG-1, Version 1.0, September 25, 2001.

[10] Analysis of IEEE 802.11e for QoS Support in Wireless LANs, Mangold, Choi, Hiertz, Klein, Walke, IEEE Wireless Communications, December 2003.

[11] WiMAX Forum Mobile Radio Conformance Tests (MRCT), Release 1.0 Approved Specification (Revision 1.1.0).

[12] WiMAX Forum System Evaluation Methodology, Draft V1.0, 1/30/2007.

[13] Mobile WiMAX VoIP Capacity System Level Simulations, Roshni Srinivasan, Tolis Papathanassiou, Shailender Timiri, Wireless Standards and Technology Mobility Group, Intel Corporation March 26, 2007.

[14] Title 47 – Telecommunication, Part 90 – Private Land Mobile Radio Services: Subpart Z - Wireless Broadband Services in the 3650-3700 MHz Band, Part 90.1301 - 90.1337, Federal Communications Commission.

[15] 3GPP TR 25.814, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Physical layer aspects for  evolved Universal Terrestrial Radio Access (UTRA). http://www.3gpp.org/ftp/Specs/html-info/25814.htm

[16] CDMA2000 Evaluation Methodology (C30-20061204-062A-Evaluation_methodology_text_V6), ftp://ftp.3gpp2.org/TSGC/Working/2006/2006-12-Maui/TSG-C-2006-12-Maui/WG3/

[17] IEEE P802.19-07/0020, Coexistence Metrics for the 3650 MHz Band, IEEE P802.19 Coexistence TAG, Stephen J. Shellhammer.

[18] IEEE P802.19-07/0027, Up-link OFDMA sub-channel allocation for Wireless Coexistence in US 3.65GHz band, IEEE P802.19 Coexistence TAG, Shahar Hauzner, Mariana Goldhamer.

# Abbreviations

AC		Access Categories

aEQP		Adaptive Extended Quiet Period

AP		Access point

BPL		Building Penetration Loss

BS		Base Station

CBP		Contention Based Protocol

CCA-CS	Clear Channel Assessment – Carrier Sense

CCA-ED	Clear Channel Assessment – Energy Detect

DCS		Dynamic Channel Selection

EIRP		Effective Isotopic Radiated Power

EQP		Extended Quiet Period

FEC		Forward Error Correction

FER		Frame Error Rate

LBT		Listen Before Talk

MCS		Modulation and Coding Schemes

MTU		Maximum Transmission Unit

OFDMA	Orthogonal Frequency Division Multiple Access

PDU		Protocol Data Unit

PUSC		Partial Usage of SubChannels

SDU		Service Data Unit

SISO		Single Input Single Output

SS		Subscriber Station

STA		Subscriber STAtion

TPC		Transmit Power Control

UCP		Uncoordinated Coexistence Protocol

# Definitions

Base Station	A general term referring to both an 802.11 AP and 802.18 BS.

Subscriber Station	A general term referring to both an 802.11 STA and 802.18 SS.

TU	Time Unit. A measurement, specific to 802.11 systems, of a time interval equal to 1024 µs.

# Annex 1

802.16h & 802.11y uplink budget evaluation

This analysis is used to calculate the maximum range of an 802.16h and 802.11y cell in the 3.65GHz band. This maximum range is used to calculate the maximum round trip time used in turn to evaluate 802.11y medium access parameters via the term aPropagationTime; the 802.16h uplink budget is for information and comparison purposes.

Assumptions

- Subscriber (max. output power 50mW (17dBm))

- Subscriber connector loss = 0.5dB

- Base Station cabling loss = 1dB

- Base Station antennas gain = 10dBi (Omni directional)

- Fade margins:

  - 802.11y (1Tx/1Rx: Uplink) = 13dB

  - 802.16h (1Tx/2Rx: Uplink) = 10dB

- The OFDMA PHY of 802.16h provides uplink subchannelization gain. At the cell edge it is assumed that for the 5MHz channel case 1 of the possible 17 subchannels are used. This equates to an effective 10log(17)=12.3dB gain.

- Assumptions aligned with Scenario A.

- The System Gain is used to calculate an appropriate cell range based on the SUI propagation model [5.1.2] and using the parameters defined. In this evaluation Shadow Margin is not applied.

| Parameter | 802.16h | 802.11y |  |  | Note |
| --- | --- | --- | --- | --- | --- |
| Subscriber output power [x] | 17 | 17 | dBm |  | 5MHz channel |
| Subscriber antenna gain [y] | 18 | 18 | dBi |  |  |
| Subscriber cabling loss [z] | 0.5 | 0.5 | dB |  |  |
| Subscriber EIRP [a] | 34.5 | 34.5 | dBm | [x]+[y]-[z] |  |
| BS antenna gain [b] | 10 | 10 | dBi |  |  |
| BS cabling loss [c] | 1 | 1 | dB |  |  |
| Fade margin [d] | 10 | 13 | dB |  | Based on antenna configuration (Section 4.2.1) |
| Effective OFDMA gain [e] | 12.3 | 0 | dB |  | 1 of 17 subchannels are used at the cell edge |
| Receiver sensitivity [f] | -91.1 | -88 | dBm |  | 802.11: 1.5Mbps BPSK 1/2; 802.16: 1.5Mbps QPSK 1/2 |
| Data rate | 0.153 | 1.5 | Mbps |  |  |
| System Gain | 136.9 | 118.5 | dB | [a]+[b]-[c]-[d]+[e]-[f] |  |
| SUI parameters | Terrain C (Rural) | Terrain C (Rural) |  |  | See Section 4.1.2 |
| Base Station height | 25 | 25 | m |  |  |
| Subscriber height | 10 | 10 | m |  |  |
| Frequency | 3675 | 3675 | MHz |  |  |
| SUI Pathloss | 136.8 | 120.4 | dB |  | Required to match System Gain with no Shadow Margin |
| Cell range | 3400 | 1400 | m |  |  |

Figure 13 802.16h & 802.11y uplink budget evaluation.

# Annex 2

802.11y recommended parameters for Coordinated Contention Based Protocol (CX-CBP) [1] operation

## Beacon period

The beacon period should be equal to 117 TUs (Time Units). This is 119.8ms. This interval is selected to be similar to the preferred 100ms beacon period of 802.11 and be similar to the 20ms period of the CX-Frame [1].

## Beacon transmission

To occur at the start of the CXCBI [1] (when 802.16 is not scheduled to transmit). At the beginning of the CXCBI there are two symbols in which 802.16 does not transmit. The 802.11 base station shall check which of the 6 possible beacon transmission slots has minimum interference and use it for beacon transmission. The CXCBI shall be used for the 802.11 base station downlink data transmission first, but at least one slot will be kept vacant for another 802.11 base station beacon transmission.

## Quiet intervals

Quiet intervals are scheduled so as to not overlap with the CXSBI intervals and to leave the start of the CXCBI for 802.11 base station downlink transmission. (11.9.2 Quieting channels for testing, and 7.3.2.23 Quiet Element [7]). The Quiet Elements should use the following parameters:

Quiet Element 1

Quiet count = 1 (start during next beacon)

Quiet period = 1 (in every beacon interval)

Quiet offset = 9 (9*1.024= 9.216ms)

Quiet duration = 11 (11*1024 = 11.26ms)

Protected start of next CXCBI: 1.024*(9+11)-20=240ns

Quiet Element 2

Quiet count = 1

Quiet period = 1

Quiet offset = 29 (29*1.024= 29.7ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(40)-40=240ns

Quiet Element 3

Quiet count = 1

Quiet period = 1

Quiet offset = 48 (48*1.024= 49.152ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(48+11)-60=416ns

Quiet Element 4

Quiet count = 1

Quiet period = 1

Quiet offset = 68 (68*1.024= 69.63ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(69+10)-80=896ns

Quiet Element 5

Quiet count = 1

Quiet period = 1

Quiet offset = 87 (87*1024= 89.08ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(87+11)-100=352ns

Quiet Element 6

Quiet count = 1

Quiet period = 1

Quiet offset = 107 (107*1024= 109.57ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(107+12)-120=830ns

The quiet intervals related to the CX-Frame are shown in Figure 14.

Figure 14 The relationship between the 802.11 quiet intervals and the CX-Frame [1]